3GPP TSG-RAN WG2 #105	R2-1900868

Athens, Greece, February 25th-March 1st 2019

Title:	[Draft] LS on capability of measurement gap patterns

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	MediaTek [to be RAN2]

To:	RAN4

Cc:

Contact Person:

#### Name:	Chun-Fan (Felix), Tsai

E-mail Address:	Chun-Fan.Tsai@mediaTek.com

1. Overall Description

RAN2 would like to inform RAN4 that RAN2 has discussed the capability signalling of measurement gap patterns.  And RAN2 thinks that there may be some inconsistence between RAN2 and RAN4 specifications.

RAN2 agrees that current RRC signaling for gap pattern capabilities as following:

- Gap patterns 0 and 1 are mandatory for all cases.

- In LTE SA

  - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 or 3.

  - LTE RRC signalling measGapPatterns (8 bits) is used to signal whether UE supports gap patterns 4 to 11.

- In EN-DC

  - LTE RRC signalling shortMeasurementGap and measGapPatterns are used to signal whether UE supports gap patterns 2 to 11 (for per-UE cap and FR1 gap of per-FR gap)

  - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23 (for FR2 gap of per-FR gap)

- In NR SA

  - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23.

- If the UE supports an optional gap pattern (#2 to #23), it supports using the gap pattern for LTE or NR measurement as long as this is applicable according to RAN4 specifications.

RAN2 would like check with RAN4 that whether the above agreements are consistent with 36.133 and 38.133, especially on the following notes:

- (38.133 Table 9.1.2-3) NOTE 3: The measurement gap pattern #2, 3, 6, 7, 8, 10 are supported by the UEs which indicate the capability signalling of supportedGapPattern to network.

- (38.133 Table 9.1.2-2) NOTE 2: The gap patterns with short MGL (gap pattern #2, 3, 6, 7, 8, 10) are supported by UEs which support shortMeasurementGap-r14.

- (36.133 Table 8.1.2.1-1) NOTE 5: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement when UEs support shortMeasurementGap-r14 or supported by UEs configured to perform inter-RAT NR measurement only.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to take above RAN2 agreement into consideration and change the specifications if necessary.

3. Date of Next RAN2 Meetings:

RAN2#105Bis, Xi'an, 8-12 Apr 2018

RAN2#106, Reno, 13-17 May 2019