3GPP TSG-RAN WG2 #105	R2-1901074

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.11.4

Source:	MediaTek Inc.

Title:	Power efficient adaptation of multi-carrier operation

Document for:	Discussion and decision

# 1 Introduction

NR is characterised by its ability to deliver exceptionally high performance, both in the amount of data delivered across the NW as well as the speed with which it does so. This can come at the cost of power consumption, which can significantly affect the performance of battery operated devices. The study into reducing the power consumption of UEs seeks to address this aspect of NR systems [1]. It is important that as RAN2 looks into reducing power consumption in NR, it also maintains the technical advantages that NR offers. In this submission, we discuss ways by which the UE can quickly switch from a power efficient state to a high capability state when configured with multiple carriers.

# 2 Discussion

Carrier aggregation (CA) was introduced in LTE and has been carried over into NR. By allowing the aggregation of spectrum in different frequencies, the amount of data that can be exchanged between the UE and the NW can increase significantly. However, the time it takes to activate the aggregation of additional carriers in LTE can be quite significant ranging between 30ms and 1s [2], and as highlighted in our companion paper [3], can go up even further in NR. These delays often result in slow initial data transfer on the primary carrier before additional carriers can be activated, leading to a poor end user experience. To counter this delay, some NWs tend to keep additional carriers activated even during periods of data inactivity. While this may improve the latency and data rates that a user experiences, it comes at the cost of increased power consumption.

Observation 1: The aggregation of carriers of different frequencies allows the NW to create a big pipe to transfer data on.

Observation 2: Activation of carrier aggregation takes a significant amount of time, and can lead to a poor user experience.

To counter the issues described above, the work done in LTE euCA resulted in the definition of a dormant state. When an SCell is in dormant state, the UE is not required to monitor the PDCCH or PDSCH for this cell, cannot transmit in the corresponding uplink, but is required to perform CSI measurements [4]. As link maintenance, i.e. CSI reporting, is done in the dormant state, activation time for a cell in dormant state comes down to between 8ms and 11ms [5].

Observation 3: Dormant state was introduced in LTE to enable the UE to quickly switch from an inactive to an active state.

As discussed in our companion paper [6], the primary cause of increased power consumption is the unnecessary monitoring of PDCCH when no data activity is expected. The benefits with switching the UE to a dormant state is that it doesn’t have to continuously monitor PDCCH on the dormant cell. Rather than defining a new state in NR, a means to configure the UE to stop monitoring PDCCH is all that is necessary to reduce power consumption.

Observation 4: The benefits of a dormant state can be realised by configuring the UE to stop monitoring the PDCCH for control channel activity.

Regardless of how this behaviour is defined, it is important to have a coherent mechanism defined in NR to reduce the UE’s power consumption. The baseline for reducing power consumption is the DRX mechanism. It allows reduced monitoring of PDCCH that flexibly adapts to activity, and is synchronised between the NW and the UE. When in DRX, the inactivity timer is triggered on a cell when some activity is present. As the timer is restarted every time activity is present, this timer duration expands or contracts according to the ongoing activity.

Figure 1: Fast activation and deactivation of CA based on the DRX framework

The inactivity timer is an excellent indication of when activity is expected by the UE. This can be exploited to allow the UE to switch to a high performance state in which all carriers are woken up. An example of such a behaviour is shown in Figure 1. On the reception of a DCI on the PDCCH of the PCell, the inactivity timer is started as the UE expects further activity within the inactivity duration. This additionally acts as a trigger for the UE to start monitoring PDCCH on additional carriers. After a period of inactivity, during which no further DCIs are received, the UE goes to sleep on the PCell and only monitors the PDCCH at predefined intervals. Once the UE enters sleep on the PCell, it expects no further activity to take place. This triggers the UE to switch to a low power state in which it stops monitoring PDCCH on additional carriers, as no further activity is expected.

Figure 2: Fast activation and deactivation of CA based on the wakeup mechanism

In addition to the DRX mechanism, a wakeup mechanism is being discussed in RAN1 that provides significant power savings [6]. If the wakeup indication is received by the UE, it expects activity to be present and starts monitoring PDCCH on the cell. Similar to DRX, this indication can be additionally used to switch the UE from a low power state to a high performance state as depicted in Figure 2. The reception of the wakeup signal can trigger the UE to start monitoring PDCCH on additional carriers, while the expiry of the inactivity timer stops the monitoring of PDCCH on SCells.

It is important to note that while for some types of traffic such as video streaming or data downloads, it is beneficial to transfer the data as quickly as possible and therefore activate all available carriers; for other traffic types such as VoNR, activation of additional carriers provide little benefit and can lead to a significant increase in power consumption. Therefore a means to stage the activation carriers based on the traffic type served needs to be additionally considered.

Proposal 1: Data activity on the primary cell can trigger the UE to start monitoring PDCCH for additional carriers.

Proposal 2: The UE stops monitoring the PDCCH for additional carriers when it doesn’t expect data activity.

# 3 Conclusion

In this contribution we make the following observations:

Observation 1: The aggregation of carriers of different frequencies allows the NW to create a big pipe to transfer data on.

Observation 2: Activation of carrier aggregation takes a significant amount of time, and can lead to a poor user experience.

Observation 3: Dormant state was introduced in LTE to enable the UE to quickly switch from an inactive to an active state.

Observation 4: The benefits of a dormant state can be realised by configuring the UE to stop monitoring the PDCCH for control channel activity.

Based on these observations, we propose:

Proposal 1: Data activity on the primary cell can trigger the UE to start monitoring PDCCH for additional carriers.

Proposal 2: The UE stops monitoring the PDCCH for additional carriers when it doesn’t expect data activity.

# 4 References

- RP-181463 – Study on UE Power Saving in NR (CATT et al.)

- RP-182076 – WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (Ericsson et al.)

- R2-1900442 – Fast SCell Activation for Data Transmission (MediaTek)

- 3GPP TS 36.300 V15.4.0 – EUTRA and EUTRAN overall stage 2 description

- 3GPP TS 36.133 V16.0.0 – EUTRA Requirements for support of radio resource management

- R2-1901077- DRX enhancements for power saving (MediaTek)