3GPP TSG RAN WG2 Meeting #105                                       	R2-191221

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	12.1.13

Source:	Lenovo, Motorola Mobility

Title:	SA2 LS on PARLOS RAN impacts

Document for:	Discussion, Decision

# Introduction

RAN2 received an LS from SA2 [1] on PARLOS where SA2 acknowledges that the PARLOS feature in Rel-16 is related to E-UTRA/EPC.

Addition of RRC establishment cause for RLOS for congestion control and MME selection. TSG RAN noted that the number of spare cause value and spare bits in LTE connection set up procedure is very limited and thus more discussion would be required in TSG RAN WG2…

SA2 indicated in [1]:

Using a new reserved logical channel to transmit the RRC messages (with existing RRC establishment cause) during RRC connection establishment procedure. This would require the RAN WGs to reserve one logical channel ID.

RAN2 needs to analyze if reserving a logical channel identity is possible and reply SA2 accordingly.

# Discussion

In MAC specification [2] there are still 3 LCID values reserved and one of these could be used for indication of PARLOS on CCCH (SRB0). This seems to be acceptable in view of the “Values of eLCID for UL-SCH”, that still has 56 more reserved LCID space. These 56 more reserved LCID space can only be used when a bigger grant (than 56 bits for SRB0) is available – so e.g. for data channels. So, for PARLOS indication the logical channel can only be reserved from the “original” set of LCIDs and since there will be still many LCIDs reserved for other purposes, the reservation of one LCID can be considered feasible from RAN2 point of view.

| Codepoint/Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-01111 | Reserved |
| 10000 | Extended logical channel ID field |
| 10001 | Reserved |
| 10010 | Activation/Deactivation of PDCP Duplication |
| 10011 | Hibernation (1 octet) |
| 10100 | Hibernation (4 octets) |
| 10101 | Activation/Deactivation of CSI-RS |
| 10110 | Recommended bit rate |
| 10111 | SC-PTM Stop Indication |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding and SC-PTM Stop Indication |  |

Figure 1: Values of LCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 000000-000110 | 32-38 | Identity of the logical channel |
| 000111-111111 | 39-95 | Reserved |

Figure 2: Values of eLCID for UL-SCH

Proposal: RAN2 reply to SA2 that it is feasible to use a new reserved logical channel identity to indicate a PARLOS connection establishment.

# Conclusions

This document discussed the SA2 LS [1] and made the following conclusion.

Proposal: RAN2 reply to SA2 that it is feasible to use a new reserved logical channel identity to indicate a PARLOS connection establishment.

A draft reply LS is available in [3].

# References

[1]: S2-1901077 LS reply on PARLOS RAN impacts

[2]: TS36.321-f40

[3]: R2-1901235 LS reply to SA2 on PARLOS