3GPP TSG-RAN WG2 Meeting #105	R2-1901237

# Athens, Greece, 25 Feb - 01 Mar 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1681 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting bearer type change with LCID change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE Rel-15 specified the support for extended number of DRBs (i.e. 15 DRBs), which also necessitated introduction of extended LCID (eLCID) values in MAC. During the bearer harmonization and bearer type change discussions, it was agreed that the bearer type change can be done by LCID change, which avoids need to do reconfiguration with sync (i.e. intra-cell handover). However, currently it is not at all clear whether EN-DC UEs would always support eLCID. The support would be needed when >4 bearers are configured and network wishes to change their type – in such a case, it is not possible to change all bearer types at once since the legacy LCID space (which consists of only 8 LCIDs for DRBs) runs out. This makes the bearer type change using LCIDs not always possible and unnecessarily restricts the functioning of the bearer type change feature which is contrary to what was originally discussed and agreed in RAN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE supporting EN-DC must also support eLCIDs. Impact analysis Impacted functionality: Extended LCID support. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE will not support bearer type change using eLCIDs, leading to network not being able to always perform bearer type change without handover. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems as network will not assume UE supports the eLCID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not posible to perform bearer type change using LCID change in all cases (e.g. when there are several bearers whose type is changed at the same time). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.34.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS36.331 CR3892 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 4.3.34	Inter-RAT Parameters NR

#### 4.3.34.1	en-DC-r15

This field indicates whether UE supports E-UTRA NR Dual Connectivity as specified in TS 36.331 [5] and TS 38.331 [32]. A UE that supports en-DC-r15 shall also support the extended number of LCIDs as specified in TS 36.321 [4].