3GPP TSG-RAN2 Meeting #105	R2-1901277

Athens, Greece, 25 February - 1 March 2019

Agenda Item:	11.11.5 Power consumption reduction in RRM measurements

Source: 	Ericsson

Title:  	Relaxed RRM measurements in Idle mode

Document for:	Discussion and Decision

# Introduction

The main tasks of the UE in Idle mode is to monitor Paging and to perform RRM measurements for Idle mode mobility, i.e. cell (re-)selection. In this contribution potential power saving enhancements related RRM measurements in Idle mode are discussed.

# Background

Relaxed neighbour cell measurement rules in NB-IoT/MTC

A UE is required to perform intra-frequency neighbour cell RSRP/RSRQ measurements, when the serving cell is below the intra-frequency measurement threshold, or more specifically the UE may choose not to perform intra-frequency measurements when Srxlev > SIntraSearchP and Squal > SIntraSearchQ. A similar rule exists for inter-frequency measurements, using the inter-frequency measurement thresholds (SnonIntraSearchP and SnonIntraSearchQ), which are typically configured above the intra-frequency measurement thresholds, i.e. intra-frequency measurements are typically before inter-frequency measurements are triggered.

Because of the specific use case, a NB-IoT or MTC UE can be permanently below the neighbour cell measurement threshold because it is stationary, and thus be required to permanently perform neighbour cell measurements, while re-selection to another neighbour cell is not possible. But a power saving enhancement for NB-IoT/MTC UE was agreed, i.e. the UE is not required to perform neighbour cell measurements when the “relaxed monitoring criteria” is met, i.e. when serving cell measurements indicate that the UE is stationary, i.e. more specifically when no drop in the serving cell signal is detected (see 36.304 section 5.2.4.12):

A NB-IoT/MTC is still required to perform neighbour cell measurements at least every 24 hours. The measured drop in serving cell signal is configured in system information (SSearchDeltaP). The UE is also required to continue neighbour cell measurements after cell (re-)selection for TSearchDeltaP = 5 minutes.

Relaxed serving cell monitoring and WUS in NB-IoT

The UE is typically required to measure the serving cell RSRP/RSRQ every DRX cycle. In NB-IoT, when WUS is configured in the cell, it is possible to relax the serving cell measurements indicated by numDRX-CycleRelaxed-r15 in system information, provided that the neighbour cell measurements are also relaxed, and additional conditions are met, see section 4.6.2.1A in 36.133.

DRX configuration options

The possible DRX cycle lengths in Idle/Inactive mode, i.e. the defaultPagingCycle in Idle mode and the ran-PagingCycle in Inactive mode, that can be configured are {0.32, 0.64, 1.28, 2.56} sec. The possible DRX cycle lengths in Connected mode are for ShortDRX {2, 3, …, 512, 640} msec, and LongDRX {10, 20, …, 5120, 10240} msec.

In NR the UE can negotiate a UE specific DRX cycle with the AMF, see section 5.4.5 in TS 23.501. The UE can indicate a preferred DRX cycle in the registration procedure. The AMF responds with the accepted DRX cycle.

# Discussion

Relaxed monitoring measurement rules for neighbour cell measurements

NB-IoT is a special use case where the UE may be permanently stationary (e.g. fixed to a wall) and in NB-IoT only low mobility is supported and handovers are not supported. The NR use cases in that sense are different, i.e. the typical NR use case is MBB with full mobility support. But also in NR the UE can be expected to be stationary for significant amount of times during a day, similar as in LTE. When the UE is stationary, certain requirements, such as RRM measurement requirements can potentially be relaxed, e.g. there is no need to discover and measure neighbour cells when the UE is below the neighbour cell measurement threshold and stationary. When the UE is above the measurement threshold it is already allowed not to measure neighbour cells:

Proposal 1: RAN2 to evaluate the feasibility to relax the neighbour cell measurements for stationary NR UE in idle mode when it is below the measurement threshold.

It is proposed to evaluate a similar approach as has been agreed for NB-IoT/MTC, i.e. with control of the NW the UE determines when it is stationary. When the UE is stationary the UE may apply relaxed neighbour cell measurements. However when the UE is not stationary the existing RAN4 RRM measurement requirements apply. RAN2 should not change the RAN4 RRM requirements, but can modify the neighbour cell measurement rules defined in 38.304. Further safeguards may be required, such as periodic neighbour cell measurements in the background even the UE is stationary and neighbour cell measurements after cell (re-)selection.

Relaxed serving cell monitoring and WUS

The serving cell measurements in Idle mode are used to determine if neighbour cell measurement shall be performed. The serving cell measurements are used to detect if the cell is suitable to camp on, and if the UE can receive Paging and PWS notifications. The serving cell measurements are the basis for critical functions, i.e. it is proposed not to change/relax the serving cell measurement requirements in NR. Furthermore and perhaps most importantly, the UE has to wake to monitor paging every DRX cycle anyways, i.e. the expected power saving gains to relax serving cell measurements are small. The serving cell measurement requirements are captured in 38.133, i.e. these requirements are determined by RAN4:

Proposal 2: The serving cell measurement requirements in NR are not relaxed.

Long DRX cycle in Idle/Inactive mode

The longDRX-Cycle in connected mode can be up to 10 sec. As confirmed in the LS from SA2 concerning CIOT [1], there is no impact on CN paging to support a DRX cycle up to 10 sec. There is a significant impact on the power consumption when the DRX cycle is doubled. But perhaps not every UE can tolerate or prefers a DRX up to 10 sec. Therefore it is proposed to allow the NR UE to negotiate a UE specific long DRX cycle via NAS up to 10 sec, and allow the UE to use the negotiated UE specific DRX cycle (instead of the shortest of UE specific DRX cycle and the DRX cycle indicated in system information):

Proposal 3: RAN2 to study UE specific LongDRX up to 10 seconds in Idle/Inactive state in NR.

# Summary

RAN2 is kindly asked to discuss relaxed RMM measurements in Idle mode:

Proposal 1: RAN2 to evaluate the feasibility to relax the neighbour cell measurements for stationary NR UE in idle mode when it is below the measurement threshold.

Proposal 2: The serving cell measurement requirements in NR are not relaxed.

Proposal 3: RAN2 to study UE specific LongDRX up to 10 seconds in Idle/Inactive state in NR.

# References

- S2-1901343, LS on eDRX cycles for CM-CONNECTED with RRC inactive, LS out, To: CT1, CT4, RAN2, SA2#130

In NR the UE is required to measure the serving cell RSRP/RSRQ every DRX cycle, with a DRX cycle larger than 640 ms and SSB periodicity of 20 ms in FR1 (see chapter 4 in 38.113).