3GPP TSG-RAN WG2 Meeting #105	Tdoc R2-1901319

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	10.5.2

Source:	Ericsson

Title:	Email discussion summary [104#33][NR/Late drop] INMs (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document reports the following e-mail discussion:

[104#33][NR/Late drop] INMs (Ericsson)

Progress the detail of the parameters that would need to be added to the INMs in NR RRC to support MR-DC.

Intended outcome: Report and draft CR to next meeting

Deadline:  Monday 2019-02-07

# 2	Inter-node messages for MR-DC

## Inter-node message for NGEN-DC

The current inter-node messages (i.e., in particular CG-Config and CG-ConfigInfo) specified in TS 38.331 are used mainly for supporting EN-DC operations. Considering that the architecture of NGEN-DC is quite similar to that one of EN-DC, our understanding is that no modification on the inter-node messages is needed for supporting NGEN-DC. In fact, even if some something may be different between NGEN-DC and EN-DC (i.e., such as the content of RadioBearerConfig), this does not reflect to the inter-node messages. Thus, we have the following question:

Question 1: Confirm that no modifications on the inter-node messages are needed for supporting NGEN-DC (i.e., what we have is applicable to NGEN-DC).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes |  |
| ZTE | Yes |  |
| DOCOMO | Yes |  |
| Samsung | Yes | As indicated there are some changes to AS-Config but these do not require changes to the INM (covered by RadioBearerConfig) For all DC cases with 5GC, MN assigns DRB identity range to SN. We assume this will be handled by extensions of IEs ConfigRestrictInfoSCG and ConfigRestrictModReqSCG. I.e. there will be a need for change but not specific to NG EN-DC (can be covered elsewhere) |
| Turkcell | Yes |  |
| Huawei | Yes |  |

Rapporteur summary: All companies agree that no modifications on the inter-node messages are needed for supporting NGEN-DC.

No modifications on the inter-node messages are needed for supporting NGEN-DC.

## Inter-node messages for NE-DC

In the last RAN2#104 meeting, in the email discussion [103bis#11][NR] [1] [2] was discussed the introduction of late drop features into the 3GPP TS 36.331. Here, there was a proposal to specify the inter-node messages for NE-DC into the 36.331 and it was also provided an analysis of which fields where needed, and which ones of the current specification could be re-used. A similar solution was also addressed in [3], where the main proposal dealt with extending the inter-node messages SCG-Config and SCG-ConfigInfo in TS 36.331 to support NE-DC. On the contrary, in [4] the proposal was to have inter-node messages for NE-DC specified in NR-DC. After discussions, the following agreements has been made regarding inter-node messages for NE-DC:

Agreements

1:	Use SCG-ConfigPartSCG to cover the NE-DC configuration parameters i.e. as follows:

-	Re-use existing fields for mobilityControlInfoSCG, RLC bearer (for DRBs), MAC-Main and the physical configuration (SCG cells)

-	Introduce extensions for setup/ modification of RLC bearers (for SRB1/ 2), for release of RLC bearers (SRBs and DRBs) and for SN configured measurements

2:	Specify inter-node signalling for NE-DC in NR RRC.

Referring to the agreement above, our understanding is that it was to decide the usage of the fields in SCG-ConfigPartSCG originally defined for LTE-DC for NE-DC, i.e. existing fields for mobilityControlInfoSCG, RLC bearer (for DRBs), MAC-Main and the physical configuration (SCG cells) will be reused. Then extensions are needed for setup/ modification of RLC bearers (for SRB1/ 2), for release of RLC bearers (SRBs and DRBs) and for SN configured measurements, as these were not covered by LTE-DC. In addition, fields for RadioBearerConfig in NR RRC Inter-Node messages can be reused.

However, the agreement above does not change the fact that the full LTE RRCConnectionReconfiguration will be used to carry the secondary cell group configuration in inter node message in NE-DC (i.e., embedded in a container  OCTECT STRING in the ASN.1). This is because in the last RAN2 meetings it was agreed to re-use the EN-DC framework for both NE-DC and NR-DC. Therefore, we would kindly ask companies to confirm that full LTE RRC message (RRCConnectionReconfiguration) in the INM for NE-DC is embedded for the secondary cell group.

Question 2: Confirm that the full LTE RRC message (i.e., RRCConnectionReconfiguration) in the INM for NE-DC is used for the secondary cell group.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes |  |
| ZTE | Yes |  |
| DOCOMO | Yes |  |
| Samsung | Yes | The RRCConnectionReconfiguration message is used but merely contains scg-Configuration (or to be precise scg-ConfigPartSCG and its extensions) |
| Turkcell | Yes |  |
| Huawei | Yes | We agree with Samsung. |

Rapporteur summary: All companies agree that the full LTE RRC message (i.e., RRCConnectionReconfiguration) in the INM for NE-DC is used for the secondary cell group.

The full LTE RRC message (i.e., RRCConnectionReconfiguration) in the INM for NE-DC is used for the secondary cell group.

According to what has been provided in [1] [2], we believe that the main principles of what fields are needed for NE-DC inter-node messages are still valid. Therefore, taking into account the previous contributions in [2] we provided a DraftCR where the fields (shown in the tables below, with relative updated field description when necessary) that we believe are necessary to introduce inter-node messages for the NE-DC option have been added.

Companies are kindly asked to provide further considerations/ motivation/ details indicating the items/fields for which the comment applies. Also, additional items/fields that is believed to be needed for NE-DC may be provided. For this latter case, we kindly ask companies to provide also the motivation on why the field is necessary.

- HandoverPreparationInformation

| Field | Comments | Type of change |
| --- | --- | --- |
| HandoverPreparationInformation |  |  |
| candidateCellInfoListEUTRA | This includes the MeasResultList3EUTRA that is within the MeasResultSCG-Failure belonging to 36.331. We note that the information element MeasResultSCG-Failure is not in place yet in 36.331 but we need it in case a SCG failure is detected in NE-DC (i.e., if we assume to re-use the same solution as in EN-DC). | New field Note that MeasResultSCG-Failure (to be implemented in 36.331) re-use the same format/fields as used in 38.331. |

Question 3: Do companies agree with the changes listed in the table above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes, basically | One comment to the draft CR. It would be better to align with the EN-DC case, where the following field description is added for NR measurements: candidateCellInfoListNR A list of NR cells including serving cells and best neighbour cells on each SSB requency, for which measurement results were available, and for each cell the best beams. We guess for the NE-DC, the following new (independent) field description seems more appropriate, especially to clarify the inclusion of serving cells. candidateCellInfoListEUTRA A list of EUTRA cells including serving cells and best neighbour cells on each serving frequency, for which measurement results were available. The field candidateCellInfoListEUTRA includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Ericsson: We have added a separate description for candidateCellInfoListEUTRA in the provided draft CR on INMs. |
| ZTE | Partially Yes | We agree to add candidateCellInfoListEUTRA field, but we have following comments: Similar to candidateCellInfoListNR, in our understanding, the above suggested candidateCellInfoListEUTRA is mainly used to trigger “inter-MN handover with SN change” procedure. So, the results included in candidateCellInfoEUTRA are derived from UE NR measurement report in case of NE-DC, such as the best cells on each LTE serving frequency. Since it is encoded in NR RRC format, the “OCTECT STRING” definition is not needed in the draft CR. Moreover, The detail definition of candidateCellInfoEUTRA can reuse the structure when it is defined in measResults in TS38.331 (to be implemented). Ericsson: Same as for EN-DC, where MeasResultList2NR was used, we have adopted the source adapts target principle. Thus, for NE-DC, candidateCellInfoEUTRA is defined as OCTET STRING because this contain a field (i.e., MeasResultList3EUTRA) that is specified in TS 36.331 (i.e., according to the CR provided in the email discussion [104#65][NR] is within MeasResultSCG-FailureMRDC). Further, we cannot re-use the existing field MeasResultList2NR as it only includes NR available measurements. |
| DOCOMO | Yes | Fine with the second version CR. |
| Samsung | Yes, but | We were initially assuming the octet string and LTE encoding, but agree with ZTE that upon change of MN (HO) with EN-DC, the NR candidates are actually encoded by LTE RRC (i.e. field candidateCellInfoListNR-r within RRM-Config). I.e. in LTE MN includes NR measurement results that may be included in the LTE MR that triggered the concerned HO (i.e. with reportAddNeigh set). I.e. these NR measurement results are provided by UE in LTE RRC encoding. Hence, we think it is appropriate to be consistent with EN-DC i.e. to use NR encoding. |
| Turkcell | Yes | Updated CR is suitable for us. |
| Huawei | Yes | We agree with the comment on candidateCellInfoListEUTRA, it is fine with the updated CR on Feb. 13. |

Companies are welcome to comment on current issues or propose new fields below.

| Company | Issue/Field | Comments |
| --- | --- | --- |
| ZTE | as-ConfigEUTRA in AS-Config | For inter-MN handover with SN change, to facilitate the target SN to generate delta configuration, the old SCG context (e.g. as-ConfigEUTRA) should also be added in HandoverPreparationInfo (similar to as-ConfigNR-r15 in 36.331). The content of as-ConfigEUTRA includes two transparent containers (OCTET STRING): one is RRCConnectionReconfiguration message, the other is RadioBearerConfig. Ericsson: Our understanding is that the target MN can retrieve the SCG context/configuration from the field rrcReconfiguration within AS-Config. In fact, this field is an OCTET STRING containing RRCReconfiguration where are included the SCG configurations coming from the SN for both NR and EUTRA according to the draft CR provided in the email discussion [104#64][NR]. Samsung: We agree to the comment from ZTE i.e. for NE-DC the AS configuration should cover the configuration set by LTE SN i.e. one field being an octet string carrying NR RadioBearerConfig and another field being an octet string carrying LTE reconfiguration message (alike similar fields in LTE RRC i.e. see AS-ConfigNR-r15). We understand that Ericsson proposes for the cellGroup and measurement configuration to use one field/ octet string with the actual message contained depending on the case/ the source. Is this the only case for such approach? |
| Ericsson | selectedBandCombinationSN | In NR-DC, the target gNB needs to know the band(s) on which the SN has been configured, thus the selected band combination should be added within the AS-Context. The field we added in the draft CR for INMs is called selectedBandCombinationSN. |
| DOCOMO | As-configEUTRA in AS-conifg | Agree with ZTE’s view that AS-conifgEUTRA should be added similar as as-ConfigNR-r15 in As-config in 36.331 |
| Turkcell | as-ConfigEUTRA in AS-Config | We agree with ZTE comment for as-ConfigEUTRA |
| Huawei | as-ConfigEUTRA in AS-Config | We also agree with ZTE and see this was updated in the Feb13 version of the CR. |

Rapporteur summary: There was a general agreement for the proposed changes among companies. However, some remarks were made regarding the following:

- candidateCellInfoListEUTRA. As pointed out ZTE and Samsung, for EN-DC, the list of candidate NR cells are communicated from source to target MN in the same LTE format as received from the UE. For NE-DC, the same approach can be applied. In 38.331 late drop email discussion #65, the IE MeasResultServFreqListEUTRA is introduced, which can be used to provide the E-UTRA measurements. We have updated the INM CR accordingly.

- As-ConfigEUTRA. Several companies suggested to include a new field AS-ConfigEUTRA in AS-Config to cover the EUTRA context to be forwarded from source to target MN during MN handover in NE-DC. The proposal is that field AS-ConfigEUTRA should include two fields as OCTET-STRING, one to carry RRCConnectionReconfiguration message including the SCG and measurement configurations, the other to carry RadioBearerConfig generated by the SN. This would be similar to as-ConfigNR-r15 in 36.331. Our understanding of this proposal is the ASN.1 snippet below.

AS-ConfigEUTRA-r15 ::=              SEQUENCE {

sourceRB-ConfigSN-EUTRA-r15         OCTET STRING            OPTIONAL,

sourceSCG-Config-EUTRA-r15          OCTET STRING            OPTIONAL

}

This could be one approach, but it would mean duplication of information in the inter node message. This is because AS-Config is defined differently in EUTRA and NR. In EUTRA, AS-Config includes all IEs required for delta signaling during inter MN handover, listed separately on top level. In this case, it was necessary to add the fields containing the SCG and radio bearer configurations for EN-DC, as they were not there before. In NR, AS-Config contains just the field rrcReconfiguration containing RRCReconfiguration, see ASN.1 snippet below:

AS-Config ::=             SEQUENCE {

rrcReconfiguration                  OCTET STRING (CONTAINING RRCReconfiguration),

...

}

With the introduction of late drop in running CR for 38.331 discussed in email discussion #65, RRCReconfiguration is being extended to include a new fields to enable the configuration of both NE-DC and NR-DC. For this purpose, radioBearerConfig2 is added to carry RadioBearerConfig generated by the SN. There is thus no need for a separate field in AS-Config to convey this information. Same goes for the SCG configuration, where a new field mrdc-SecondaryCellGroup is introduced, using a CHOICE structure to contain either the nr-SCG or eutra-SCG. Thus, we see no need to extend AS-Config in order to support late drop options. We are not sure all responding companies made above considerations, so we propose RAN2 to discuss whether AS-Config in 38.331 needs to be extended in order to support delta configuration during inter MN handover with SN change.

Based on the input the following is proposed:

The NR IE MeasResultServFreqListEUTRA is used to provide the EUTRA measurement results in field candidateCellInfoListEUTRA during inter MN handover.

RAN2 to discuss whether AS-Config in 38.331 needs to be extended in order to support delta configuration during inter MN handover with SN change.

- CG-Config

| Field | Comments | Type of change |
| --- | --- | --- |
| CG-Config |  |  |
| selectedBandCombination | We intend to reuse the existing field named selectedBandCombinationNR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| scg-CellGroupConfigEUTRA | According to the agreement made in RAN2#104, this is an OCTECT STRING that includes the LTE RRCConnectionReconfiguration. | New field added according to agreement made in RAN2#104 |
| candidateCellInfoListSN-EUTRA | This includes the MeasResultList3EUTRA that is within the MeasResultSCG-Failure belonging to 36.331. We note that the information element MeasResultSCG-Failure is not in place yet in 36.331 but we need it in case a SCG failure is detected in NE-DC (i.e., if we assume to re-use the same solution as in EN-DC). | New field |
| candidateServingFreqListEUTRA |  | New field |
| needForGaps | For NE-DC and NR-DC we agreed that the MN configures perFR1, perFR2, and perUE gaps, and, in case of NE-DC, the SN sends the gap request to the MN. | New field added according to the agreement made in RAN2#104 |
| CandidateServingFreqListEUTRA |  | New IE |

Question 4: Do companies agree with the changes listed in the table above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes, | Looking at the draft CR, the intention seems to define as follows: needForGaps ENUMERATED {true}, OPTIONAL, However, the agreement was that the MN configures the gap regardless of the gap type, while the SN sends the gap request to the MN. The latter part (i.e. SN request) is the same as the perUE gap or FR1 gap of perFR gap in EN-DC, for which such explicit request is not sent but “MeasConfigSN (including measuredFrequenciesSN)“ is sent for imlicitly indicating a request for gap. Our assumption was to reuse this way as well and will add new field under the MeasConfigSN (including e.g. measuredFrequenciesSN-EUTRA) or define e.g. MeasConfigSN-EUTRA. We are open for further discussions, if many companies support the explicit indication. => we take back this comment thanks to the clarification by ZTE |
| ZTE | FFS or No | After going through the current fields in CG-Config, except scg-RB-Config, configRestrictModReq and drx-InfoSCG, all the remaining fields are not applicable to the NE-DC case. Since we have to introduce a lot LTE related fields, isn't it cleaner and more future proof to define another CG-Config branch for NE-DC? See below example: CG-Config ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ cg-Config CG-Config-IEs, cg-ConfigEUTRA CG-ConfigEUTRA-IEs, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } With this change, we don’t have to specify the absence/presence principle for each specific field in the future. Ericsson: Having a dedicated branch for NE-DC and NR-DC is another option on how to implement INMs. However, we preferred to implement INMs for NE-DC and NR-DC as an extension of the existing CG-Config and CG-ConfigInfo. In this way we can avoid to have duplicate fields in case those are used by both NE-DC and NR-DC but in general, by all the MR-DC options. For the sake of having a more clear implementation, in the fields description we have added a clarification on if a certain field is (not) supported for a given MR-DC option. Samsung: we think current approach is the baseline i.e. we decided to use NR RRC for these INMs because of re-use of existing fields. When considering a separate branch aproach we should have proper overview of fields re-used and fields replaced e.g. by LTE versions (maybe it’s like 4 re-used and 6 replaced?) Regarding comment from NEC, according to the agreement in RAN2#103bis, the measured LTE frequency list will not be delivered for gap coordination, because the period of LTE CRS is fixed. We think the current CR reflects the agreement correctly. 8 Measurement gap coordination for NE-DC: MN->SN: per UE or FR1 gap pattern SN->MN: gap request (without frequency list) |
| DOCOMO | Yes |  |
| Samsung |  | drx-InfoMCG: we think the existing field would be re-used reportCGI-Request: we think an LTE specific field should be introduced We are not entirely sure about ph-InfoSCG. CandidateServingFreqListEUTRA appears twice in table |
| Turkcell | Yes |  |
| Huawei | Yes | Agree with Samsung on LTE specific field for CGI request (see below). |

Companies are welcome to comment on current issues or propose new fields below.

| Company | Issue/Field | Comments |
| --- | --- | --- |
| ZTE | pSCellFrequency for LTE frequency | To facilitate SFTD measurement on PSCell, SN can forward the LTE frequency of PSCell to MN, the current definition only includes NR frequency. For NE-DC, LTE frequency should be indicated. NEC: agree Ericsson: We agree that PSCell frequencies need to be forwarded from the SN to the MN. We have added the field pSCellFrequencyEUTRA to cover this aspect Samsung: agree |
| Samsung | candidateCellInfoListSN-EUTRA | In ASN.1 we should just have an octet string (i.e. no containing as concerned IE is defined in LTE RRC) Ericsson: Agree, we updated the draft CR accordingly. |
| Huawei | reportCGI-RequestEUTRA | to request E-UTRA CGI reporting |
| Huawei | requestedP-MaxEUTRA: | to inform the MgNB of the requested value for the maximum power for the serving cell the UE can use in LTE SCG |

Rapporteur summary: There was a general agreement for the proposed changes among companies. However, some remarks were made regarding the following:

- reportCGI-Request: One company thought an LTE specific field covering EUTRA frequencies should be introduced. Rapporteur tend to agree. This could be achieved by introducing a new field reportCGI-RequestEUTRA, containing EUTRA specific fields. The suggestion is to let RAN2 to discuss whether this field should be added.

- pSCellFrequency: Several companies agreed this is needed also for EUTRA frequencies in order to support SFTD measurements. The field pSCellFrequencyEUTRA has been added in the draft CR on INM.

RAN2 to discuss whether to add in CG-Config a new field reportCGI-RequestEUTRA, containing EUTRA specific fields.

- CG-ConfigInfo

| Field | Comments | Type of change |
| --- | --- | --- |
| CG-ConfigInfo |  |  |
| candidateCellInfoListMN-EUTRA | This includes the MeasResultList3EUTRA that is within the MeasResultSCG-Failure belonging to 36.331. We note that the information element MeasResultSCG-Failure is not in place yet in 36.331 but we need it in case a SCG failure is detected in NE-DC (i.e., if we assume to re-use the same solution as in EN-DC). | New field Note that MeasResultSCG-Failure (to be implemented in 36.331) re-use the same format/fields as in 38.331. |
| candidateCellInfoListSN-EUTRA | This includes the MeasResultList3EUTRA that is within the MeasResultSCG-Failure belonging to 36.331. We note that the information element MeasResultSCG-Failure is not in place yet in 36.331 but we need it in case a SCG failure is detected in NE-DC (i.e., if we assume to re-use the same solution as in EN-DC). | New field Note that MeasResultSCG-Failure (to be implemented in 36.331) re-use the same format/fields as in 38.331. |
| sourceConfigSCG-EUTRA | According to the agreement made in RAN2#104, this is an OCTECT STRING that includes the LTE RRCConnectionReconfiguration. | New field added according to agreement made in RAN2#104 |
| scgFailureInfoEUTRA | Sequence needed to forward failure type and measurements in case an SCG failure is detected. | New field |
| failureTypeEUTRA |  | New field |
| measResultSCG-EUTRA | This includes the MeasResultSCG-Failure IE belonging to 36.331. We note that the information element MeasResultSCG-Failure is not in place yet in 36.331 but we need it in case a SCG failure is detected in NE-DC (i.e., if we assume to re-use the same solution as in EN-DC). | New field |
| maxMeasFreqsSCG | We intend to reuse the existing field named maxMeasFreqsSCG-NR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| maxMeasIdentitiesSCG | We intend to reuse the existing field named maxMeasIdentitiesSCG-NR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| powerCoordinationFR2 | Indicated the maximum power that the UE can use in FR2. This field is only used in NR-DC, since only NR-DC can deploy FR2 in both MCG and SCG. | New field |
| p-maxNR-FR2 | Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 2 (FR2) (see TS 38.104 [12]) the UE can use in NR SCG. This field is only used in NR-DC. | New field |
| p-maxUE-FR2 | Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2). This field is only used in NR-DC. | New field |

Question 5: Do companies agree with the changes listed in the table above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes, other than power config for FR2 | Since the SN (i.e. EUTRA) in NE-DC does not use the FR2, we are not sure if the following information are needed. To our understanding, not needed. - powerCoordinationFR2 - p-maxNR-FR2 - p-maxUE-FR2 Ericsson: Our understanding is that those fields are only needed for the NR-DC option when MCG and SCG are deployed on a set of frequencies belonging to FR2. Thus, it is correct that they are not applicable to NE-DC. |
| ZTE | FFS or No | Similar to the Q4, we think it is clearer and cleaner to define another CG-ConfigInfo branch for NE-DC, which applies for both NE-DC and NR-DC, see below example: CG-ConfigInfo ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ cg-ConfigInfo CG-ConfigInfo-IEs, cg-ConfigInfoNR CG-ConfigInfoNR-IEs, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } Ericsson: Having a dedicated branch for NE-DC and NR-DC is another option on how to implement INMs. However, we preferred to implement INMs for NE-DC and NR-DC as an extension of the existing CG-Config and CG-ConfigInfo. In this way we can avoid to have duplicate fields in case those are used by both NE-DC and NR-DC but in general, by all the MR-DC options. For the sake of having a more clear implementation, in the fields description we have added a clarification on if a certain field is (not) supported for a given MR-DC option. Samsung: See our response to Q4 Regarding the listed fields, we have following comments: candidateCellInfoListMN-EUTRA In our understanding, this is derived from MN(NR) Bx measurement report, which is sent to SN for LTE PSCell selection, and MN is responsible for filling this field. So the definition should not be OCTECT STRING. Ericsson: Same as for EN-DC, where MeasResultList2NR was used, we have adopted the source adapts target principle. Thus, for NE-DC, candidateCellInfoListMN-EUTRA is defined as OCTET STRING because this contains a field (i.e., MeasResultList3EUTRA) that is specified in TS 36.331 (i.e., according to the CR provided in the email discussion [104#65][NR] is within MeasResultSCG-FailureMRDC). Samsung: Note that change to another SN may be initiated by MN upon receiving a) MR or b) SCG failure, or by c) SN (with MN involved). In EN-DC we agreed to have 3 containers i.e. one set by MN i.e. case a), one generated by SN and transparently forwarded i.e. case c) and one to carry the results provided by UE upon SCG failure info (case b). In EN-DC the fields all apply NR encoding i.e. adjusted to target SN. In case of NE-DC this would mean LTE format. Assuming we follow the same principle, this would mean: - MN generated field using LTE encoding (not within octet string) - SN generated field using LTE encoding within octet string (i.e. source adjusts) - Field for SCG failure results using LTE encoding within octet string) Ericsson: We agree with Samsung, except for for MN generated field using LTE encoding where OCTET-STRING is still needed as the measurement is defined in another spec (36.331). maxMeasIdentitiesSCG According to the conclusion in RAN2#104, this field is not used for EN-DC. Similarly, we think this is not used for NE-DC unless RAN4 defines different UE measurement requirements. => RAN2 understand the maxMeasIdentitiesSCG-NR between MN and SN is not used for EN-DC. Ericsson: Regarding this issue we think that input from RAN4 is needed to decide whether maxMeasIdentitiesSCG is needed or not. Therefore, we propose to keep the use of this field as FFS. We plan to draft an LS asking RAN4 for inputs on this issue. powerCoordinationFR2, p-maxNR-FR2, p-maxUE-FR2 We also think these fields are not needed for NE-DC. Ericsson: Our understanding is that those fields are only needed for the NR-DC option when MCG and SCG are deployed on a set of frequencies belonging to FR2. Thus, it is correct that they are not application to NE-DC. |
| DOCOMO | Yes | Fine with second version CR |
| Samsung |  | configRestrictInfo: we think some more fields are re-used i.e. allowedBC-ListMRDC, servCellIndexRangeSCG. We share the view that power coordination field for FR1 are not needed for NE-DC drx-InfoSCG: we think the existing field is re-used measConfigMN: existing field can be re-used but some clarification to be provided that MN just includes measGapConfig measResultReportCGI: we think an LTE specific field should be introduced measResultSFTD: we think existing field is re-used affectedCarrierFreqCombIDC: we think existing field is re-used We are not entirely sure about ph-InfoSCG |
| Turkcell | Yes | The updated version is suitable for us. |
| Huawei | Yes (but see comment) | configRestrictInfo: p-MaxEUTRA can be reused to inform the SeNB of the maximum total transmit power to be used by the UE for the E-UTRA SCG. |

Companies are welcome to comment on current issues or propose new fields below.

| Company | Issue/Field | Comments |
| --- | --- | --- |
| Samsung | candidateCellInfoListXN-EUTRA, measResultSCG-EUTRA | In ASN.1 we should just have an octet string (i.e. no containing as concerned IE is defined in LTE RRC) Ericsson: Agree. CR has been updated accordingly |
| Huawei | measResultReportCGI-EUTRA | For CGI report results |

Rapporteur summary: There was a general agreement for the proposed changes among companies. However, some remarks were made regarding the following:

- maxMeasIdentitiesSCG. There was a question whether this is needed for NE-DC, as it is not used for EN-DC. We suggest RAN2 to send LS to RAN4 asking whether separate pools of measurement identities can be used for MCG and SCG in NGEN-DC, NE-DC and NR-DC. We have provided a draft LS in [ref] that can be used as basis.

- measResultReportCGI. One company thought an LTE specific field covering EUTRA frequencies should be introduced. Rapporteur tend to agree. This could be achieved by introducing a new field measResultReportCGI-EUTRA, containing EUTRA specific fields. The suggestion is to let RAN2 to discuss whether this field should be added.

RAN2 to discuss whether to add in CG-ConfigInfo a new field measResultReportCGI-EUTRA, containing E-UTRA specific fields.

RAN2 to discuss whether to send an LS to RAN4 asking whether separate pools of measurement identities can be used for MCG and SCG in NGEN-DC, NE-DC and NR-DC.

Further, the following FFS stated in the chairman notes of RAN2#104 has been added regarding INMs for NE-DC:

- FFS To introduce VoiceOverSCG-Bearer for NE-DC SCG bearer in either MR-DC capability or NR SA UE capability.

  - Comment: This FFS overlaps with email discussion [104#66][NR] about capabilities in MR-DC. Therefore, this FFS will be discussed in email discussion [104#66][NR] UE NR and E-UTRA capabilities for Late drop (Ericsson).

Companies are welcome to add further FFSs (Note, FFSs on capabilities should be discussed in email discussion [104#66][NR]):

| Company | FFS | Comments |
| --- | --- | --- |

## Inter-node messages for NR-DC

As for NE-DC, in the provided DraftCR the following fields (shown in the table below) have been added for supporting inter-node message for NR-DC with relative updated field description, when necessary.

According to these new fields and paraments introduced in the table above, companies are kindly asked to provide further considerations/ motivation/ details indicating the items/fields for which the comment applies. Also, additional items/fields that is believed to be needed for NR-DC may be provided. For this latter case, we kindly ask companies to provide also the motivation on why the field is necessary.

- CG-Config

| Field | Comments | Type of change |
| --- | --- | --- |
| CG-Config |  |  |
| selectedBandCombination | We intend to reuse the existing field named selectedBandCombinationNR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| requestedP-MaxFR2 | Requested value for the maximum power for the serving cells on frequency range 2 (FR2) in this secondary cell group the UE can use in NR SCG | New field |

Question 6: Do companies agree with the changes listed in the table above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes |  |
| ZTE | Yes | Compared to our comment in Q4, for NR-DC, we think the current CG-Config-IEs can be reused, because the sending node is NR, same as in EN-DC. |
| DOCOMO | Yes |  |
| Samsung | Yes |  |
| Turkcell | Yes |  |
| Huawei | Yes |  |

Companies are welcome to comment on current issues or propose new fields below.

| Company | Issue/Field | Comments |
| --- | --- | --- |
| Ericsson | drx-InfoSCG | This field contains the NR IE DRX-info, which codepoints for long and short DRX cycle is a superset of the EUTRA codepoints. This field can be reused for all MR-DC option. NOTE Since the field is already present in the current version of the specification, a field description is added in the draft CR on INM to clarify this aspect. |
| ZTE | drx-InfoSCG | We think slotOffset is missing in current drx-InfoSCG field. For NR-DC, the DRX alignment between MN and SN can be more precise. Ericsson: We agree that slotOffset within DRX-Info is missing. In our draft CR on INMs for MR-DC we have added a new field, i.e., drx-ConfigSCG, that is pointing to DRX-Config (i.e., that includes the complete DRX configuration). |
| ZTE | needForGaps | Regarding the rapporteur’s explanation of this field in section 2.2, in our understanding, this field is not needed in NR-DC according to the RAN2 agreement. For NR-DC, “measuredFrequenciesSN” field is reused to inform MN about the measured NR frequencies, so MN can generate suitable gap location. Ericsson: To avoid such confusion, in our draft CR on INMs in each field description we have added a clarification on if a certain field is (not) supported for a given MR-DC option. |

Rapporteur summary: There was a general agreement for the proposed changes among companies. There was some discussion on detail fields and the CR has been updated to address these.

Companies agree with the proposed changes to CG-Config to support NR-DC.

- CG-ConfigInfo

| Field | Comments | Type of change |
| --- | --- | --- |
| CG-ConfigInfo |  |  |
| maxMeasFreqsSCG | We intend to reuse the existing field named maxMeasFreqsSCG-NR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| maxMeasIdentitiesSCG | We intend to reuse the existing field named maxMeasIdentitiesSCG-NR. Our proposal is to delete the suffix “NR” since this field is applicable to all the MR-DC options. | Re-use existing field with different name |
| powerCoordinationFR2 | Indicated the maximum power that the UE can use in FR2. | New field |
| p-maxNR-FR2 | Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 2 (FR2) (see TS 38.104 [12]) the UE can use in NR SCG. | New field |
| p-maxUE-FR2 | Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2). | New field |
| gapPurposeFR2 | Regarding the gap pattern, this is already inside the GapConfig IE so no need to signal explicitly within the INMs. Also, this field is only applicable to NR-DC. | New field added according to the agreement made in RAN2#104 |

Question 7: Do companies agree with the changes listed in the table above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes |  |
| ZTE | No | Similar to our comments in Q5, we suggest to introduce a new CG-ConfigInfoNR branch for NR-DC and NE-DC (both sending nodes are NR). Ericsson: Having a dedicated branch for NE-DC and NR-DC is another option on how to implement INMs. However, we preferred to implement INMs for NE-DC and NR-DC as an extension of the existing CG-Config and CG-ConfigInfo. In this way we can avoid to have duplicate fields in case those are used by both NE-DC and NR-DC but in general, by all the MR-DC options. For the sake of having a more clear implementation, in the fields description we have added a clarification on if a certain field is (not) supported for a given MR-DC option. Regarding above listed fields, we have following comments: maxMeasIdentitiesSCG Similar to our comments in Q5, whether this is useful depends on the UE measurement requirement defined by RAN4. So far, RAN4 haven’t discussed this. If RAN4 defines separate maximum measID requirements for MN and SN (as in EN-DC), then this field is not needed. Ericsson: Regarding this issue we think that inputs from RAN4 is needed to decide wether maxMeasIdentitiesSCG is needed or not. Therefore, we propose to keep the use of this field as FFS. We plan to draft an LS asking RAN4 for inputs on this issue. gapPurposeFR2 In case of per-FR gap, MN may configure up to two gap patterns (i.e. FR1gap and FR2gap), but currently, there’s only one measGapConfig. Although in Rel-15 the MCG operates on FR1 and SCG operates on FR2, do we need to add another measGapConfig for FR2 to be future proof? In this case, the gapPurposeFR2 is not needed if it is implicitly indicated by field name (e.g. measGapConfig-fr2). Ericsson: We agree with the comment. In our draft CR on INMs we added a field named measGapConfigFR2 for the case when measurements gaps are configured on FR2 and we deleted the field gapPurposeFR2 as it is not needed. Samsung: We think we can leave signaling of a second gap configuration to the future. Nothing really needs to be done (could generalize the existing gap purpose value to FRx with meaning depending on DC case |
| DOCOMO | Yes |  |
| Samsung | Yes |  |
| Turkcell | Yes |  |
| Huawei | Yes |  |

Companies are welcome to comment on current issues or propose new fields below.

| Company | Issue/Field | Comments |
| --- | --- | --- |
| Ericsson | drx-InfoMCG | This field contains the NR IE DRX-info, which codepoints for long and short DRX cycle is a superset of the EUTRA codepoints. This field can be reused for all MR-DC option. NOTE Since the field is already present in the current version of the specification, a field description is added in the draft CR on INM to clarify this aspect. |
| ZTE | drx-InfoMCG | We think slotOffset is missing in current drx-InfoSCG field. For NR-DC, the DRX alignment between MN and SN can be more precise. Ericsson: We agree that slotOffset within DRX-Info is missing. In our draft CR on INMs for MR-DC we have added a new field, i.e., drx-ConfigMCG, that is pointing to DRX-Config (i.e., that includes the complete DRX configuration). |
| ZTE | scgFailureInfo | The SCG failure handling upon NR-DC has not been discussed yet. At least, we think the measResultSCG field can be enhanced according to the issue spotted in R2-1813986. Ericsson: We think that the issue spotted in R2-1813986 is not directly related to the INMs but rather to an IE that is used. In our opinion, this issue should be raised in the late drop email discussion [104#64][NR] on 38.331. |
| DOCOMO | scgFailureInfo | For NR-DC SCG failure, the enhancement of MeasReusltSCG-failure can be discussed |

Rapporteur summary: There was a general agreement for the proposed changes among companies. However, some remarks were made regarding the following:

- scgFailureInfo. There was some discussion about the content of this field, and whether it could be enhanced for NR-DC. Enhancement of IE MeasReusltSCG-failure was mentioned, but though it is carried in INM, the issue is not directly linked to INM. No detailed proposals were presented, thus there is no clear proposal to raise.

Companies agree with the proposed changes to CG-ConfigInfo to support NR-DC.

Possible further enhancements of MeasReusltSCG-failure may be discussed in RAN2.

Further, the following FFSs have been identified and added regarding INMs for NR-DC:

- FFS1 If power coordination in NR-DC is handled differently from EN-DC.

  - Comment: RAN1 understanding is that power coordination of EN-DC is re-used in NE-DC [5]. Only change will be UE capability signaling where there will be separate capability for dynamic power sharing for NE-DC and EN-DC. For NR-DC, in Rel15 late drop (i.e., sync, and MCG in FR1 and SCG in FR2) there is no power sharing between FR1 and FR2 [6] so most likely there is no need for extra signaling for this case as well. For the case where FR2 is used in both MCG and SCG, there is a need for power coordination for NR-DC. For this purpose, new fields are introduced in CG-ConfigInfo and CG-Config.

- FFS2 How to signal restricted bands in a band combination to be used for the SN in NR-DC.

  - Comment: This FFS was addressed in the contributions [7] [8] and the main proposal was to include then MCG configuration in CG-ConfigInfo. However, this approach was not agreed. The signalling of the share of NR-DC bands between MN and SN will depend on how the NR-DC band combinations are defined in RAN4. This FFS will be discussed in email discussion [104#66][NR] UE NR and E-UTRA capabilities for Late drop (Ericsson).

- FFS3 PDCCH blind decoding capability coordination in NR-DC.

  - Comment: This FFS was addressed was addressed in contributions[9] [10]. However, the contributions where not discussed in the last RAN2 meeting. This FFS will be discussed in email discussion [104#66][NR] UE NR and E-UTRA capabilities for Late drop (Ericsson).

Question 8: Do companies agree with the conclusion on FFS1 above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| NEC | Yes | but only for FR1 as commented to the Question 5. |
| ZTE | Yes |  |
| DOCOMO | Yes |  |
| Turkcell | Yes |  |

Rapporteur summary: All responding companies agreed with the conclusion for FFS1.

For NR-DC, in Rel15 late drop (i.e., sync, and MCG in FR1 and SCG in FR2) there is no power sharing between FR1 and FR2, and thus no need for extra inter node signaling. For the case where FR2 is used in both MCG and SCG, there is a need for power coordination for NR-DC. For this purpose, new fields are introduced in CG-ConfigInfo and CG-Config.

Companies are welcome to add further FFSs in the table below (Note, FFSs on capabilities should be discussed in email discussion [104#66][NR]):

| Company | FFS | Comments |
| --- | --- | --- |

## 2.5	Other

Companies are welcome to add further FFSs concerning other issues not related to CG-Config and CG-ConfigInfo:

| Company | FFS | Comment |
| --- | --- | --- |
| ZTE | as-ConfigSCG in HandoverPreparationInfo | For NR-DC UE, in order to support inter-MN handover with SN change, the old SCG context should be added in HandoverPreparationInfo message. And the definition of this field is OCTET STRING, which includes the entire NR RRCReconfiguration message. Ericsson: Our understanding is that the target MN can retrieve the SCG context/configuration from the field rrcReconfiguration within AS-Config. In fact, this field is an OCTET STRING containing RRCReconfiguration where are included the SCG configurations coming from the SN for both NR and EUTRA according to the draft CR provided in the email discussion [104#64][NR]. |
| ZTE | candidateCellInfoListSN in HandoverPreparationInfo | For NR-DC UE, in order to support inter-MN handover with SN change, the measurement results related to target SN should be added in HandoverPreparationInfo. Although the definition might be same with current candidateCellInfoList field, do we need a separate IE to differentiate MCG candidates and SCG candidates? |
| NEC | candidateCellInfoListSN in HandoverPreparationInfo | Agree with ZTE, it is good to discuss this. We think it would be better to add this as the new field, because to our understanding, candidate cell list for SN/SCG can include a serving cell which may not be a best cell on the corresponding frequency. This is the same comment as Question 3. Ericsson: Agree with ZTE and NEC. In our draft CR on INMs we have added a choice structure that includes candidateCellInfoListEUTRA and candidateCellInfoListSN. |

Rapporteur summary: Some more issues were discussed, and the necessary updates were made in the draft CR. To conclude the email discussion, the proposal is for RAN2 to agree the draft CR for INM in [11] to be included into the running CR for 38.331 late drop.

The draft CR on INM to be included in the running CR for 38.331 late drop.

# 3	Conclusion

Rapporteur would like to thank all companies participating in the email discussion. Thanks to many good comments we have increased the quality of the draft CR. Based on the discussion, the following observations and proposals are made:

Observation 1	No modifications on the inter-node messages are needed for supporting NGEN-DC.

Observation 2	The full LTE RRC message (i.e., RRCConnectionReconfiguration) in the INM for NE-DC is used for the secondary cell group.

Observation 3	The NR IE MeasResultServFreqListEUTRA is used to provide the EUTRA measurement results in field candidateCellInfoListEUTRA during inter MN handover.

Observation 4	Companies agree with the proposed changes to CG-Config to support NR-DC.

Observation 5	Companies agree with the proposed changes to CG-ConfigInfo to support NR-DC.

Observation 6	Possible further enhancements of MeasReusltSCG-failure may be discussed in RAN2.

Observation 7	For NR-DC, in Rel15 late drop (i.e., sync, and MCG in FR1 and SCG in FR2) there is no power sharing between FR1 and FR2, and thus no need for extra inter node signaling. For the case where FR2 is used in both MCG and SCG, there is a need for power coordination for NR-DC. For this purpose, new fields are introduced in CG-ConfigInfo and CG-Config.

Proposal 1	RAN2 to discuss whether AS-Config in 38.331 needs to be extended in order to support delta configuration during inter MN handover with SN change.

Proposal 2	RAN2 to discuss whether to add in CG-Config a new field reportCGI-RequestEUTRA, containing EUTRA specific fields.

Proposal 3	RAN2 to discuss whether to add in CG-ConfigInfo a new field measResultReportCGI-EUTRA, containing E-UTRA specific fields.

Proposal 4	RAN2 to discuss whether to send an LS to RAN4 asking whether separate pools of measurement identities can be used for MCG and SCG in NGEN-DC, NE-DC and NR-DC.

Proposal 5	The draft CR on INM to be included in the running CR for 38.331 late drop.

# 4	References

R2-1817618, Report of [103bis#11][NR] Late drop 36.331 CR, Samsung, RAN2#104, November 2018

R2-1817731, Late drop 36.331 CR - issues remaining from eMail 11, Samsung, RAN2#104, November 2018

R2-1817797, CR on 36.331 for inter-node messages for NE-DC, Huawei, RAN2#104, November 2018

R2-1818392, Inter-node RRC messages for NE-DC, Ericsson, RAN2#104, November 2018

R1-1814320, LS on power control for NR-LTE Dual Connectivity, Ericsson, RAN1#95, November 2018

R1-1809886, LS on power control for NR(FR1) - NR(FR2) dual connectivity, Ericsson, RAN1#94, August 2018

R2-1818405, Capability coordination for NR-DC, Ericsson, RAN2#104, November 2018

R2-1817371, CR on 38.331 for UE capability coordination in NR-DC, Huawei, RAN2#104, November 2018

R2-1818303, CR on PDCCH blind decoding capability coordination (approach 1), Samsung, RAN2#104, November 2018

R2-1818306, CR on PDCCH blind decoding capability coordination (approach 2-1), Samsung, RAN2#104, November 2018

R2-1901320, Introduction of inter-node messages for NE-DC and NR-DC, CR0908, 38.331, Ericsson