3GPP TSG-RAN WG2 Meeting #105	R2-1901322

Athens, Greece, 25th February– 1st March 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3894 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG failure handling in case of NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SCGFailureInformation message is missing in the ASN.1 for the NE-DC option. Also, the procedural text needs to be updated since it covers only EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SCGFailureInformation message added in the ASN.1 for NE-DC and the procedural text has been updated accordingly. Impact analysis If the network implements the CR and the UE does not, the UE cannot trigger the SCG failure procedure and inform the network. If the UE implements the CR and the network does not, the network will not be able to decode some of the IEs within sent in the SCG failure procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented the UE can not perform the SCG failure procedure. Also, inter-node messages for NE-DC will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TR 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.6.13	SCG failure information

#### 5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.6.13.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a; or

1>	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.17.2 of TS 36.133 [29].

In case of DC, upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	if the UE is operating in NE-DC:

2>	initiate transmission of the SCGFailureInformationEUTRA message as specified in TS 38.331 [X, 5.7.xa];

1>	else

2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

END OF CHANGES

START OF CHANGES

#### 5.6.13.x	Failure type determination in NE-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationEUTRA message due to T313 expiry:

2>	set the failureType as t313-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide SCG change failure information:

2>	set the failureType as scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

#### 5.6.13.x	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-FailureMRDC as follows:

1>	for each MeasObjectEUTRA for which a measId is configured and measurement results are available;

2>	include an entry in measResultsPerMOList;

2>	if a serving cell is associated with the MeasObjectEUTRA:

3>	set measResultServingCell to include the available quantities of the concerned cell and in accordance with the performance requirements in [FFS_Ref];

2>	set the measResultNeighCells to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR,

3>	for each neighbour cell included:

4>	include the optional fields that are available.

END OF CHANGES

START OF CHANGES

### 6.3.5	Measurement information elements

#### –	MeasResultSCG-FailureMRDC

The IE MeasResultSCG-FailureMRDC is used to provide E-UTRA measurement information regarding SCG failures detected by the UE.

MeasResultSCG-FailureMRDC information element

-- ASN1START

MeasResultSCG-FailureMRDC ::=           SEQUENCE {

measResultPerMOList                     MeasResultList3EUTRA-r15,

...

}

MeasResultList3EUTRA ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult3EUTRA

MeasResult3EUTRA ::=  SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA-r9                        OPTIONAL,

measResultServingCell               MeasResultEUTRA                            OPTIONAL,

measResultNeighCellListNR           MeasResultListEUTRA                        OPTIONAL,

...

}

-- ASN1STOP

END OF CHANGES