3GPP TSG-RAN WG2 #105	TDoc R2-1901402

Athens, US, 25th February – 1st March 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	Overview of performance measurements in different RAN deployment scenarios

Document for:	Discussion, Decision

# Introduction

SA5 has been working on standardizing the L2 measurements mainly for the performance monitoring aspects [1]. In this contribution we analyse those parameters that are affected by the split architecture at the network side and this possibly requiring close attention from RAN2 and RAN3.

# Discussion

## Scenario description

There are three types of deployments considered by SA5 are as follows;

- No split scenario

gNB includes functionality associated to RRC, PDCP, RLC, MAC and PHY layers. There is no standardized interface between these layers as all these functionalities are handled by the same node. This deployment option is shown in figure 1a.

- Two-split scenario

gNB functionality is split among two nodes, one of which will carry out RRC and PDCP associated functionality and the other will carry out the RLC, MAC and PHY associated functionality. The interface between these two network nodes is F1-interface [3]. This deployment option is shown in figure 1b.

- Three-split scenario

gNB functionality is split among three nodes, first of which will carry our RRC functionality, second of which will carry out PDCP functionality and the third of which carries out the RLC, MAC and PHY functionality. The communication between the nodes performing RRC and PDCP functionality is specified in E1 interface [4]. The communication between the nodes performing RRC and RLC+MAC+PHY functionality is specified in F1-C interface [3]. The communication between the nodes performing PDCP and RLC+MAC+PHY functionality is specified in F1-U interface [3]. This deployment option is shown in figure 1c.

Figure 1 : Different deployment options. (1a): No-split scenario. (1b): 2-split scenario. (1c): 3-split scenario.

The L2 measurements used by the OAM for observability purposes need to be collected for all these three deployments.

- L2 measurements are to be collected for no-split, 2-split and 3-split deployment scenarios.

Based on the deployment type, the network nodes have different information available. Considering, 3-split scenario is the most diverse deployment of the three, we can discuss what information is available in which network nodes. gNB-CU-CP is aware of the cell identity to which a given UE is connected to, awareness of the QoS to DRB mapping associated to the UE and the awareness of the slice(s) flow to DRB mapping for the UE. gNB-CU-UP is aware of PDCP buffering status associated to a UE. gNB-DU is aware of the air interface delay, loss etc. associated to a UE’s connection and the cell identity for a given UE.

- PCI of the cell to which a UE is connected to is visible to the gNB-CU-CP and the gNB-DU only.

## Performance measurements to be collected in all deployment scenarios

SA5 has listed the following measurements as candidates to be standardized in NR that are to be collected by the RAN nodes, in all deployment scenarios of the gNB.

- Packet Delay

  - Average delay DL air-interface

- Radio resource utilization

  - DL Total PRB usage

  - UL Total PRB Usage

  - Distribution of DL Total PRB Usage

  - Distribution of UL Total PRB Usage

- UE throughput

  - Average DL UE throughput in gNB

  - Distribution of DL UE throughput in gNB

  - Average UL UE throughput in gNB

  - Distribution of UL UE throughput in gNB

  - Volume of unrestricted DL UE data in gNB

  - Volume of unrestricted UL UE data in gNB

- RRC connection number

  - Mean number of RRC Connections

  - Max number of RRC Connections

- PDU Session Management

  - Number of PDU Sessions requested to setup

  - Number of PDU Sessions successfully setup

  - Number of PDU Sessions failed to setup

- Mobility management

  - Inter-gNB handovers

    - Number of requested handover preparations

    - Number of successful handover preparations

    - Number of failed handover preparations

    - Number of requested handover resource allocations

    - Number of successful handover resource allocations

    - Number of failed handover resource allocations

- TB related Measurements

  - Total number of DL initial TBs

  - Initial error number of DL TBs

  - Total number of DL TBs

  - Total error number of DL TBs

  - Residual error number of DL TBs

  - Total number of UL initial TBs

  - Error number of UL initial TBs

  - Total number of UL TBs

  - Total error number of UL TBs

  - Residual error number of UL TBs

These measurements can be checked from different requirement point of view, for example, whether these measurements are required per QoS level or not, whether these measurements are UL related or DL related, in which gNB node do these measurements etc. These aspects are captured in the Table 1.

| Performance Measurement name | UL/DL | Network entity in which the measurement performed | Sub-counter |
| --- | --- | --- | --- |
| Average delay DL air-interface | DL | gNB-CU | Per-DRB- level-QoS |
| DL Total PRB Usage | DL | gNB-DU | - |
| UL Total PRB Usage | UL | gNB-DU | - |
| Distribution of DL Total PRB Usage | DL | gNB-DU | - |
| Distribution of UL Total PRB Usage | UL | gNB-DU | - |
| Average DL UE throughput in gNB | DL | gNB-DU | Per-DRB- level-QoS |
| Distribution of DL UE throughput in gNB | DL | gNB-DU | - |
| Average UL UE throughput in gNB | UL | gNB-DU | Per-DRB- level-QoS |
| Distribution of UL UE throughput in gNB | UL | gNB-DU | - |
| Volume of unrestricted DL UE data in gNB | DL | gNB-DU | Per-DRB- level-QoS |
| Volume of unrestricted UL UE data in gNB | UL | gNB-DU | Per-DRB- level-QoS |
| Mean number of RRC Connections | - | gNB-CU-CP / gNB-CU | - |
| Max number of RRC Connections | - | gNB-CU-CP / gNB-CU | - |
| Number of PDU Sessions requested to setup | - | gNB-CU-CP / gNB-CU | Per S-NSSAI |
| Number of PDU Sessions successfully setup | - | gNB-CU-CP / gNB-CU | Per S-NSSAI |
| Number of PDU Sessions failed to setup | - | gNB-CU-CP / gNB-CU | Per failure cause |
| Number of requested handover preparations | - | gNB-CU-CP / gNB-CU | - |
| Number of successful handover preparations | - | gNB-CU-CP / gNB-CU | - |
| Number of failed handover preparations | - | gNB-CU-CP / gNB-CU | Per failure cause |
| Number of requested handover resource allocations | - | gNB-CU-CP / gNB-CU | - |
| Number of successful handover resource allocations | - | gNB-CU-CP / gNB-CU | - |
| Number of failed handover resource allocations | - | gNB-CU-CP / gNB-CU | Per failure cause |
| Total number of DL initial TBs | DL | gNB-DU | Per modulation scheme |
| Initial error number of DL TBs | DL | gNB-DU | Per modulation scheme |
| Total number of DL TBs | DL | gNB-DU | Per layer at MU-MIMO case |
| Total error number of DL TBs | DL | gNB-DU | Per layer at MU-MIMO case |
| Residual error number of DL TBs | DL | gNB-DU | - |
| Total number of UL initial TBs | UL | gNB-DU | Per modulation scheme |
| Initial error number of UL TBs | UL | gNB-DU | Per modulation scheme |
| Total number of UL TBs | UL | gNB-DU | Per layer at MU-MIMO case |
| Total error number of UL TBs | UL | gNB-DU | Per layer at MU-MIMO case |
| Residual error number of UL TBs | UL | gNB-DU | - |

Table 1: Table illustrating the dependencies of different performance measurements that needs to be collected in all gNB deployment scenarios

Based on our understanding of the available information at the gNB-DU and gNB-CU-CP, the performance measurements captured in Table 1 can be performed with the existing information.

- Existing information at gNB-DU and gNB-CU-CP is sufficient to log the performance measurements listed in Table 1.

## Performance measurements impacted by 2-split/3-split deployment scenarios

SA5 has listed the following measurements as candidates to be standardized in NR that are affected by the 2-split/3-split deployment of the gNB.

- Packet loss rate

  - UL PDCP SDU loss rate

  - UL F1-U packet loss rate

  - DL F1-U packet loss rate

- Packet drop rate

  - DL PDCP SDU drop rate in gNB-CU-UP

  - DL PDCP SDU drop rate in gNB-DU

- Packet delay

  - Average delay DL in CU-UP

  - Average delay on F1-U

  - Average delay DL in gNB-DU

- IP latency measurements

  - IP latency DL in gNB-DU

- UE context release

  - UE context release request (gNB-DU initiated)

  - Number of UE Context Release Requests (gNB-CU initiated)

- PDCP data volume measurements

  - DL PDCP PDU Data Volume

  - UL PDCP PDU Data Volume

  - DL PDCP SDU Data Volume

  - UL PDCP SDU Data Volume

These measurements can be checked from different requirement point of view, for example, whether these measurements are required per QoS level or not, whether these measurements are UL related or DL related, in which ‘module’ do these measurements can be collected etc. These aspects are captured in the Table 2.

| Performance Measurement name | UL/DL | Network entity in which the measurement performed | Sub-Counter |
| --- | --- | --- | --- |
| UL PDCP SDU loss rate | UL | gNB-CU-UP/ gNB-CU | Per-DRB- level-QoS |
| UL F1-U packet loss rate | UL | gNB-CU-UP | Per-DRB- level-QoS |
| DL F1-U packet loss rate | DL | gNB-CU-UP | Per-DRB- level-QoS |
| DL PDCP SDU drop rate in gNB-CU-UP | DL | gNB-CU-UP/ gNB-CU | Per-DRB- level-QoS |
| DL PDCP SDU drop rate in gNB-DU | DL | gNB-DU | Per-DRB- level-QoS |
| Average delay DL in CU-UP | DL | gNB-CU-UP | Per-DRB- level-QoS |
| Average delay in F1-U | DL (?) | gNB-CU-UP | Per-DRB- level-QoS |
| Average delay DL in gNB-DU | DL | gNB-DU | Per-DRB- level-QoS |
| IP latency DL in gNB-DU | DL | gNB-DU | Per-DRB- level-QoS |
| UE context release request (gNB-DU initiated) | - | gNB-DU | - |
| UE context release request (gNB-CU initiated) | - | gNB-CU-CP / gNB-CU | - |
| DL PDCP PDU Data Volume | DL | gNB-CU-UP / gNB-CU | Per-DRB- level-QoS and per PLMN |
| UL PDCP PDU Data Volume | UL | gNB-CU-UP / gNB-CU | Per-DRB- level-QoS and per PLMN |
| DL PDCP SDU Data Volume | DL | gNB-CU-UP / gNB-CU | Per-DRB- level-QoS and per PLMN |
| UL PDCP SDU Data Volume | UL | gNB-CU-UP / gNB-CU | Per-DRB- level-QoS and per PLMN |

Table 2: Table illustrating the dependencies of different performance measurements that are affected by split gNB deployments.

In the current SA5 specification [1], the measurements are specified some of which needs to be collected at QoS level (mapped-5QI). RAN3 has sent an LS to SA5 [5] saying that the mapped-5QI used in the SA5 specification is same as the DRB level QoS parameter captured in F1 interface in the UE CONTEXT SETUP REQUEST message from the gNB-CU-CP to the gNB-DU [3], see Table 3. Based on this information, the gNB-DU is aware of the DRB level QoS requirements and it can also collect the DRB level L2 measurements accordingly.

- gNB-DU is provided with DRB level QoS mapping (mapped-5QI) in the UE CONTEXT SETPUP REQUEST message from the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ….. | …… | … | … | … | … | …. |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| …. | …. | ….. | …. | …. | …. | … |

Table 3 :  Contents of interest from the UE CONTEXT SETUP RESQUEST message from gNB-CU-CP to gNB-DU.

As per the current SA5 specification [1], the gNB-CU-UP also needs to perform many of the L2 measurements per mapped-5QI level. However, the gNB-CU-UP is not aware of the DRB level QoS or the mapped-5QI as informed by gNB-CU-CP to gNB-DU. In order to make gNB-CU-UP capable of logging the measurements at mapped-5QI level, gNB-CU-CP needs to inform gNB-CU-UP about the DRB level QoS that is also sent to the gNB-DU.

- gNB-CU-UP is not aware of the DRB-level-QoS mapping (mapped-5QI).

- RAN2 is requested to ask RAN3 to make gNB-CU-UP aware of the DRB-QoS mapping (mapped-5QI) by including the same in the appropriate signaling over E1 interface.

- RAN2 is requested approve LS [6] to be sent to RAN3.

# Conclusion

In section 2 we made the following observations:

Observation 1	L2 measurements are to be collected for no-split, 2-split and 3-split deployment scenarios.

Observation 2	PCI of the cell to which a UE is connected to is visible to the gNB-CU-CP and the gNB-DU only.

Observation 3	Existing information at gNB-DU and gNB-CU-CP is sufficient to log the performance measurements listed in Table 1.

Observation 4	gNB-DU is provided with DRB level QoS mapping (mapped-5QI) in the UE CONTEXT SETPUP REQUEST message from the gNB-CU-CP.

Observation 5	gNB-CU-UP is not aware of the DRB-level-QoS mapping (mapped-5QI).

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 is requested to ask RAN3 to make gNB-CU-UP aware of the DRB-QoS mapping (mapped-5QI) by including the same in the appropriate signaling over E1 interface.

Proposal 2	RAN2 is requested approve LS [6] to be sent to RAN3.

# References

28.552, 3GPP TSG SA, Management and Orchestration, 5G performance measurements, Release-16, v16.0.0, 2018-12.

36.314, 3GPP TSG RAN, EUTRA L2 measurements Release-15, v15.2.0, 2018-12.

38.473, 3GPP TSG RAN, F1 application protocol Release-15, v15.4.1, 2019-01.

38.463, 3GPP TSG RAN, E1 application protocol Release-15, v15.2.0, 2018-12.

S5-187038, Reply LS to SA5 on L2 measurements, RAN3, 3GPP TSG-SA5 meeting#122, Spokane, USA, 12-16 November 2018.

R2-1901403, Draft LS on DRB-QoS information inclusion in E1 interface, Ericsson, RAN2#105 meeting, Athens, Greece.