3GPP TSG-RAN2 Meeting #105 	R2-1901414

Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast MCG recovery in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-02-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the Rel-16 WI on CA & DC enhancements is to add support for fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. One way to achieve this objective for UEs in NE-DC or NR-DC is to introduce a procedure in 38.331 for MCG failure information to recover from MCG RLF for UEs configured with split SRB. In case of MCG RLF, such UE suspends MCG transmissions and sends MCG Failure Information using the SCG RLC bearer. In the case the SCG is already suspended e.g. due to SCG RLF, the UE falls back to the RRC connection re-establishment procedure and does not try to report MCG Failure Information to SCG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 7.7 is updated to introduce the necessary functionality to report MCG failure information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item objective is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 SCG/MCG failure handling |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331, TS 36.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

In MR-DC, fast MCG recovery due to MCG radio link failure is supported when split SRB is configured. Fast MCG recovery is supported for the following cases:

- MCG RLF;

- MCG configuration with sync failure;

Fast MCG recovery involves the follows steps; upon detecting one of the above MCG failure cases when split SRB is configured and when SCG transmissions are not suspended, the UE suspends MCG transmissions and reports the MCG Failure Information to the SN, instead of triggering re-establishment if MCG transmission is not already suspended. If MCG transmissions are suspended prior to the MCG failure detection, the UE does nothing.

In case fast MCG recovery is not supported, the UE initiates RRC connection re-establishment procedure if radio link failure is detected on MCG.

The following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	For EN-DC, NGEN-DC and NR-DC, SCG configuration failure (only for messages on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, SCG RRC integrity check failure (on SRB3).

Upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE can not maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

END OF CHANGES