3GPP TSG RAN WG2 Meeting #105                                                                      R2-1901517

Athens, Greece, 25 February – 1 March 2019

Agenda item:	11.4.2.2

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	On NR V2X Resource Allocation

Document for:	Discussion

# Introduction

NR V2X will be designed to target the new advanced driving uses case groups including vehicle platooning, extended sensors, advanced driving and remote driving [1]. During the RAN#80 meeting, the NR study item on NR V2X [2] was approved and the following objectives with respect to sidelink (SL) resource allocation were identified:

1: Sidelink design [RAN1, RAN2]: - Identify technical solutions for a NR sidelink design to meet the requirements of advanced V2X services, including - Study the support of sidelink unicast, sidelink groupcast and sidelink broadcast - Study NR sidelink physical layer structures and procedure(s) - Study sidelink synchronization mechanism - Study sidelink resource allocation mechanism (also including objective 3) - Study sidelink L2/L3 protocols NOTE: Only the performance of advanced V2X use cases will be evaluated in the design of NR sidelink. 3: Uu-based sidelink resource allocation/configuration (LTE V2X Mode3-like and Mode4-like) [RAN1, RAN2]

The email discussion report regarding NR V2X resource allocation [3], consists of a set of proposals summarizing companies’ views.

This contribution discusses a few issues in more detail related to the design of resource allocation mechanisms for NR V2X in relation to the proposals in [3].

# Resource Allocation for Advanced V2X Services

For NR V2X, four different advanced V2X use case groups (vehicle platooning, extended sensors, advanced driving and remote driving) are defined with stringent requirements for latency, reliability and data rates. Therefore, the capabilities of the NR V2X specification need to be enhanced to satisfy the wide variation of QoS requirements. The following section discusses the resource allocation aspects in NR V2X.

## Segregation of Resource Pools

As the structure and requirements for transmissions vary, it can be beneficial to segregate traffic into different resource pools. For example, the structure can vary depending on the presence of feedback resources. The following section motivates this.

### Resource Pool structure

In the case, where feedback is used in unicast or groupcast, resources need to be allocated for this feedback. Having a unified resource pool structure leaves these resources unused when no feedback is sent in broadcast or some unicast and groupcast cases. So segregating pools based on their structure avoids having unused resources due to parts of the structure not being used as highlighted above with the feedback case.

Observation 1: The presence or absence of feedback resources can motivate separate resource pools.

### Different QoS requirements

Ensuring high QoS especially in Mode 2 requires transmissions to be scheduled quickly and with little interference. Avoiding interference and blocking from low QoS traffic can ensure this. One way to achieve this is by separating traffic depending on its QoS into different resource pools.

Observation 2: The blocking of high QoS transmissions by lower QoS transmissions should be minimized.

Proposal 1: RAN2 should evaluate the benefits of supporting resource pool segregation.

## Mode 2 SL Grant Selection

In the case of platooning it can be beneficial when the leader UE is assisted by gNB in granting/scheduling resources to the group members. In this case the members can operate in mode 2 being assisted by the group leader.

In an out of coverage case the group leader can sense resources from a pre-configuration and assist group members by giving them this resource pool. Each member UE can then autonomously select resources from the pool.

Observation 3: In the platooning use case, the leader UE can be assisted by the network in scheduling/granting resources to the group members.

Proposal 2: RAN 2 should support the leader UE in assisting the selection of resources for member UEs in Mode 2.

# Conclusion

This contribution has noted the following observation:

The following proposals are provided, in relation to the resource allocation techniques for NR V2X.:

Observation 1: The presence or absence of feedback resources can motivate separate resource pools.

Observation 2: The blocking of high QoS transmissions by lower QoS transmissions should be minimized.

Observation 3: In the platooning use case, the leader UE can be assisted by the network in scheduling/granting resources to the group members.

As a result, the following proposals are provided:

Proposal 1: RAN2 should evaluate the benefits of supporting resource pool segregation.

Proposal 2: RAN 2 should support the leader UE in assisting the selection of resources for member UEs in Mode 2.

# References

- TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services”, V16.0.0, 3GPP SA#80, June 2018.

- RP-181064, “Study on NR V2X”, Vodafone, 3GPP RAN#80, June 2018.

- R2-191xxxx, “Report of [104#59][NR/V2X] Resource allocation”, LG, 3GPP RAN2#105, Feb-Mar 2019.