3GPP TSG-RAN WG2 #105	R2-1901524

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	9.8.3

Source:	Ericsson, SONY

Title:	Discussion on misalignment of sensor chapters in 36.305

Document for:	Discussion, Decision

# Introduction

Currently the sensor chapters in stage2 36.305/38.305 are unaligned with the stage3 36.355. The purpose of this email discussion is to basically get an opinion from companies on how to resolve this issue.

# Discussion

3gpp TS 36.305 has the detailed stage2 description on Barometric Pressure sensor which has been captured in Chapter 8.6

3gpp TS 36.355 has the generic ASN.1 framework to provide Signalling support for the sensor-based positioning method.

Basically, stage2 defines Barometric Pressure sensor as “the” sensor positioning method which is basically an error existing since Rel-13. Barometric Pressure sensor should have been introduced in a section under a generic framework where all different sort of Sensor based positioning method could have been defined.

The same issue/error has been also inherited in 38.305; ie; the misalignment also exists between 38.305 and 36.355.

Ideally the TS 36.305 should have captured the sensor positioning method as below

8.6 Sensor Positioning Method

8.6.1 Barometric Positioning Method

8.6.1.1 General

8.6.1.2

8.6.1.3

8.6.2 Motion Sensors

8.6.2.1 General

8.6.2.2

8.6.2.3

One option to correct the TS 36.305/38.305 would basically be to void the existing barometric pressure sensor sections and add new sections/subsections for sensors. The voided sections would have to point to the new sections for backwards compatibility.

Another option would be to describe any new Sensor based Positioning method as an independent section as similar to Barometric pressure sensor in stage2; basically, with its own section header but then to have a reference in stage3 36.355 as below.

### 6.5.5	Sensor based Positioning

This section defines support for sensor-based positioning. The relevant sensor methods in this version of the specification are the “Barometric pressure sensor method” and “Motion sensors positioning method” which are defined in 3GPP TS 36.305, chapter 8.6 and chapter 8.x respectively.

RAN2 to discuss whether the existing error/bug with respect to the misalignment of sensor positioning methods in stage2 and stage3 needs a correction or not

If a correction is needed, RAN2 to discuss and agree on a solution.

RAN2 to pick one of the options described in this paper.

- Option1: To void previous sections and create new sections.

- Option2: To continue with the error/bug, however to solve it using a reference in Stage3.

.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to discuss whether the existing error/bug with respect to the misalignment of sensor positioning methods in stage2 and stage3 needs a correction or not

Proposal 2	If a correction is needed, RAN2 to discuss and agree on a solution.

Proposal 3	RAN2 to pick one of the options described in this paper.

Option1: To void previous sections and create new sections.

Option2: To continue with the error/bug, however to solve it using a reference in Stage3.

# References

RP-171508, WID Update: UE Positioning Accuracy Enhancements for LTE.