3GPP TSG-RAN WG2 Meeting #105	R2-1901565

Athens, Greece, 25 Feb - 01 Mar 2019

Agenda item:	12.2.10

Source:	Nokia, Nokia Shanghai Bell

Title:	UP solution support for NB-IoT and MTC connected to 5GC

WID/SID:	NB_IOTenh3-Core, LTE_eMTC5-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

SA2 has sent an LS [1] to RAN2 asking the following:

SA2 question: SA2 kindly requests RAN2 to inform SA2 whether they intend to support UP optimization and RRC_INACTIVE for NB-IoT and eMTC connected to 5GC.

In this contribution we discuss further support for UP optimization and RRC_INACTIVE for NB-IoT and eMTC connected to 5GC and way forward is proposed.

# 2	Discussion

The objectives for ongoing REL-16 work items for NB-IoT [2] and MTC [3] have been updated to include support for RRC_INACTIVE. It was agreed to support RRC_INACTIVE for both NB-IoT and eMTC connected to 5GC.

Proposal 1: Reply to SA2 that RAN2 intends to support RRC_INACTIVE for both NB-IoT and eMTC connected to 5GC

RRC_INACTIVE is already part of Rel-15 5GS and on the other hand there is no support for suspend-resume in 5GC. RRC_INACTIVE is clearly in the scope of the RAN2 objectives [2,3] and we see no benefits to support multiple CIOT UP solution and RRC_INACTIVE, because they are targeting the same use cases. Therefore, it would be logical to support RRC_INACTIVE for both NB-IoT and MTC connected to 5GC and not to support suspend-resume.

Proposal 2: Reply to SA2 that RAN2 intends not to support UP optimization for NB-IoT and eMTC connected to 5GC

# 3	Conclusions

Based on the discussion the following is proposed:

Proposal 1: Reply to SA2 that RAN2 intends to support RRC_INACTIVE for both NB-IoT and eMTC connected to 5GC

Proposal 2: Reply to SA2 that RAN2 intends not to support UP optimization for NB-IoT and eMTC connected to 5GC

Draft LS according to above is provided in [4].

# 4	References

- S2-1813400, LS on Completion of Study on Cellular IoT support and evolution for the 5G System, SA2

- RP-182902, Additional enhancements for NB-IoT

- RP-182891, Additional MTC enhancements for LTE

- R2-1901568, Reply LS on Completion of Study on Cellular IoT support and evolution for the 5G System, Nokia