3GPP TSG-RAN WG2 #105	 TDoc R2-1901663

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.3

Source:	Ericsson

Title:	NR Uu Enhancements for Advanced V2X Use Cases

Document for:	Discussion, Decision

# Introduction

Uu enhancements for advanced V2X use cases is one of the objectives of the NR V2X SI for Rel.16 [1]. It first aims at evaluating whether Rel-15 NR Uu and LTE Uu interfaces can support advanced V2X use cases, and then study possible enhancements. On this topic, the following was agreed during RAN1#94bis, RAN1#95 and RAN1#ah1901.

Agreements: - For Uu for advanced V2X use cases, NR supports having multiple active UL configured grants in a given BWP in a given cell. - Details FFS Agreements: - NR supports that UE reports assistance information to the gNB, consisting of at least (with details FFS): - UE-related geographic information (e.g., position). - Reports of Uu V2X traffic-related information (at least for periodic traffic). Agreements: - DCI is used to identify the type-2 UL configured grant to be activated or released. - FFS Single DCI for multiple type-2 UL configured grants. Agreements: - From RAN1 perspective, NR UE reporting on Uu that include at least traffic periodicity, timing offset, and message size is beneficial at least for the purpose of managing multiple UL configured grants. Conclusion: - For this and future meetings, enhancements on Uu reporting for managing of Uu grants and configuration will be discussed in this AI (“Evaluation of Uu for advanced V2X use cases”). - For this and future meetings, enhancements on Uu reporting for managing of SL grants and configuration will be discussed in the AI “Uu-based sidelink resource allocation/configuration”. Agreements: - NR supports UE reporting over Uu of sidelink traffic-related information. - FFS contents. Conclusion: - For type 2 configured grant, discuss signalling details and whether single DCI can activate/release multiple configured grants during WI (if included in the WI). Agreements: - A UE is not expected to transmit simultaneously according to more than one UL grant (e.g., dynamic or CG) in a given BWP.

In this paper, we propose some more enhancements on NR Uu interface to support advanced V2X use cases.

# Discussion

Similar to LTE, NR Uu support DL SPS configuration. However, the supported periodicities are not well suited for V2X uses cases. For example, CAM-like periodicities are not currently supported. Moreover, very short periodicities (i.e., below 10 ms), which are required by advanced V2X use cases are not supported either. In our view, the specification should support them.

NR Uu supports DL SPS configurations with periodicities that are relevant for V2X services, including 1 and 100 ms.

Furthermore, V2X use cases results in periodic and aperiodic traffic which requires to be delivered with low latency. Therefore, as in multiple UL configured grants, it is beneficial to support multiple DL SPS configurations active simultaneously. This reduces the DCI overhead which would be required in case of dynamic scheduling.

NR Uu supports having multiple simultaneously active DL SPS configurations.

In Rel. 15 NR Uu, the UE is mandated to send HARQ feedback, i.e. ACK or NACK, after the decoding attempt of scheduled PDSCH. Transmission of NACK, however, is only useful for the gNB to distinguish between the failure cases i.e. DCI not received and PDSCH not decodable. Since the DCI in DL SPS is preconfigured, the transmission of NACK might be not required at least for certain V2X services, e.g. for those services with strict latency requirement for which more prompt dynamically scheduled retransmissions might be required. So, it is possible that the UE is configured to skip the transmission of NACK in case PDSCH is not decoded. However, ACK is transmitted regularly if PDSCH can be decoded properly.

NACK transmissions might be not required in case of DL SPS.

UE may be configured to skip NACK transmissions when it is configured with DL SPS.

However, skipping NACK transmissions requires DL SPS grant confirmation similar to UL configured grant. The UE may send such confirmation using different ways and it is to be further studied by RAN1/RAN2.

RAN1/RAN2 considers procedures to support DL SPS confirmation upon activation/release of DL SPS via DCI.

Similar to the DL, we believe that also for the UL, configured UL grant confirmation should be investigated. RAN1 has agreed that it is supported the possibility to have multiple active UL configured grants in a given BWP in a given cell. Since in current NR specification, the configured grant confirmation is a zero-bit message it might be ambiguous to figure out at the gNB side which configured UL grant activation/release command the UE has really received. Without any enhancement to this functionality, it might be challenging for the gNB to activate/release at the same time multiple configured UL grant, due to the mentioned ambiguity issue.

RAN2 studies procedures to support UL SPS confirmation for multiple active UL configured grants, upon activation/release of UL SPS via DCI.

## Broadcast/Multicast support

The existing NR specifications do not support broadcast/multicast downlink transmissions. In the previous meetings, there were discussions on whether BC/MC for NR Uu should be studied or not. In our view, this study is out of scope for two reasons:

- At the time of drafting the NR V2X SID, there was a discussion on whether broadcast/multicast should be one of the objectives or not. Given that the study was supported only by a minority of companies, that the scope of the SI was already quite larger, and that the duration was quite short, it was agreed to leave out BC/MC for NR Uu.

- BC/MC is a complex feature with many implications beyond RAN2 and even other RAN WGs. When it comes to use cases, V2X is only one of them, arguably not even the most prominent one. Consequently, we believe that BC/MC should be studied in a dedicated SI. Only after the basic structure of NR Uu BC/MC is in place, it is reasonable to study the application to V2X and potential enhancements targeting specific use cases.

Broadcast and multicast for NR Uu are not studied in the NR V2X study item.

## Other QoS enhancements

Another area that SA2 is currently investigating is QoS Support for eV2X over Uu interface [2], for instance, RAN can notify the unfulfillment/re-fulfilment of the QoS characteristics (e.g. GFBR, PDB and PER) to the 5GC, and then to the application function, the V2X application can then take proper action based on the information provided by the 3GPP system. In addition, R)AN may also notify the UE of the QoS target fulfilment/unfulfillment based on request from 5GC. There are some issues that are RAN(2) related, e.g. how and when to notify the UEs, what should/could be notified, etc. These aspects need to be studied by RAN2.

RAN2 considers support of QoS fulfilment/unfulfillment notification to the UE, in coordination with SA2

# Conclusion

In the previous sections we made the following observations:

Observation 1	NACK transmissions might be not required in case of DL SPS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	NR Uu supports DL SPS configurations with periodicities that are relevant for V2X services, including 1 and 100 ms.

Proposal 2	NR Uu supports having multiple simultaneously active DL SPS configurations.

Proposal 3	UE may be configured to skip NACK transmissions when it is configured with DL SPS.

Proposal 4	RAN1/RAN2 considers procedures to support DL SPS confirmation upon activation/release of DL SPS via DCI.

Proposal 5	RAN2 studies procedures to support UL SPS confirmation for multiple active UL configured grants, upon activation/release of UL SPS via DCI.

Proposal 6	Broadcast and multicast for NR Uu are not studied in the NR V2X study item.

Proposal 7	RAN2 considers support of QoS fulfilment/unfulfillment notification to the UE, in coordination with SA2

# References

RP-181480 “New SID: Study on NR V2X”, RAN#80

TR 23.786 V0.10.0, “Study on architecture enhancements for EPS and 5G System to support advanced V2X services, (Release 16)”