3GPP TSG-RAN WG2 #105	 TDoc R2-1901664

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.2.2

Source:	Ericsson

Title:	Layer-2 implications of Platoon header

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss the possible L2 implications of platoon header. In particular, we take into account the issues that arose in the email discussion [1], and the latest progress on the latest RAN1 meeting.

# Discussion

Regarding platooning and the possibility to define a platoon header in AS, RAN2 concluded the following in RAN2#104.

From RAN2#104: No AS-level mechanism to determine a group manager (i.e. head UE) is stuided. FFS for platooning, on the visibility of a group manager (head UE) to AS and AS-level functionalities.

According to SA1 TS 22.186 [2], “all the vehicles in the platoon receive periodic data from the leading vehicle, in order to carry on platoon operations”, hence the presence of the platoon leader at higher layers is possible for some platoon management operations which are outside the 3GPP scope. However, the impact on AS is not straightforward. In our understanding the question of whether the platoon header is visible in AS or not boils down to the issue of whether some specific AS procedures are foreseen for the platoon header.

For example, the platoon header may be in charge of admitting/rejecting certain UEs from a platoon as well as in charge of maintaining/releasing the UEs in the platoon, on the basis of SL radio conditions, QoS requirements, etc. However, as many companies expressed in [1], specifying such tasks for the platoon header may not be trivial and it may also cause some significant overhead over the SL interface, since the platoon header would need to perform the above tasks for each and every UE in the platoon.

From AS perspective, a possible task of the platoon header would be to manage the SL connection of UEs in the platoon. This might imply significant specification effort and potential large overhead in the SL interface.

RAN2 does not consider procedures for the platoon header to manage SL group connection of group members at AS level.

Another possible task of the platoon header is to assign SL resources to other UEs in the platoon. This operational mode is currently discussed in RAN1, and it corresponds to the so-called mode-2d scheme.

From AS perspective, another possible task of the platoon header is to provide SL resources to other UEs according to the so-called mode-2d scheme which RAN1 is currently discussing.

Therefore, in our understanding, the question of whether the platoon header has to make visible in the AS strictly depends on what actions are expected for the UEs in the AS, which very much depends on RAN1 progress on the mode-2d scheme.

Before introducing visibility of the platoon header in the AS, RAN2 waits for RAN1 progress on mode-2d scheduling.

In our companion paper [4], we discuss more in details the mode-2d impacts in terms of RAN2.

In that paper, we propose to down-prioritize the mode-2d resource allocation given the non-trivial implications in L2 protocols. However, if mode-2d is eventually adopted for Rel.16, we have a preference to minimize specification impact, and select the simplest mode-2d alternative. See [4] for more details.

If mode-2d is introduced, it consists of a platoon header being configured with a set of mode-2d resources that it forwards to other UEs in the same platoon.

In any case, irrespective of the mode-2d scheme that eventually will be selected, it is expected that the resources provided by the platoon header will be received by all platoon members which can be in different coverage situation. As highlighted in our paper [3], a uniform resource allocation within the platoon is beneficial to guaranteed stable platoon performances. However, that may also bring to potential interference issues towards neighbouring areas/cells.

Some of such UEs can be in the same coverage status of the platoon header, e.g. same cell, while some others may be in different coverage status, e.g. different cells, or out-of-coverage. Therefore, if mode-2d is adopted in Rel.16, RAN2 should discuss how to handle such case, to avoid interference between platoon SL operations, and SL operations in neighbouring areas.

If mode-2d is introduced, RAN2 should discuss how to handle the case in which the platoon header and the platoon members are in different Uu coverage status, e.g. different cells/areas.

Additionally, if mode-2d is introduced, RAN2 should also discuss whether the mode-2d resource allocation can be provisioned only by the platoon leader or also by other UEs in the platoon. In fact, due to the limited SL communication range and different Uu coverage status between platoon members, as highlighted above, the resource allocation cannot be uniform throughout the whole platoon.

If mode-2d is introduced, RAN2 to discuss whether the mode-2d resource allocation can only be performed by the platoon header, or also by other UEs in the platoon.

## Visibility of platoon header to gNB

An additional issue which was touched upon in the email discussion [1] is how the gNB gets to know that this UE is a platoon header. In our understanding, this information should be provided by higher layers, since in the first place the platoon header is appointed following certain higher layers procedures, and certainly not by RAN rules.

Even if AS visibility of the platoon header is introduced, a UE is appointed platoon header by higher layer procedures.

Therefore, this information should be conveyed to the gNB by higher layers when the platoon header attaches to the network, and that should be subject to certain authorization/subscription requirements.

If AS visibility of the platoon header is introduced, the information related to whether a UE is a platoon header or not should be provisioned to the gNB by higher layers and it should be subject to authorization/subscription requirements.

# Conclusions

In section 2 we made the following observations:

Observation 1	From AS perspective, a possible task of the platoon header would be to manage the SL connection of UEs in the platoon. This might imply significant specification effort and potential large overhead in the SL interface.

Observation 2	From AS perspective, another possible task of the platoon header is to provide SL resources to other UEs according to the so-called mode-2d scheme which RAN1 is currently discussing.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 does not consider procedures for the platoon header to manage SL group connection of group members at AS level.

Proposal 2	Before introducing visibility of the platoon header in the AS, RAN2 waits for RAN1 progress on mode-2d scheduling.

Proposal 3	If mode-2d is introduced, it consists of a platoon header being configured with a set of mode-2d resources that it forwards to other UEs in the same platoon.

Proposal 4	If mode-2d is introduced, RAN2 should discuss how to handle the case in which the platoon header and the platoon members are in different Uu coverage status, e.g. different cells/areas.

Proposal 5	If mode-2d is introduced, RAN2 to discuss whether the mode-2d resource allocation can only be performed by the platoon header, or also by other UEs in the platoon.

Proposal 6	Even if AS visibility of the platoon header is introduced, a UE is appointed platoon header by higher layer procedures.

Proposal 7	If AS visibility of the platoon header is introduced, the information related to whether a UE is a platoon header or not should be provisioned to the gNB by higher layers and it should be subject to authorization/subscription requirements.

# References

R2-19xxxxx, Summary Report of [104#60][NR/V2X] Groupcast (Qualcomm), Athens, Greece, 25 Feb - 1 Mar. 2019

3GPP, TS 22.186, Service requirements for enhanced V2X scenarios

R2-1901656, Resource allocation for Sidelink Mobility, RAN2#105, Athens, Greece, Feb. 2019

R2-1901665, Layer-2 Implications of Mode-2d Resource Allocation, RAN2#105, Athens, Greece, Feb. 2019