3GPP TSG-RAN WG2 #105	 TDoc R2-1901675

Athens, Greece, 25th February – 1st March, 2019 	Revision of R2-1817972

Agenda Item:	11.2.1.2

Source:	Ericsson

Title:	On Autonomous UL Transmissions for NR-U

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss the support of autonomous UL transmissions for NR-U. As agreed in the NR-U WID [1], “NR Type-1 and Type-2 configured grant mechanisms are the baseline for NR-U operation with modifications in line with agreements during the study phase”.

In the NR-U TR capturing the agreements from the SI the following is captured:

The following modifications to the configured grant procedures are beneficial.

-	Removing dependencies of HARQ process information to the timing. This can be achieved by introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID

-	Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

-	Increased flexibility on time domain resource allocation for the configured grant transmissions

-	Supporting retransmissions without explicit UL grant

Allowing consecutive configured grant resources in time without any gaps in between the resources and non-consecutive configured grant resources (not necessarily periodic) with gaps in between the resources is beneficial and should be considered for NR in unlicensed spectrum

UE selects the HARQ process ID from an RRC configured set of HARQ IDs for NR-unlicensed configured grant transmission.

It is identified to be beneficial to support DFI to include pending HARQ ACK feedback for prior configured grant transmissions from the same UE.

For the retransmission of a HARQ process that was initially transmitted via configured grant resource, both retransmission via same configured grant resource and retransmission via resource scheduled by UL grant are supported.

UE may autonomously initiate retransmission for a HARQ process that was initially transmitted via configured grant mechanism for NR-unlicensed when it receives NACK feedback via DFI for the corresponding HARQ process.

In RAN1-1901, the additional agreements were made:

Agreement: CG-UCI should at least include the following information: - HARQ ID - NDI - RV - COT sharing information, FFS details - FFS: other information including UE ID

# Discussion

For unlicensed spectrum that uses LBT as the sharing mechanism, the physical control signal transmissions not only generate overhead, but may require more LBT and/or new LBT mechanisms when the UE and gNB roles are switched between transmission and reception. For unlicensed spectrum, low latency between transmissions can be of high importance to reduce the need for time-consuming LBT procedures.

NR supports two types of pre-configured resources. Both of which are different flavours of existing LTE semi-persistent scheduling with some further aspects such as supporting repetitions for a transport block:

- Type 1, UL data transmission with configured grant is only based on RRC (re)configuration without any L1 signalling.

- Type 2 is very similar to LTE SPS feature. UL data transmission with configured grant is based on both RRC configuration and L1 signalling for activation/deactivation of the grant. The gNB needs to explicitly activate the configured resources on PDCCH and the UE confirms the reception of the activation/deactivation grant with a MAC control element.

For LTE, the existing SPS feature was the baseline for designing autonomous uplink operation on LAA sCells in Rel.15. Similarly, for NR-U, the existing schemes (type 1 and 2) have been agreed to serve as a baseline to achieve AUL operation for NR-U. Nevertheless, some adjustments are needed to provide efficient operation on unlicensed bands.

It is beneficial to follow the same procedure as in feLAA to indicate the slots allowed for configured UL, i.e. an RRC configured Bitmap of X bits is used to indicate the allowed time resource on slot level.

This should be applicable to both type 1 and type 2 UL grant.

- RRC configured Bitmap of X bits is used to indicate the allowed time resource on slot level for both type 1 and type 2 UL grant.

Another enhancement for NR-U compared to NR is that in NR-U does not use the synchronous HARQ ID selection, where HARQ ID corresponding to a transmission on a preconfigured resource is derived from a formula similar to LTE SPS, i.e., the HARQ process ID is determined based on the resources that is used for transmission. This synchronous HARQ ID behaviour may impose large delays due to the uncertainty of channel availability on unlicensed bands and therefore inefficient for unlicensed channel operation.

The same issue was discussed during the feLAA WI, and to resolve it, a new uplink control information (UCI) was introduced to enable fully asynchronous HARQ for LAA AUL. For every AUL transmission, the UE selects HARQ process, RV, and NDI and report it on the new AUL UCI. This is now available also in NR-U according to the RAN1-1901 agreement.

As RAN1 agree, for every AUL transmission, the UE selects HARQ process, RV, and NDI and report it on the new AUL UCI.

NR, similar to eLAA Rel-14, does not support non-adaptive HARQ retransmissions. This means, any uplink retransmission shall be scheduled through UL grant on PDCCH. To reduce the signalling overhead on unlicensed operation for FeLAA Rel-15, AUL downlink feedback information ("AUL-DFI") was specified to carry pending HARQ feedback for several uplink HARQ processes for the same UE through a bitmap. Signalling similar to AUL-DFI would also be beneficial for AUL NR-U to enable non-adaptive HARQ retransmissions. However, details are left to RAN1.

For the NR autonomous UL access, downlink feedback information shall be introduced that enables carrying pending HARQ feedback for several UL HARQ processes for the same UE. Details of the format design are up to RAN1.

The autonomous UL grant may be used for retransmissions without an explicit UL retransmission grant.

Similar to feLAA Rel.15 also the retransmission timer, which indicates for how long the UE shall at least wait before performing a retransmission, can be used. This functionality is also supported in NR, however in NR ACK feedback is assumed if no explicit NACK is received before the timer expires. This approach does not work well on the unlicensed carrier since the absence of feedback might be due to failed LBT. The UE may misinterpret a delayed retransmission grant as an ACK. Since the channel availability is not guaranteed on the unlicensed channel, the UE might run into this situation often.

On the other hand, we suggest following similar principle as Rel-15 feLAAi, i.e. at retransmission timer expiry, with no implicit ACK or dynamic grant related to the specific HARQ process, the UE assumes NACK and perform non-adaptive retransmission on a later configured occasion.

For configured UL on NR-U, a timer starts when the MAC PDU is transmitted, and if no ACK or dynamic grant for the concerned HARQ process is received before the timer expires, the UE assumes NACK and performs non-adaptive retransmission.

As an enhancement to the LTE AUL scheme, RAN2 could study whether it is beneficial to configure the UE with multiple different UL grant configurations. In legacy feLAA, the eNB can configure only one AUL configuration in one SCell. Having multiple configured UL grants in a given SCell can be beneficial to allow some more network flexibility. The gNB can configure multiple UL configured UL grants for a given SCell and then activate/release a configuration more promptly on the basis of current cell load, and DL traffic flows.

Multiple configured UL grants can be configured per serving cell for type-2 operations.

# Conclusions

In section 2 we made the following observations:

Observation 1	As RAN1 agree, for every AUL transmission, the UE selects HARQ process, RV, and NDI and report it on the new AUL UCI.

Based on the discussion in section 2 we propose the following:

Proposal 1	RRC configured Bitmap of X bits is used to indicate the allowed time resource on slot level for both type 1 and type 2 UL grant.

Proposal 2	For the NR autonomous UL access, downlink feedback information shall be introduced that enables carrying pending HARQ feedback for several UL HARQ processes for the same UE. Details of the format design are up to RAN1.

Proposal 3	The autonomous UL grant may be used for retransmissions without an explicit UL retransmission grant.

Proposal 4	For configured UL on NR-U, a timer starts when the MAC PDU is transmitted, and if no ACK or dynamic grant for the concerned HARQ process is received before the timer expires, the UE assumes NACK and performs non-adaptive retransmission.

Proposal 5	Multiple configured UL grants can be configured per serving cell for type-2 operations.

# References

RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc.

3GPP TR 38.889 v16.0.0 “Study on NR-based access to unlicensed spectrum (Release 16)