3GPP TSG-RAN2 Meeting RAN2#105	R2-1901800

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0271 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PDCP structure for split bearer and LWA bearer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_HRLLC-Core |  |  |  |  |  |  | Date: |  | 2018-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | When the DC PDCP duplication is deactivated, the PDCP entity performs the fall-back operation. In this case, the PDCP entity for RLC UM can be associated up to four RLC entities, i.e., two RLC entities for uplink and downlink respectivly. However, in the specficiation, the PDCP entity is associated with only two UM RLC entities. In addition, for LWA bearers, the PDCP entity can be configured with RLC UM. However, in the specficiation, it addresses that the PDCP entity is associated only for an RLC AM entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the text ‘two or four UM RLC entities’ for split bearer. Add the text ‘an RLC AM entity or two RLC UM entities’ for LWA bearer. Impact analysis: Impacted functionality: PDCP structure Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For the PDCP duplication, the PDCP entity can not associated with four RLC UM entities if the DC PDCP duplication is de-activated for RLC UM. For LWA bearer, the PDCP entity cannot be confiugred with RLC UM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

4.2.1	PDCP structure

Figure 4.2.1.1 represents one possible structure for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 36.300 [2].

Figure 4.2.1.1 - PDCP layer, structure view

Each RB (i.e. DRB, SLRB and SRB, except for SRB0 and SRB1bis) is associated with one PDCP entity. Each PDCP entity is associated with one or two (one for each direction) RLC entities depending on the RB characteristic (i.e. uni-directional or bi-directional) and RLC mode. For split bearers, each PDCP entity is associated with two AM or two or four UM RLC entities. Each RB configured with PDCP duplication has a corresponding PDCP entity that is associated with two (bi-directional) RLC AM or two (bi-directional) RLC UM or four (uni-directional) RLC UM entities. For LWA bearers, each PDCP entity is associated with an RLC AM entity or two RLC UM entities and the LWAAP entity. The PDCP entities are located in the PDCP sublayer.

End of changes