3GPP TSG-RAN WG2 #105	Tdoc R2-1901876

Athens, Greece, 25th February– 1st March 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	Analysis on Performance measurements for gNB in TS28.552

Document for:	Discussion, Decision

# Introduction

Performance measurements for gNB have been standardization by SA5 in TS 28.552 R15 for O&M. LSs from SA5 asked RAN2 and RAN3 to analyse and define the performance measurements for gNB in RAN2 specifications, some of the exchanged LSs are in S5-182574, S5-183622, S5-184337, R3-184480 . Due to tight time schedule, RAN2 did not standardize the measurements in R15. RAN3 replied in R3-186232 that “The L2 measurement that may involve both RAN2 and RAN3 may be discussed and determined in the scope of the “RAN-centric data collection and utilization” SI”.

# Discussion

Observation 1: In TS 28.552, the following parameters were defined for performance measurement for gNB.

The proposed set of data to be collected from the gNB include but are not limited to:

- Performance measurement for all gNB deployment scenarios

  - Packet Delay

  - Radio resource utilization

  - UE throughput

  - RRC connection number

  - PDU Session Management

  - Mobility Management

  - TB related measurement

  - PDU session modification

  - PDU session release

  - DRB setup Management

- Performance measurement for non-split gNB deployment scenarios

- Performance measurement for spilt gNB deployment scenarios

  - Packet Loss Rate

  - Packet Drop Rate

  - Packet Delay

  - IP Latency measurement

  - UE context Release

  - PDCP data volume measurements

In the Performance measurement for all gNB deployment scenarios section, there are measurements on PDU session. However, PDU session is NAS concept, the measurements on PDU session are out of L2 measurement scope.

Observation 2: PDU session is NAS concept, measurements on PDU session are out of L2 measurement scope.

Except measurements on PDU session, other parameters are RAN2/RAN3 measurements, it is under RAN2/RAN3 responsibility to define. RAN2/RAN3 need to check/study the feasibility of each parameter.

Observation 3: RAN2 need to check/study the feasibility of rest parameters except PDU session parameters from RAN2 perspective.

RAN3 needs further check whether these measurements can be performed or not when CU/DU split and/or CU-CP/CU-UP split are deployed. RAN3 is encouraged to define necessary parameters on E1/FI interfaces to make these measurements feasible. However, they are under RAN3 responsibility.

Obseration 4: After RAN2 check, RAN3 need to furter check/study measurement feasibility if the measurement parameters have to do with gNB deployment scenario, i.e. CU/DU split and CU-CP/CU-DP split.

Observation 5: A LS to SA5 is needed the indicate the feasibilty of measurement of these paramters after the RAN2/RAN3 feasibility check/study.

The following gives the analysis result in highlighted text on parameters one by one which are defined in TS28.552 from RAN2 perspective.

# Performance measurement for all gNB deployment scenarios

### Packet Delay (Average delay DL air-interface)

The objective of this measurement is to measure L2 Packet Delay over air interface for OAM performance observability.

This measurement provides the average (arithmetic mean) time it takes to get a response back on a HARQ transmission in the downlink direction. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

RAN2:  It is feasible.

Protocol Layer: MAC

| Definition | Average delay DL air-interface. This measurement is obtained as: sum of (time when the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information, minus time when the last part of the same packet was transmitted over the air) divided by total number of RLC SDUs arriving at MAC lower SAP. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3). Detailed Definition: ,where explanations can be found in the table X.X.X.1-1 below. |
| --- | --- |

Table X.X.X.1-1

|  | Average delay DL air-interface. L2 Packet Delay in the DL over air interface, optionally split into subcounters per Qos level (mapped 5QI or QCI in NR option 3), averaged during time period . Unit: Integer ms. |
| --- | --- |
|  | The point in time when RLC SDU arrives. |
|  | The point in time when the last piece of RLC SDU i was received by the UE according to received HARQ feedback information. |
|  | A RLC SDU that arrives at the RLC lower SAP during time period . RLC SDU for which HARQ acknowledgement is not received for all parts shall not be included in the calculation. |
|  | Total number of RLC SDUs . |
|  | Time Period during which the measurement is performed |

### Radio resource utilization (PRB usage)

The objective of the PRB usage measurements is to measure usage of time and frequency resources. A use case is cell load balancing, where PRB usage is used for information signalled across the Xn interface. Another use-case is OAM performance observability.

If there is one or more RNs served in a cell, for that cell the gNB performs PRB usage measurements separately for all traffic (including transmissions to/from RNs and UEs directly connected to the gNB) and for RN traffic.

RAN2: Feasible.

#### total PRB usage

Protocol Layer: MAC

| Definition | Total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The measurement is done separately for: - DL - UL Detailed Definitions: , where explanations can be found in the table X.X.X.1-1 below. |
| --- | --- |

Table X.X.X.1-1

|  | Total PRB usage. Percentage of PRBs used, averaged during time period . Value range: 0-100% |
| --- | --- |
|  | A count of full physical resource blocks. For the DL, all PRBs used for transmission shall be included. For the UL, all PRBs allocated for transmission shall be included. |
|  | Total number of PRBs available during time period . For an eNB serving one or more RNs, all PRBs regardless of RN subframe configurations shall be counted. (NOTE) |
|  | The time period during which the measurement is performed. |

NOTE:	It is up to the gNB implementation how to calculate P(T) with respect to PRBs that may be considered not available.

### Radio resource utilization (Distribution of PRB usage)

The objective of the distribution of PRB usage measurements is to measure the distribution of the usage of time and frequency resources. A use case is cell load balancing, where PRB usage is used for information signalled across the Xn interface. Another use-case is OAM performance observability.

If there is one or more RNs served in a cell, for that cell the gNB performs PRB usage measurements separately for all traffic (including transmissions to/from RNs and UEs directly connected to the gNB) and for RN traffic.

RAN2: Feasible

#### Distribution of total PRB usage

Protocol Layer: MAC

| Definition | Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The measurement is done separately for: - DL - UL The distribution of PRB usage provides the histogram result of the samples collected during time period T. Considering there are n samples during time T and each sample has the same time period tn, detailed definitions of each sample value at instant n are: Mn=M1nPn*100, where explanations can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| Mn | Total PRB usage at sample n. Percentage of PRBs used, averaged during time period tn. Value range: 0-100% |
| --- | --- |
| M1n | A count of full physical resource blocks. For the DL, all PRBs used for transmission shall be included. For the UL, all PRBs allocated for transmission shall be included. |
| Pn | Total number of PRBs available during time period tn. For an eNB serving one or more RNs, all PRBs regardless of RN subframe configurations shall be counted. (NOTE) |
| n | The sample with time period tn during which the measurement is performed. |

NOTE:	It is up to the gNB implementation how to calculate P(n) with respect to PRBs that may be considered not available.

### UE throughput

The objective of this measurement is to measure over Uu the RLC SDU throughput independent of traffic patterns and packet size. This measurement is mainly intended for data bursts that are large enough to require transmissions to be split across multiple TTIs. Initial buffering time in UE or gNB is excluded.

For an gNB serving one or more RNs, packets transmitted between the gNB and RNs are excluded, i.e., only packets transmitted between the gNB and UEs are counted.

RAN2: Feasible.

#### Average DL UE throughput in gNB

Protocol Layer: RLC, MAC

| Definition | Average UE throughput in gNB. Throughput of RLC SDU bits in downlink for packet sizes or data bursts that are large enough to require transmissions to be split across several TTIs, by excluding transmission of the last piece of data in a data burst. Only data transmission time is considered, i.e. when data transmission over Uu has begun but not yet finished. Each measurement is a real value representing the throughput in kbits/s. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). For successful reception, the reference point is MAC upper SAP. This measurement is obtained by the following formula for a measurement period: , where For small data bursts, where all buffered data is included in one initial HARQ transmission, , otherwise Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbits successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time ( may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

#### Distribution of DL UE throughput in gNB

Protocol Layer: RLC, MAC

| Definition | Average UE throughput in gNB. Throughput of RLC SDU bits in downlink for packet sizes or data bursts that are large enough to require transmissions to be split across several TTIs, by excluding transmission of the last piece of data in a data burst. Only data transmission time is considered, i.e. when data transmission over Uu has begun but not yet finished. Each measurement is a real value representing the throughput in kbits/s. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). For successful reception, the reference point is MAC upper SAP. This measurement is obtained by the following formula for a measurement period: , where For small data bursts, where all buffered data is included in one initial HARQ transmission, , otherwise Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbits successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time ( may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

#### Average UL UE throughput in gNB

Protocol Layer: RLC, MAC

| Definition | Average UE throughput in gNB. Throughput of RLC SDU bits in uplink for packet sizes or data bursts that are large enough to require transmissions to be split across several TTIs, by excluding transmission of the last piece of data in a data burst. Only data transmission time is considered, i.e. when data transmission over Uu has begun but not yet finished. Each measurement is a real value representing the throughput in kbits/s. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). For successful reception, the reference point is MAC upper SAP. This measurement is obtained by the following formula for a measurement period: , where For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise: Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

#### Distribution of UL UE throughput in gNB

Protocol Layer: RLC, MAC

| Definition | Distribution of UL UE throughput in gNB. Throughput of RLC SDU bits in uplink for packet sizes or data bursts that are large enough to require transmissions to be split across several TTIs, by excluding transmission of the last piece of data in a data burst. Only data transmission time is considered, i.e. when data transmission over Uu has begun but not yet finished. Each measurement is a real value representing the throughput in kbits/s. For successful reception, the reference point is MAC upper SAP. This measurement is obtained by the following formula for a measurement period: , where For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise: Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

#### Volume of unrestricted DL UE data in gNB

Protocol Layer: RLC, MAC

| Definition | This measurement provides the percentage of DL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained by the following formula for a measurement period: For periods when no data is transferred at all Percentage Unrestricted Volume DL = 0, otherwise: Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X.X-1

| ThpUnresVolDl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolDl is the data volume counted on RLC SDU level in kbits sent in DL for one DRB. |
| --- | --- |
| ThpVolDl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume counted on RLC SDU level in kbits sent in DL for one DRB. |

Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of mapped 5QIs.

#### Volume of unrestricted UL UE data in gNB

Protocol Layer: RLC, MAC

| Definition | This measurement provides the percentage of UL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained by the following formula for a measurement period: For periods when no data is transferred at all Percentage Unrestricted Volume UL = 0, otherwise: Explanations of the parameters can be found in the table X.X.X.X-1 below. |
| --- | --- |

Table X.X.X6.X-1

| ThpUnresVolUl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB. |
| --- | --- |
| ThpVolUl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB. |

Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of mapped 5QIs.

### RRC connection number

RAN2: Feasible.

#### Mean number of RRC Connections

Protocol Layer: RRC

| Definition | This measurement provides the mean number of users in RRC connected mode during each granularity period. |
| --- | --- |

#### Max number of RRC Connections

Protocol Layer: RRC

| Definition | This measurement provides the maximum number of users in RRC connected mode during each granularity period. |
| --- | --- |

### PDU Session Management

RAN2: PDU session is NAS concept and it is out of L2 measurement scope.

### Mobility Management

RAN2: Feasible.

#### Inter-gNB handovers

##### Number of requested handover preparations

Protocol Layer: RRC

| Definition | This measurement provides the number of handover preparations requested by the source gNB. |
| --- | --- |

##### Number of successful handover preparations

Protocol Layer: RRC

| Definition | This measurement provides the number of successful handover preparations received by the source NR cell CU. |
| --- | --- |

##### Number of failed handover preparations

Protocol Layer: RRC

| Definition | This measurement provides the number of failed handover preparations received by the source NR cell CU. This measurement is split into subcounters per failure cause. |
| --- | --- |

##### Number of requested handover resource allocations

Protocol Layer: RRC

| Definition | This measurement provides the number of handover resource allocation requests received by the target NR cell CU. |
| --- | --- |

##### Number of successful handover resource allocations

Protocol Layer: RRC

| Definition | This measurement provides the number of successful handover resource allocations at the target NR cell CUfor the handover. |
| --- | --- |

##### Number of failed handover resource allocations

Protocol Layer: RRC

| Definition | This measurement provides the number of failed handover resource allocations at the target NR cell CUfor the handover. This measurement is split into subcounters per failure cause. |
| --- | --- |

### TB related measurement

RAN2: Feasible.

#### Total number of DL initial TBs

Protocol Layer: MAC

| Definition | This measurement provides the total number of initial TBs transmitted on the downlink in a cell. HARQ re-transmissions are excluded from this measurement.This measurement is optionally split into subcounters per modulation schema. |
| --- | --- |

#### Intial error number of DL TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of initial faulty TBs transmitted on the downlink in a cell.This measurement is optionally split into subcounters per modulation schema. |
| --- | --- |

#### Total number of DL TBs

Protocol Layer: MAC

| Definition | This measurement provides the total number of TBs transmitted on the downlink in a cell.The measurement is split into subcounters per layer at MU-MIMO case. This measurement includes all transmitted TBs (including the successful and failed TBs during initial transmission and HARQ re-transmission). |
| --- | --- |

#### Total error number of DL TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of total faulty TBs transmitted on the downlink in a cell .The measurement is split into subcounters per layer at MU-MIMO case.This measurement include all transmitted faulty TBs of initial transmission and re-transmission . |
| --- | --- |

#### Residual error number of DL TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of final faulty TBs transmitted on the downlink in a cell at last HARQ re-transmissions. |
| --- | --- |

#### Total number of UL initial TBs

Protocol Layer: MAC

| Definition | This measurement provides the total number of initial TBs on the uplink in a cell.This measurement is optionally split into subcounters per modulation schema. |
| --- | --- |

#### Error number of UL initial TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of initial faulty TBs on the uplink in a cell. This measurement is optionally split into subcounters per modulation schema. |
| --- | --- |

#### Total number of UL TBs

Protocol Layer: MAC

| Definition | This measurement provides the total number of TBs on the uplink in a cell.The measurement is split into subcounters per layer at MU-MIMO case.This measurement includes all transmitted TBs (including the successful and failed TBs during initial transmission and HARQ re-transmission). |
| --- | --- |

#### Total error number of UL TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of total faulty TBs on the uplink in a cell. The measurement is split into subcounters per layer at MU-MIMO case.This measurement include all transmitted faulty TBs of initial and re-transmission . |
| --- | --- |

#### Residual error number of UL TBs

Protocol Layer: MAC

| Definition | This measurement provides the number of final faulty TBs on the uplink in a cell. |
| --- | --- |

### PDU session modification

RAN2: PDU session is NAS concept and it is out of L2 measurement scope.

### PDU session release

RAN2: PDU session is NAS concept and it is out of L2 measurement scope.

### DRB setup Management

RAN2: Feasible.

#### Number of DRBs attempted to setup

Protocol Layer: RRC, PDCP

| Definition | This measurement provides the number of DRBs attempted to setup to support all requested QoS flows during time period T. This measurement is split into subcounters per mapped 5QI and per S-NSSAI. |
| --- | --- |

RAN2: Feasible

#### Number of DRBs successfully setup

Protocol Layer: RRC, PDCP

| Definition | This measurement provides the number of DRBs successfully setup to support all requested QoS flows during time period T. This measurement is split into subcounters per mapped 5QI and per S-NSSAI. |
| --- | --- |

# Performance measurement for non-spilt gNB deployment scenarios

### Void (No requirements from SA5)

# Performance measurement for spilt gNB deployment scenarios

### Packet Loss Rate

#### UL PDCP SDU Loss Rate

RAN2: Feasible.

Protocol Layer: PDCP

| Definition | This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained as: 1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3). |
| --- | --- |

#### UL F1-U Packet Loss Rate

RAN2: Out of L2 measurement scope and under RAN3 responsibility.

#### DL F1-U Packet Loss Rate

RAN2: Out of L2 measurement scope and under RAN3 responsibility.

### Packet Drop Rate

#### DL PDCP SDU Drop rate in gNB-CU-UP

RAN2: Feasible.

Protocol Layer: PDCP

| Definition | This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to congestion, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). |
| --- | --- |

#### DL Packet Drop Rate in gNB-DU

RAN2: Feasible.

Protocol Layer: RLC, MAC

| Definition | This measurement provides the fraction of RLC SDU packets which are dropped on the downlink, due to congestion, traffic management etc in the gNB-DU. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-DU without any part of it having been transmitted on the air interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained as: 1000000*Number of DL packets, for which no part has been transmitted over the air, of a data radio bearer, that are discarded in the gNB-DU divided by Number of DL packets for data radio bearers that were received from gNB-CU-UP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3). |
| --- | --- |

### Packet Delay

#### Average delay DL in CU-UP

RAN2: Feasible.

Protocol Layer: PDCP

| Definition | This measurement provides the average (arithmetic mean) PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained as: sum of (time when sending a PDCP SDU to the gNB-DU at the egress PDCP layer on F1-U/Xn-U, minus time of arrival of the same packet at NG-U ingress IP termination) divided by total number of PDCP SDUs arriving at NG-U ingress IP termination. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3). |
| --- | --- |

#### Average delay on F1-U

RAN2: Out of L2 measurement scope and under RAN3 responsibility.

#### Average delay DL in gNB-DU

RAN2: Feasible.

Protocol Layer: RLC, MAC

| Definition | This measurement provides the average (arithmetic mean) RLC SDU delay on the downlink within the gNB-DU, for initial transmission of all RLC packets. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). This measurement is obtained as: sum of (time when the last part of an RLC SDU was scheduled and sent to the MAC layer for transmission over the air, minus time of arrival of the same packet at the RLC ingress F1-U termination) divided by total number of RLC SDUs arriving at the RLC ingress F1-U termination. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3). Each measurement is an integer representing the mean delay in microseconds. |
| --- | --- |

### IP Latency measurement

#### IP Latency DL in gNB-DU

RAN2: Feasible

Protocol Layer: RLC, MAC

| Definition | This measurement provides the average IP Latency in DL (arithmetic mean) within the gNB-DU, when there is no other prior data to be transmitted to the same UE in the gNB-DU. The measurement is optionally split into subcounters per QoS level. This measurement is obtained as: sum of (time when the first piece of an RLC SDU transmitted on the air interface, minus time of arrival of the same packet at the RLC ingress F1-U termination, for IP packets arriving when there is no other prior data to be transmitted to the same UE in the gNB-DU) divided by total number of RLC SDUs arriving at the RLC ingress F1-U termination when there is no other prior data to be transmitted to the same UE in the gNB-DU. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3). |
| --- | --- |

### UE context Release

RAN2: Out of L2 measurement scope and under RAN3 responsibility.

### PDCP data volume measurements

RAN2: Feasible

Protocol Layer: PDCP, RLC

| Definition | This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from GNB-DU to GNB-CU or in the downlink delievered from gNB-CU to GNB-DU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). The reference point is the Service Access Point between PDCP and RLC. The measurement is done separately for: - DL - UL The unit is Mbit (1MBits=1000*1000 bits). |
| --- | --- |

# Conclusion

In section 2 we made the following observations:

Observation 1:	In TS 28.552, the following parameters were defined for performance measurement for gNB.

The proposed set of data to be collected from the gNB include but are not limited to:

- Performance measurement for all gNB deployment scenarios

  - Packet Delay

  - Radio resource utilization

  - UE throughput

  - RRC connection number

  - PDU Session Management

  - Mobility Management

  - TB related measurement

  - PDU session modification

  - PDU session release

  - DRB session Management

- Performance measurement for non-gNB deployment scenarios

- Performance measurement for spilt gNB deployment scenarios

  - Packet Loss Rate

  - Packet Drop Rate

  - Packet Delay

  - IP Latency measurement

  - UE context Release

  - PDCP data volume measurements

Observation 2:	PDU session is NAS concept, measurements on PDU session are out of L2 measurement scope.

Observation 3: RAN2 need to check/study the feasibility of rest parameters except PDU session parameters from RAN2 perspective.

Obseration 4: After RAN2 check, RAN3 need to furter check/study measurement feasibility if the measurement parameters have to do with gNB deployment scenario, i.e. CU/DU split and CU-CP/CU-DP split.

Observation 5: A LS to SA5 is needed the indicate the feasibilty of measurement of these paramters after the RAN2/RAN3 feasibility check/study.

Based on the discussion in chapter 3-5 we propose the following:

Proposal 1	A LS to RAN3 to to provide RAN2 check/stduy results in chapter 4-6 and ask for further check/confirm the feasibiltiy of these measurements which have to do with gNB. deployment scenario, i.e. CU/DU split and CU-CP/CU-DP slpit.

Proposal 2	After RAN3 check, a LS to SA2 to provide the feasibilty study output.

# References

TS 28.552 Technical Specification Group Services and System Aspects; Management and orchestration; 5G performance measurements (Release 15)

3GPP TS 36.314, Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 Measurements (Release 15)

S5-182574 LS on L2 measurement specification for NR in R15

S5-183622 LS on L2 measurement specification for NR in R15c

S5-184337 LS on L2 measurement specification for NR in R15

R3-186232 Reply LS to SA5 on L2 measurements

R3-184480 Reply LS to SA5 on L2 measurements (Reply to: S5-182574, S5-193622, S5-184337)