3GPP TSG-RAN WG2 Meeting #105	R2-1901978

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	11.9.2

Source:	CMCC

Title:	Consideration on Approaches to reduce Interruption Time during HO/SCG change

Document for:	Discussion, Decision

Introduction

According to RAN#80 in June 2018, a new WID [1] was achieved to further enhance mobility, the following is one of these objectives:

- To study solution(s) to reduce interruption time during HO/SCG change focusing on the following identified solutions but not limited.

  - Handover/SCG change with simultaneous connectivity with source cell and target cell

  - Make-before-break

  - RACH-less handover

In addition, the work starts with a SI phase to study potential solutions for their pros and cons to choose the most proper option(s), which is followed by a WI phase where the selected solution will be discussed in further detail.

In Rel-15, how to realize ‘0ms’ UP interruption during HO in NR was discussed, and following preliminary agreements were made:

- The mobility enhancement similar to that discussed for LTE (“Maintaining Source gNB connection during handover”) should be considered also for NR. - For DC (NR-NR), study how to reconfigure the UE from an MgNB to a SgNB to target the 0 ms UP interruption. FFS whether also applicable to LTE-NR - We will aim to define HO for NR with an interruption as close to zero as possible while only having single Tx/Rx in the UE, and 0ms interruption at least for the case that the UE supports simultaneous Tx/Rx with source cell and target cell during HO

In Rel-16, a new WID [2] in LTE was approved to further enhance mobility, and RAN1 just provided feedback the LS on the Interruption time during mobility in LTE [3], which can be reference to the study on the same topic in NR.

In this contribution, we give comparison between MBB HO (make before break handover) with RACH-less HO and DC-based HO, which can help to provide more explicit recognition for these two solutions.

Discussion

Handover latency

In NR, the handover latency occurs mainly on handover execution phase, a simple example of assessment of sources of latency during handover execution is presented in the following Table 1, assuming below 6GHz in NR FDD. Considering that for RACH below 6GHz, only 15 kHz and 30 kHz SCS are supported, the SCS of 15kHz and 30kHz is considered.

Table 1: Typical radio access latency components during handover in NR System

| Component/ Step | Description | Time (ms) |
| --- | --- | --- |
| 6 | Processing on RRC Connection Reconfiguration (Handover Command, L2 and RRC) | 3 |
| 7 | SN Status Transfer | 0 |
| 8.1 | Target cell search | 0 |
| 8.2 | Average UE processing time for RF/baseband re-tuning, security update | 20ms |
| 8.3 | Delay due to RACH scheduling period (1TTI) | 0 |
| 8.4 | PRACH preamble transmission | 1(non-)slot/slot |
| 8.5 | Preamble detection and processing in gNB | 2 (example, the value depending on implementation) |
|  | Transmission of RA response | 1(non-)slot/slot |
| 8.5 | UE Processing Delay | 3 |
| 8.6 | UE sends RRC Connection Reconfiguration Complete | 1(non-)slot/slot |
|  | Typical Total delay | (20+8) ms +3(non-)slots |

Table 2. Latency results for NR FDD

| Slot / non-slot duration | SCS |  |
| --- | --- | --- |
|  | 15kHz | 30kHz |
| M=2 (2OS non-slot) | 28.4 | 28.2 |
| M =4 (4OS non-slot) | 28.85 | 28.4 |
| M =14 (14OS slot) | 31 | 29.5 |

Based on above analysis, the follows observations can be derived:

Observation 1: the typical delay during handover execution in these cases is about 28.2ms~31, and it significantly impacts the user experience, especially for some URLLC services.

Observation 2: Among above various delay components, the following steps contribute to a major portion of total delay and can be addressed for possible latency reduction:

-	RACH procedure including delay to acquire first available PRACH in target cell, PRACH preamble transmission and UL allocation + TA

-	UE processing time after RA procedure including decoding of scheduling grant and timing alignment + L1 encoding of UL data, and transmission of RRCReconfigurationComplete

Potential solutions

As indicated above, the follows are three potential candidates for implementing 0ms interruption time:

- Handover/SCG change with simultaneous connectivity with source cell and target cell.

- Make-before-break

- RACH-less handover

In the following sections, about the potential solutions of DC-based HO, MBB HO and RACH-less HO will be elaborated for the target of 0 ms UP interruption.

DC-based HO

The original motivation of Dual-Connectivity (DC) is to support multi-cell simultaneous communications with a UE in the overlapped coverage of the cells even with non-ideal backhaul. Obviously, because DC-capable UE has the capability to communicate simultaneously with MgNB and SgNB, the DC framework can be adopted to achieve 0ms interruption time at least for intra-NR mobility, as shown in the figure 1, the procedure is briefly consisted of the following phases:

Phase 1: The gNB of target cell is added as S-gNB.

Phase 2: A “role change” is performed between the M-gNB and the S-gNB.

Phase 3: The S-gNB is released.

Additionally, in this mechanism, there is no necessity for UE to execute RACH procedure to establish connection with the target gNB during HO procedure, which can be completed during S-gNB addition.

Figure 1 Procedure for DC-based handover solution   Figure 2 Protocol stack for DC-based HO

      - Potential Issues

Issue 1 Interruption for PCell change

In LTE, if PCell needs to be changed, a handover procedure should be executed with RACH procedure and L2 reset which will cause interruption. In NR, to avoid interruption, enhancements for PCell change should be considered, during role change between M-gNB and S-gNB. Actually, if SgNB is changed to be MgNB, there is no need for the UE to perform RACH as it has already synchronized with the new MgNB when it worked as the SgNB. Furthermore, L2 reset/re-establishment is not required for the UE. As the cells in M-gNB and S-gNB are activated in parallel, data transmission can continue and no interruption occurs during role change.

Issue 2 PDCP relocation

When UE moves from source cell to target cell, PDCP is relocated, accompanied with path switch procedure towards CN. Then the UE anchor is changed from source gNB to target gNB, the specific protocol stack as shown in figure 2.

In order to support lossless handover, PDCP SN continues during HO when PDCP is relocated. The SN Status Transfer procedure and data forwarding is sent following RRC Connection Reconfiguration message. UE accessing to target cell and data forwarding can be performed in parallel. Generally, the SN STATUS TRANSFER message can only be generated after the source gNB stopping the data transmission. If simultaneous Tx/Rx operation with source gNB and target gNB is supported, how to perform SN status transfer and data forwarding at the source gNB, which simultaneously performs data transmission with UE, should be considered.

One baseline behavior to perform PDCP relocation through split bearer to MCG bearer change. Similar as current HO procedure, UP protocol is reset including PDCP re-establishment, RLC reestablishment and MAC reset. Although the interruption due to UP protocol reset is several microseconds and negligible, 0ms interruption can’t be achieved with this baseline behavior. The security key for target gNB can be used after UP reset. To address this, some enhancements are needed, for example, to perform PDCP relocation without UP protocol stack reset at the UE side. And one problem with this solution is that UE doesn’t know when to apply the new security key for the target gNB. One potential solution is that a time duration or the number of PDCP PDUs applying the old key corresponding to the source gNB is pre-defined or informed to UE. UE complexity to avoid UP protocol reset is concerned and needs further evaluation.

Observation 3: Considering the EN-DC and intra-NR DC is a mature feature in NR and the deployment is on the road, the existing DC framework can be adopted to achieve 0ms interruption time at least for intra-NR mobility, there will be small impact on Protocol stacks, UE implementation, gNB implementation and security handling, with the enhancements to address the issues of role change and PDCP relocation.

MBB HO

As indicated above, in current NR HO, the UE resets MAC and reestablishes PDCP upon receiving the HO command and thus communication with the source gNB is stopped. The UP interruption will happen until the UE receives the first packet from the target gNB. Make-Before-Break HO/SgNB change, which is introduced in Rel-14 LTE, is that maintaining source gNB/SgNB connection after reception of RRC message for handover or change of SgNB before the initial uplink transmission to the target gNB during handover or change of SgNB, as shown in the figure 3. Hence, only if RACH procedure is completed successfully, the source gNB stops the respective transmission to the UE.

Figure 3 Procedure for MBB HO               Figure 4 Protocol stack in UE side for MBB HO

This solution can reduce data interruption during handover by not releasing the connection to the source gNB until handover is completed at the target gNB. Continuous transmission of user data from source cell well after handover procedure start can significantly improve the user experience. And the protocol stack for the source and the target gNB is displayed in Figure 4, the UE maintain two keys at the same time, which would not bring about confusions. Moreover, different to DC-based HO, the MBB HO does not need the role change procedure which demands much signaling messages.

- Potential Issues

This requires that the UE monitor both source and target links simultaneously. To continue the data transmission with the source gNB, the difference from current HO is that the UE should continue sending CSI and HARQ feedback to the source gNB.

Issue 1 Interruption due to RLM in source gNB

If the radio link quality becomes bad before the target gNB is ready, this would force the UE to declare RLF since that would trigger RRC Connection Re-establishment, which leads the UP interruption. Hence, RLM for the source gNB needs to be suspended until the end of handover. Impact on uplink transmission should be evaluated if the UE still sends uplink signal in bad radio link quality due to suspension of RLM. And to address this issue, more than one level thresholds for handover may be setting.

Issue 2 Zero Interruption cannot be achieved in current LTE MBB mechanism

In LTE MBB, UE release connection with source just after UE sent the RACH preamble, which is before its connection with target is ready for packet transmission/reception. This cause the defined make-before-break in LTE cannot meet 0 ms interruption requirement. Hence, currently LTE MBB HO need to be enhanced to enable the UE can keep its connection with the source cell until it is able to receive packets from the target cell. For source cell, it should continue serve UE in parallel to forward data to target cell after sending HO command to the UE.

Issue 3 Intra-PDCP function split in MBB mechanism

For eMBB, the UE establishes and maintains an independent protocol stack for the target gNB including PDCP entity, which is more like DC 3B proposed in DC SID phase. Since the UE has established the independent PDCP for the target gNB, before the PDCP is relocated from the source to the target gNB, PDCP SDUs with SN assigned by the source gNB can be forwarded by the source gNB to the target gNB, then the target gNB ciphers the PDCP SDUs by its PDCP layer. To address the security issue, the UE shall maintain two keys simultaneously during HO procedure, one for the source gNB and the other for the target gNB, and it can use the corresponding key for deciphering based on which cell the PDCP packet is from. At the same time, eMBB needs to conduct intra-PDCP function split which increases the complexity. Based on above discussion, it can be observed that:

Observation 4: Based on current LTE defined MBB HO mechanism, there will be some enhancements needed to address the issue of interruption due to RLM in source gNB, to address the issue of Interruption of current LTE MBB mechanism, and to support Intra-PDCP function split in the UE side.

RACH-less mechanism

As we known, executing a RACH procedure completely may require a long time delay, which is not expected. In Rel-14 LTE, 3GPP has studied a solution of RACH-less handover, where most of the RACH procedure can be skipped during handover. Therefore, an approach was proposed is to employ the RACH-less HO and current MBB together to achieve aggregated reduction of service interruption during HO.

Obviously, RACH less handover is helpful of reduction in the data interruption time, however, several issues still need to be addressed. As we know, the main purposes of RACH procedure is to obtain target cell TA for uplink synchronization and UL grant from target cell. Regarding TA, RAN4 has evaluated that the scheme of UE calculating the TA value without explicit TA value from eNB cannot met the accuracy of TA for initial uplink transmission in either synchronous or asynchronous network, therefore LTE has agreed that the RACH-less solution is only applicable when the TA value of the source cell can be reused for the target cell or TA=0 in the target cell. Hence, existing RACH-less HO in LTE can only work if strict synchronization conditions are met, which are used only when the TA value of the source cell is reused for the targeted cell, or TA=0. It is intended for very limited scenarios such as HO across co-located carriers or across small cells with very small coverage. Therefore, for most scenarios of the HOs across different geographical cells, the condition for LTE RACH-less HO cannot be met. Whereas, current RACH-less HO may lead to that TA (Timing Advance) is not up to date. Therefore, necessary enhancement of the RACH-less HO needs to be studied if the approach is adopted.

Observation 5: Regarding TA, RAN4 has evaluated that the scheme of UE calculating the TA value without explicit TA value from eNB cannot met the accuracy of TA for initial uplink transmission in either synchronous or asynchronous network, therefore agreed RACH-less solution is only applicable in limited scenarios.

Observation 6: Based on above analysis, a comparison table between the DC-based and MBB approaches is provided as follows:

| Metric | DC-based | MBB |
| --- | --- | --- |
| Total interruption time | 0ms interruption time for both UL and DL from L2 point of view. The RF interruption time is up to RAN4 to decide. | 0ms UP interruption time and RF interruption time for both UL and DL can be achieved since UE keeps connection with source gNB until successfully accessed to the target gNB. |
| RF requirements | Dual Tx/Rx chains should be supported by UE to achieve 0ms user plane interruption time, but further consideration needs to wait for RAN4’s LS response for the requirement. | Dual Tx/Rx chains should be supported by UE to achieve 0ms user plane interruption time, but further consideration needs to wait for RAN4’s LS response for the requirement. |
| Protocol stacks Complexity | Split bearer defined in intra-NR DC can be basically re-used. And an indication to differentiate the security context is needed in PDCP. | UE shall keep two sets of protocol stacks (for SRB, DRB) for source gNB and target gNB respectively. The PDCP layer further splits into two separated (de)compression and (de)ciphering functionality and a unified SN assignment and reordering / duplication detection functionality. |
| UE Complexity | Split bearer defined in intra-NR DC can be basically re-used. And the PDCP needs to keep two security keys and two ROHC contexts in a certain period. | For the user plane aspect, UE shall be able to maintain two sets of keys and ROHC profiles. For the control plane, UE follows most of the legacy HO procedure. |
| NW Complexity | Split bearer defined in intra-NR DC can be basically re-used, where two GTP tunnels are required to reduce the downlink interruption due to data forwarding. | The NW complexity is to enable the source gNB continue data transmission/reception upon the handover command, until the target gNB is ready; Impact on the data forwarding on Xn interface. |
| Security Complexity | Only one security key needs to be maintained as in DC. And there may be security key ambiguity issue during the role change for the UE. | Two sets of keys need to be maintained at the UE side, one for source gNB and the other for target gNB. |
| PDCP Function Complexity | The PDCP reordering function in the source gNB is used to reorder PDCP PDUs from both source gNB and target gNB. The enhancement for the PDCP relocation is needed. | A tentative PDCP reordering function is used to reorder PDCP PDUs delivered from the PDCP entities in both source gNB and target gNB; This option doesn’t need UP protocol reset for PDCP relocation at the network side. |
| Applicable deployment scenarios | The intra-NR DC deployed scenarios | The eMBB solution is more applicable for LTE, since LTE has not deploy the Dual Connectivity. |
| Necessity of simultaneous RLM | The RLM procedures defined in intra-NR DC can be reused. | The simultaneous RLM is not required for simultaneous connectivity. RLM should be either based on target once the UE has camp on target cell or UE perform RLM on both links and declare RLF only when both links are RLF. |

Proposal 1: RAN2 need identify the specific metrics firstly for the comparison of these candidates to determine which one is more suitable for the NR network.

- Common issue: 0ms interruption in intra-frequency mobility scenario

For intra-frequency mobility, RAN2 had sent a LS to RAN4/RAN1 in the Rel-15 NR phase, and the following are some observations from RAN4 [4]:

A: It is feasible that UE performs simultaneous reception from two intra-frequency cells in synchronous network with single RF chain or dual RF chains.

Regarding UE with single or dual RF chain in asynchronous network case, the feasibility is FFS.

Note: RAN4 think that the dual RF chains for intra-frequency operation on the same carrier are not typical implementation.

That is, for UE with single Tx/Rx, TDM solution between two legs seems a reasonable solution. On the other hand, more study on the device implementation is need to make clear which use case is possible to realize the 0ms HO interruption. Hence, in our understanding, study of this issue in NR also need the input from RAN4/RAN1.

In addition, during RAN2#103bis meeting for LTE_feMob WI, RAN2 has sent a LS [3] to RAN1/RAN4 for the feedback on the feasible to perform simultaneous transmission/reception of various UL & DL physical channels/signals to/from two cells considering some aspects. And only RAN1 just provided the feedback [3] as follows:

Intra-frequency synchronous

Reception: Feasible for some UEs at least if source/target bandwidth are the same and under some conditions (e.g. condition of received signal power difference between source/target cells).

Transmission: Feasible for UEs with single RF chain, and source/target eNB in same PC group and under some conditions (e.g. condition of MTTD). UEs that have multiple RF chains that can be enabled for this band may be able to relax this condition.

Intra-frequency asynchronous

Reception: Feasible for UEs with single RF chain under some conditions (e.g. condition of received power difference between source/target cells). Requires separate FFT/loops with source and target cells.

Transmission: RAN1 has not concluded whether simultaneous Tx and/or Rx can be supported for this case.

However, this LS is just limited for LTE, RAN2 need send another LS to RAN4/RAN1 for the feedback for the NR network. And a draft LS to RAN4/RAN1 is provided as well.

Proposal 2: it is proposed that the study the issue of 0ms interruption in intra-frequency mobility scenarios in NR need the input from RAN4/RAN1.

Analysis on LTE vs. NR differences

As we already known, in NR, beamforming was introduced for high frequency range. As indicated in RAN4 LS [4], it is feasible that UE performs simultaneous reception from two intra-frequency cells in synchronous network with single RF chain or dual RF chains in LTE. However, in NR, at high frequency range, it is not feasible that the UE performs simultaneous reception from two intra-frequency cells since it is difficult for a UE to perform simultaneous reception from two beams of two different directions from intra-frequency cells.

Observation 7: it is feasible that UE performs simultaneous reception from two intra-frequency cells in synchronous network with single RF chain or dual RF chains in LTE. However, in NR, at high frequency range, it is difficult for a UE to perform simultaneous reception from two beams of two different directions from two intra-frequency cells.

Conclusion

Based on the discussion mentioned above, in this contribution we have the following observations and proposal:

Observation 1: the typical delay during handover execution in these cases is about 28.2ms~31, and it significantly impacts the user experience, especially for some URLLC services.

Observation 2: Among above various delay components, the following steps contribute to a major portion of total delay and can be addressed for possible latency reduction:

-	RACH procedure including delay to acquire first available PRACH in target cell, PRACH preamble transmission and UL allocation + TA

-	UE processing time after RA procedure including decoding of scheduling grant and timing alignment + L1 encoding of UL data, and transmission of RRCReconfigurationComplete

Observation 3: Considering the EN-DC and intra-NR DC is a mature feature in NR and the deployment is on the road, the existing DC framework can be adopted to achieve 0ms interruption time at least for intra-NR mobility, there will be small impact on Protocol stacks, UE implementation, gNB implementation and security handling, with the enhancements to address the issues of role change and PDCP relocation.

Observation 4: Based on current LTE defined MBB HO mechanism, there will be some enhancements needed to address the issue of interruption due to RLM in source gNB, to address the issue of Interruption of current LTE MBB mechanism, and to support Intra-PDCP function split in the UE side.

Observation 5: Regarding TA, RAN4 has evaluated that the scheme of UE calculating the TA value without explicit TA value from eNB cannot met the accuracy of TA for initial uplink transmission in either synchronous or asynchronous network, therefore agreed RACH-less solution is only applicable in limited scenarios.

Observation 6: Based on above analysis, the following comparison between the DC-based and MBB approaches is provided:

| Metric | DC-based | MBB |
| --- | --- | --- |
| Total interruption time | 0ms interruption time for both UL and DL from L2 point of view. The RF interruption time is up to RAN4 to decide. | 0ms UP interruption time and RF interruption time for both UL and DL can be achieved since UE keeps connection with source gNB until successfully accessed to the target gNB. |
| RF requirements | Dual Tx/Rx chains should be supported by UE to achieve 0ms user plane interruption time, but further consideration needs to wait for RAN4’s LS response for the requirement. | Dual Tx/Rx chains should be supported by UE to achieve 0ms user plane interruption time, but further consideration needs to wait for RAN4’s LS response for the requirement. |
| Protocol stacks Complexity | Split bearer defined in intra-NR DC can be basically re-used. And an indication to differentiate the security context is needed in PDCP. | UE shall keep two sets of protocol stacks (for SRB, DRB) for source gNB and target gNB respectively. The PDCP layer further splits into two separated (de)compression and (de)ciphering functionality and a unified SN assignment and reordering / duplication detection functionality. |
| UE Complexity | Split bearer defined in intra-NR DC can be basically re-used. And the PDCP needs to keep two security keys and two ROHC contexts in a certain period. | For the user plane aspect, UE shall be able to maintain two sets of keys and ROHC profiles. For the control plane, UE follows most of the legacy HO procedure. |
| NW Complexity | Split bearer defined in intra-NR DC can be basically re-used, where two GTP tunnels are required to reduce the downlink interruption due to data forwarding. | The NW complexity is to enable the source gNB continue data transmission/reception upon the handover command, until the target gNB is ready; Impact on the data forwarding on Xn interface. |
| Security Complexity | Only one security key needs to be maintained as in DC. And there may be security key ambiguity issue during the role change for the UE. | Two sets of keys need to be maintained at the UE side, one for source gNB and the other for target gNB. |
| PDCP Function Complexity | The PDCP reordering function in the source gNB is used to reorder PDCP PDUs from both source gNB and target gNB. The enhancement for the PDCP relocation is needed. | A tentative PDCP reordering function is used to reorder PDCP PDUs delivered from the PDCP entities in both source gNB and target gNB; This option doesn’t need UP protocol reset for PDCP relocation at the network side. |
| Applicable deployment scenarios | The intra-NR DC deployed scenarios | The eMBB solution is more applicable for LTE, since LTE has not deploy the Dual Connectivity. |
| Necessity of simultaneous RLM | The RLM procedures defined in intra-NR DC can be reused. | The simultaneous RLM is not required for simultaneous connectivity. RLM should be either based on target once the UE has camp on target cell or UE perform RLM on both links and declare RLF only when both links are RLF. |

Observation 7: it is feasible that UE performs simultaneous reception from two intra-frequency cells in synchronous network with single RF chain or dual RF chains in LTE. However, in NR, at high frequency range, it is difficult for a UE to perform simultaneous reception from two beams from two intra-frequency cells.

Proposal 1: RAN2 need identify the specific metrics firstly for the comparison of these candidates to determine which one is more suitable for the NR network.

Proposal 2: it is proposed to send a LS to RAN4/RAN1 for the study the issue of 0ms interruption in intra-frequency mobility scenarios in NR network.

Reference

- RP-181475 New Work Item on even further Mobility enhancement in E-UTRAN

- R2-1900020 Reply LS on the Interruption time during mobility in LTE

- R4-1706913 Reply LS on the feasibility of DC-related mobility enhancements in NR

- R2-1815706 LS on the interruption time during mobility in LTE NOKIA

- R2-1901979 Draft LS on the interruption time during HO SCG in NR