3GPP TSG-RAN WG2 Meeting #105	R2- 1902005

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.11.3 (FS_NR_UE_pow_sav)

Source	: LG Electronics Inc.

Title	: Quick transition to RRC_INACTIVE

Document for	: Discussion and Decision

1.	Introduction

For better UE battery life, Study on UE Power Saving in NR has been approved with Study the enhancement of  UE power saving procedure in supporting efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE mode as one of the objectives [RP-181463].

In this contribution, we discuss efficient transition from RRC_CONNECTED to RRC_INACTIVE and provide one potential method for this.

2.	Discussion

In NR, RRC_INACTIVE has newly been introduced for purpose of power saving and signaling overhead reduction. Thanks to RRC_INACTIVE, the UE can stay in RRC_INACTIVE rather than RRC_CONNECTED or RRC_IDLE if data transmission/reception is not expected for a while. Staying in RRC_INACTIVE, the UE can save power more dramatically than in RRC_CONNECTED while there is no data transfer activity and make an RRC_CONNECTION efficiently when data transfer is expected again.

Thus, it is understood that RRC_CONNECTION is more targeted by throughput enhancement for a UE having data activity while RRC_INACTIVE is well motivated by power saving for a UE having no data transfer activity. In this sense, it is important to quickly transit from RRC_CONNECTED to RRC_INACTIVE when no data transfer activity is expected in the UE side.

Currently, transition to RRC_INACTIVE is only initiated by the network and the UE is not allowed to reject it. Given that RRC_INACTIVE is designed for the UE having no data transfer activity, the network may rely on UE’s buffer size to determine whether to transit the UE to RRC_INACTIVE. For instance, if zero buffer size is reported, the network may send the UE to RRC_INACTIVE for the sake of UE’s power saving. In the meanwhile, the UE keeps monitoring the PDCCH in RRC_CONNECTED even after reporting zero buffer size, which consumes the UE’s power only to go to RRC_INACTIVE. Therefore, we think one potential method to quickly go to RRC_INACTIVE is to allow UE to go to RRC_INACTIVE based one its buffer status report, e.g., zero buffer size, without waiting for the suspension of the RRC connection from the network.

Proposal. Discuss efficient transition from RRC_CONNECTED to RRC_INACTIVE based on UE’s buffer size.

3.	Conclusion

In this contribution, we discuss efficient transition to RRC_INACTIVE and provided one potential method to use buffer size.

Proposal. Discuss efficient transition from RRC_CONNECTED to RRC_INACTIVE based on UE’s buffer size.