3GPP TSG-RAN2 105                                                 R2-1902081

Athenes, Greece, 25th February - 1st March 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0963 | Rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MIB acquisition upon Reconfiguration with Sync |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of reconfiguration with sync, the UE acquires MIB in order to get the timining information i.e. acquirng the SFN if needed. All the IEs in MIB except SFN are included in the RRCReconfiguration message, i.e. MIB Parameters; Needed in RRC_CONNECTED?; Signaled in RRCReconfiguration?. systemFrameNumber; Yes; No. subCarrierSpacingCommon; Yes; Yes [ServingCellConfigCommon downlinkConfigCommon initialDownlinkBWP genericParameters ]. ssb-SubcarrierOffset*; No; No. dmrs-TypeA-Position; Yes; Yes [ ServingCellConfigCommon ]. pdcch-ConfigSIB1; Yes; Yes [ServingCellConfigCommon downlinkConfigCommon initialDownlinkBWP pdcch-ConfigCommon ]. cellBarred; No; N/A. intraFreqReselection; No; N/A. * ssb-SubcarrierOffset is not needed in RRC connected state. During initial access, this parameter together with ‘offset’ indicated by pdcch-ConfigSIB1 determines the starting location (in frequency domain) of RMSI. In the RRC connected, absoluteFrequencyPointA (ServingCellConfigCommon àdownlinkConfigCommonà frequencyInfoDL) and locationAndBandwidth (ServingCellConfigCommon àdownlinkConfigCommonà initialDownlinkBWPà genericParameters) can be used to determine the starting location (in frequency domain) of RMSI. So, the UE does not need to acquire MIB if the UE already know SFN. According to the current procedure text in clause 5.3.5.5.2, regardless of whether UE acquires MIB or not, the UE shall perform the actions in clause 5.2.2.4.1. But the only relevant action from 5.2.2.4.1 is storing the acquired MIB. If UE follows the current statement, even if UE already knows SFN, UE has to acquire MIB just to perform the action in 5.2.2.4 (i.e. acquire MIB and store the information which was already available in the UE even before MIB acquisition) which only pose unnecessary UE requirement without any reason. |
| Summary of change: |  | Remove the bullet "1> perform the actions specified in clause 5.2.2.4.1;" in clause 5.3.5.5.2 Impact analysis Impacted functionality: Reconfiguration with sync Inter-operability: 1. If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may pose unnecessary UE requirement to store the the acquired MIB in case of reconfiguration with sync. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

<Start of modification 1>

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell;

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

Editor's Note: Verify that this does not configure some common parameters which are later discarded due to e.g. SCell release or due to LCH release.

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

<End of modification 1>