3GPP TSG-RAN WG2 Meeting #105	R2-1902305

# Athens, Greece, 25th Feb – 1st Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0886 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the undefined RACH parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R15 NR, we hvae defined RACH resource in the following IEs: - RACH-ConfigCommon, same as LTE - BeamFailureRecoveryConfig - SI-SchedulingConfig for on-demand SI request - RACH-ConfigDedicated for rapid RACH access While for each RACH configurationv of the last three bullets in the above four, it is configured via RACH-ConfigGeneric and SSB-perRACH-Occasion. It is still unclear whether other RACH parameters in RACH-ConfigCommon are needed or not or when some of them are needed, according to what value the UE should perform RACH. In the following we would like to examine them one by one. But we first observe that for all of them the following parameters inside RACH-ConfigCommon are not needed: - totalNumberofRA-Preambles=> not needed - ssb-perRACH-OccasionAndCB-PreamblesPerSSB =>not needed, already given by ssb-PerRACH-Occasion - RACH-ConfigGeneric=> already given The following table summarizies for the othre parameters, whether they are needed for the three cases BeamFailureRecoveryConfig; RACH-ConfigDedicated; SI-ScheudlingConfig. groupBconfigured; N; N; N. ra-ContentionResolutionTimer; N; N; N. rsrp-ThresholdSSB; Y, already given; Y; Y. rsrp-ThresholdSSB-SUL; N; N; YNOTE1. prach-RootSequenceIndex; Y, already given; Y; Y. msg1-SubcarrierSpacing; Y, already given; Y; YNOTE2. restrictedSetConfig; NNOTE4; Y; YNOTE3. msg3-transformPrecoder; N; N; N. NOTE1: Useful for UL and SUL selection if SUL is configured, only for SUL NOTE2: Needed for determining the SCS for short sequence preamble format. NOTE3: Needed, since the preamble format can be short and long. In case of long sequence, support for restricted sets needs to be configured. NOTE4: Not needed since BFR uses short sequence We make the following observations based on the above analysis: - For SI-ScheudlingConfig, the following parameters: rsrp-ThresholdSSB, rsrp-ThresholdSSB-SUL (only for SUL), prach-RootSequenceIndex, msg1-SubcarrierSpacing, and restrictedSetConfig are needed for the dedicated PRACH configuation - For BeamFailureRecoveryConfig, the current configuration is enough - For RACH-ConfigDedicated, rsrp-ThresholdSSB, prach-RootSequenceIndex, msg1-SubcarrierSpacing and restrictedSetConfig are needed Since some of them are needed for BFR and RACH-ConfigDedicated, the next step is how they should be provided, given the fact that these parameters are currently not available in the RACH configuration. - One solution is to add new fields to these IEs, but considering the NBC issue, this is highly un-desirable - Another solution is to reuse the current parameters in RACH-ConfigCommon. This solution can let the RACH mechanism work under the current framework, while also avoiding the NBC issue. - For SI, it can use the RACH-configCommon in SIB1 - For RACH-ConfigDedicated, it can use the RACH-ConfigCommon in the handover command |
| Summary of change: |  | 1/ Add in the field desctiption of si-RequestConfig and si-RequestConfig-SUL that for the following parameters for RACH: rsrp-ThresholdSSB, rsrp-ThresholdSSB-SUL, prach-RootSequenceIndex, msg1-SubcarrierSpacing, and restrictedSetConfig, it shall adopt the correspoding values in RACH-ConfigCommon for UL and SUL 2/ In the field description of occasions under RACH-ConfigDedicated, clarify that the parameters rsrp-ThresholdSSB, prach-RootSequenceIndex, msg1-SubcarrierSpacing and restrictedSetConfig follow the configuration in RACH-ConfigCommon. The first change is applicable for SA only while the second is applicable for both SA and NSA. Impact analysis Impacted functionality SI-SchedulingInfo, RACH-ConfigDedicated Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE is not able to perform RACH on the indicated dedicated RACH resource since it does not know what parameters to use, while the network expects the UE to do so on the configured RACH. - If the UE is implemented according to the CR and the network is not, the network does not know how to receive RACH from the UE since it does not know what root sequence the UE shall use. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | On-demand SI request and handover on the dedicated RACH resource cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

==================================================FIRST CHANGE================================================================

–	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG-OTHER-SI-INFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280},

si-RequestConfig                    SI-RequestConfig                                                        OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                        OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                                  OPTIONAL,   -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                          OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                                        OPTIONAL -- Need S

}

-- Configuration for Msg1 based SI Request

SI-RequestConfig::=                 SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                               OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}               OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                             OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                             OPTIONAL    -- Need R

}

-- TAG-OTHER-SI-INFO-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occassions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus is set to notBroadcasting. Otherwise the 1st entry in the list corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is not present, the SIB is cell specific.

si-Periodicity Periodicity of the SI-message in radio frames. rf8 corresponds to 8 radio frames, rf16 corresponds to 16 radio frames, and so on.

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. If the field is not present the UE uses Msg3 to request SI-messages for which si-BroadcastStatus is set to notBroadcasting (if any). For the RACH parameters rsrp-ThresholdSSB, prach-RootSequenceIndex, msg1-SubcarrierSpacing, and restrictedSetConfig, they follow the correspoding values in RACH-ConfigCommon in SIB1.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. If the field is not present the UE uses Msg3 to request SI-messages for which si-BroadcastStatus is set to notBroadcasting (if any) on supplementary uplink. For the RACH parameters rsrp-ThresholdSSB, rsrp-ThresholdSSB-SUL, prach-RootSequenceIndex, msg1-SubcarrierSpacing, and restrictedSetConfig, they follow the correspoding values in RACH-ConfigCommon configured for supplementary uplink in SIB1.

si-WindowLength The length of the SI scheduling window. s5 corresponds to 5 slots, s10 to 10 slots and so on.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN.

SchedulingInfo field descriptions

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting.

| Conditional presence | Explanation |
| --- | --- |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. For SIB6, SIB7 and SIB8 it is not present. |
| SUL-MSG-1 | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |

===========================================SECOND CHANGE====================================================================

–	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIG-DEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                            OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                               OPTIONAL, -- Need N

...

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}  OPTIONAL  -- Cond SSB-CFRA

}                                                                                                               OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles-v1530 INTEGER (1..63)                         OPTIONAL        -- Cond Occasions

]]

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIG-DEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP. For RACH parameters rsrp-ThresholdSSB, prach-RootSequenceIndex, msg1-SubcarrierSpacing and restrictedSetConfig, they follow the corresponding values in RACH-ConfigCommon in handover command.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321. The mask is valid for all SSB resources signalled in ssb-ResourceList

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion').

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If the field is absent, the UE performs contention based random access.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| SSB-CFRA | The field is mandatory present if the field resources in CFRA is set to ssb; otherwise it is not present. |
| Occasions | The field is optionally present if the field occasions is present, Need S; otherwise it is not present. |

============================================END OF CORRECTIONS===============================================================