3GPP TSG-RAN WG2 Meeting #105	R2-1902437

# Athens, Greece, 25th February – 1st March 2019

Title:	DRAFT Reply LS on EDT integrity protection

Response to:	R2-1900088_S3-190454

Release:	Release 15

Work Item:	LTE_eMTC4-Core, NB_IOTenh2-Core

Source:	Huawei [TSG RAN WG2]

To:	SA3

Cc:	RAN3

Contact Person:

#### Name:	Odile Rollinger

E-mail Address:	odile.rollinger@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA3 for their LS on EDT data integrity protection.

RAN2 has discussed the three options for the case of multiple PDCP PDUs and would like to report the following feedback.

- Option 1 and option2:

RAN2 understands that it is not possible for the target eNB to extract the PDCP PDUs to calculate the HASH values as the UE context is not available. From RAN2 point of view, an alternative could be to forward the UL EDT data to the source eNB for the integrity protection check. RAN2 further discussed that a single PDCP PDU is an appropriate use case for EDT and that it would simplify solutions based on PDCP PDU by avoiding the need to specify or convey the order of HASH calculation.

- Option 3:

Option 3 has large RRC/MAC modeling impact and large impact on UE complexity and processing delay as the MSG3 MAC PDU is built upon reception of the UL grant and the RRC message should be available at this time.  Furthermore, it is RAN2 understanding that conventionally the security aspects are not handled in MAC layer. Therefore, this solution is unacceptable.

RAN2 respectfully asks SA3 to take the above feedback into account and to reconsider Integrity Protection for User data solution.

2. Actions:

To SA3:

RAN2 respectfully asks SA3 to take the above feedback into account and to reconsider Integrity Protection for User data solution.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105bis 		8 - 12 Apr, 2019    			China, Xi’an

3GPP RAN2#106		13 - 17 May, 2019   	 		USA, Reno