3GPP TSG-RAN WG2 Meeting #105bis                                                   R2-1903122

Xi’an, China, 08th Apr – 12th Apr 2019                                        Revision of R2-1900203

Source:	CATT

Title:	Uplink Transmission Enhancements for Power Saving

Agenda Item:	11.11.4.4

Document for:	Discussion and Decision

# Introduction

In RAN#80 the UE Power Saving SI was agreed, which some of the objectives are listed below [1]:

- Identify techniques for UE power saving study with focus in RRC_CONNECTED mode [RAN1, RAN2]

  - Study UE adaptation to the traffic and UE power consumption characteristics in frequency, time, antenna domains, DRX configuration, and UE processing timeline for UE power saving

(Note: existing UE capabilities are assumed for UE processing timeline)

    - Network and/or UE assistance information

    - Include mechanism in reducing PDCCH monitoring, taking into account current DRX scheme

  - Study the power saving signal/channel/procedure for triggering adaptation of UE  power consumption characteristics

It can be seen that one of the objectives is to study mechanism in reducing PDCCH monitoring for DRX. In this document, further analysis on uplink transmission enhancement for DRX is carried out. And it can be seen from the simulation results that the proposed uplink transmission enhancements for DRX bring gains for power saving.

# Discussion

## Description of the issue

In Rel-15 NR, unlike in LTE, a UE can expect receiving/transmitting DL configured assignments (DL CA) and/or configured UL grants (UL CG) during DRX Inactive Time, which triggers switching to Active Time so as to monitor any potential retransmission request (for UL) and PDSCH re-transmission (for DL), as illustrated in Figure 1. This actually makes NR DRX less power efficient than LTE regarding SPS/CG handling.

Figure 1: SPS/CG transmission/reception during DRX Inactive Time with current specification

Of course, this issue can be circumvented by gNB configuring, as in LTE, the DRX parameters to match the configured assignments/grants, e.g. same period and offset, so that DL CA and UL CG always fall within the DRX Active Time. This is possible when only one SPS/CG configuration is supported. However RAN1 agreed [5] to support multiple CG configurations per BWP in Rel-16: then how to deal with different CG periods in DRX? Activity bursts are no longer periodic in which case a single DRX configuration can only aim at matching one SPS/CG configuration while allocated resources from other SPS/CG configurations will unavoidably fall within DRX Inactive Time.

Observation 1: Supporting multiple SPS/CG configurations with independent periodicities/offsets with a single DRX configuration unavoidably implies that some SPS/CG resources occur within DRX Inactive Time.

However, any UL/DL transmission/reception during DRX Inactive Time comes with a significant power overhead compared with Active Time mainly due to the required ramp up and down times at UE wake-up / go-to-sleep, as shown in Figure 2 extracted from Appendix 8.1.1 of [2]:

Figure 2:  Illustration of UE power consumption at state transition

In the following, we focus on the cost of an UL transmission in a configured grant during DRX Inactive Time: Table 1 summarizes, from power values provided in Appendix 8.1.1 of [2] (for FR1), the relative power consumption with respect to Deep Sleep (=1) of the various processes involved during DRX with UL-only traffic. Specifically, [2] defines two components for the relative power consumption of the ramp-up/down state transitions: a power component per ms, proportional to the total transition time (= 20 / ms for Light Sleep) and an additional constant energy (= 100 for Light Sleep). [2] further defines a total transition time of 6 ms for Light Sleep thus resulting in a total relative power for transitioning from/to Light Sleep of 20x6 + 100 = 220. The equivalent calculation for Deep Sleep yields: 20x1 + 450 = 470. Thus, the additional transition energy required to transmit from DRX Inactive Time is quite significant compared to the PUSCH transmission itself.

Table 1: Relative Power Consumption Assumption [2]

| Parameter | Assumption |
| --- | --- |
| Deep Sleep | 1 (per ms) |
| Light Sleep | 20 (per ms) |
| PDCCH-only | 100 |
| Long PUCCH or PUSCH | 250, 0dBm; 700, 23 dBm |
| Ramp Up + Ramp Down to/from Light Sleep | 220 |
| Ramp Up + Ramp Down to/from Deep Sleep | 470 |

Observation 2: Uplink transmission during DRX Inactive Time comes with significant power consumption overhead involving the ramp up / down times at UE wake-up / go-to-sleep and the PDCCH monitoring during drx-RetransmissionTimerUL.

However, some services do not require strict low latency, like web browsing, social networking background tasks, etc. Therefore, it can be beneficial to delay their uplink transmission to the next DRX Active Time for reducing the power consumption especially for an aperiodic uplink transmission dominant traffic.

Observation 3: Enhancements are necessary for reducing the power consumption of uplink transmission in DRX.

## Uplink Transmission Enhancement Schemes for DRX

As mentioned above, one of the potential solutions can be to delay the uplink transmission of non-time-critical LCHs to the next DRX Active Time. In other words, based on their QoS or LCP channel restriction parameter(s) some LCH would not be allowed to use configured grant resources if not in DRX Active Time. It can be argued that LCP channel restrictions could be used to restrict the usage of such configured grants to high-priority / time-critical LCHs (via existing configuredGrantType1Allowed parameter), thus preventing other (best effort) LCHs to trigger a switch to Active Time during Inactive Time. However, this would also apply to the DRX Active Time and would unnecessarily prevent such best-effort LCHs to use configured grants in these time intervals. We think it can also be quite restrictive to forbid all best effort LCHs to use configured grants, also considering the larger usage and amount of configured grants configurations in Rel-16.

We can generalize the delaying to the next Active Time slot of low latency LCH MAC PDU transmitted on configured grant resources to other UL transmissions e.g. RA preamble and SR:

For SR triggered by delay tolerant traffic, it was proposed in [3] that it is not transmitted during DRX inactive state. Due to the time budget, this option was not supported for Rel-15. This can be re-discussed.

The random access procedure is triggered by a number of events including RRC connection setup/re-establishment/resume, handover, out of sync, SCell addition, SI request, BFR, RA-SR and etc. However, similar to the D-SR, an RA-SR triggered by delay tolerant traffic in DRX inactive state can be delayed to the next DRX Active Time slot.

The above uplink transmission enhancement schemes are illustrated in Figure 3.

Figure 3: Uplink Transmission Enhancement Schemes for DRX

## Simulation Results for Uplink Transmission Enhancement in DRX

In this section, simulation results are provided to show the gains provided by delaying a non-time-critical PUSCH transmission to the next DRX Active Time. Figure 4 illustrates the power consumption gain w.r.t. different drx-RetransmissionTimerUL values. The simulation assumption on FTP traffic model and DRX parameters are provided in Annex A1 and A2 respectively.

Figure 4: Power Consumption Gains from Uplink Transmission Enhancements in DRX

It can be observed that above 20% power consumption gain can be achieved with enhanced uplink transmission scheme. The gain is due to:

- No ramp up/down power consumption during DRX Inactive

- No power consumption for PDCCH monitoring during DRX Inactive

- Less PUSCH transmissions since multiple UL transmissions (including the delayed one) may be transmitted in one opportunity

Observation 4: With uplink transmission enhancement scheme, power saving consumption gain can be achieved. For example: with FTP3 file size of 0.5Mbytes, above 20% power consumption gain can be achieved.

In conclusion, we propose:

Proposal 1: Uplink transmission enhancements in DRX should be studied including:

- Delaying to the next DRX Active Time UL transmissions of low latency data arriving during DRX Inactive Time

- Delaying to the next DRX Active Time a D-SR triggered during DRX Inactive Time by a low-latency LCH

- Delaying to the next DRX Active Time an RA-SR triggered during DRX Inactive Time by a low-latency LCH

Proposal 2: Capture the evaluations in this contribution into the TR 38.840.

Proposal 3: Capture the proposed uplink enhancement schemes in this contribution into the TR 38.840.

# Conclusion

In this document, we studied the impact of simulation result on percentage of power consumption percentage is presented and shows that:

Observation 1: Supporting multiple SPS/CG configurations with independent periodicities/offsets with a single DRX configuration unavoidably implies that some SPS/CG resources occur within DRX Inactive Time.

Observation 2: Uplink transmission during DRX Inactive Time comes with significant power consumption overhead involving the ramp up / down times at UE wake-up / go-to-sleep and the PDCCH monitoring during drx-RetransmissionTimerUL.

Observation 3: Enhancements are necessary for reducing the power consumption of uplink transmission in DRX.

And possible solutions are provided in this contribution. We also provided the evaluation of the gains of uplink transmission enhancement and it can be seen that:

Observation 4: With uplink transmission enhancement scheme, power saving consumption gain can be achieved. For example: with FTP3 file size of 0.5Mbytes, above 20% power consumption gain can be achieved.

And we kindly propose that:

Proposal 1: Uplink transmission enhancements in DRX should be studied including:

- Delaying to the next DRX Active Time UL transmissions of low latency data arriving during DRX Inactive Time

- Delaying to the next DRX Active Time a D-SR triggered during DRX Inactive Time by a low-latency LCH

- Delaying to the next DRX Active Time an RA-SR triggered during DRX Inactive Time by a low-latency LCH

Proposal 2: Capture the evaluations in this contribution into the TR 38.840.

Proposal 3: Capture the proposed uplink enhancement schemes in this contribution into the TR 38.840.

# Reference

- RP-181463, New SID: Study on UE Power Saving in NR, 3GPP TSG RAN Meetings #80, La Jolla, USA, 11th  – 14th  June 2018;

- TR 38.840 v0.1.0, Study on UE Power Saving (Release 16);

- R2-1815305	, Power saving for pending SR of delay-tolerate service, Huawei, HiSilicon, 3GPP TSG-RAN WG2#103bis, Chengdu, China, 8– 12 Oct 2018;

- 3GPP TR 36.814, Further advancements for E-UTRA physical layer aspects

- R1-1812102, Final Report of 3GPP TSG RAN WG1 #94bis v1.0.0, 3GPP TSG-RAN WG1#94bis, Chengdu, China, 8 – 12 Oct 2018;

# Annex A.1

Table A1: FTP traffic model [4]

| Parameter | Assumption |
| --- | --- |
| FTP Model | Model 3 |
| Packet Size | 0.5Mbytes |
| User arrival rate λ | Poisson distributed with arrival rate λ, λ = 0.2 |

# Annex A.2

Table A2: DRX Parameters [2]

| Parameter | Assumption |
| --- | --- |
| On Duration Timer (ms) | 8 |
| DRX Cycle (ms) | 160 |
| drx-RetransmissionTimerUL | sl24 (12ms); sl64 (32ms); sl128 (64ms); sl160 (80ms); sl320 (160ms) |
| PDCCH | One per slot |
| PUSCH Uplink transmission power | Random distribution from 0dB to 23dB among different UEs |
| Sleep State during DRX Inactive Time | Light Sleep |