3GPP TSG-RAN WG2 Meeting #105bis	R2-1903254

Xian, China, 08 – 12 April 2019

Agenda item:	11.10.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Failure Cases

WID/SID:	LTE_NR_DC_CA_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In order to structure the discussion around possible improvements for recovery, it is important to distinguish the different failure cases that can arise when CA, DC or both are used. The purpose of this contribution is to investigate those cases.

# 2	Radio Link Failure

Detection of radio link failure is specified in 38.331:

#### 5.3.10.3 Detection of radio link failure The UE shall: 1> upon T310 expiry in PCell; or 1> upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or 1> upon indication from MCG RLC that the maximum number of retransmissions has been reached: 2> if CA duplication is configured and activated; and for the corresponding logical channel allowedServingCells only includes SCell(s): 3> initiate the failure information procedure as specified in 5.7.5 to report RLC failure. 2> else: 3> consider radio link failure to be detected for the MCG i.e. RLF; 3> if AS security has not been activated: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'; 3> else if AS security has been activated but SRB2 and at least one DRB have not been setup: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 3> else: 4> initiate the connection re-establishment procedure as specified in 5.3.7. The UE shall: 1> upon T310 expiry in PSCell; or 1> upon random access problem indication from SCG MAC; or 1> upon indication from SCG RLC that the maximum number of retransmissions has been reached: 2> if CA duplication is configured and activated; and for the corresponding logical channel allowedServingCells only includes SCell(s): 3> initiate the failure information procedure as specified in 5.7.5 to report RLC failure. 2> else: 3> consider radio link failure to be detected for the SCG i.e. SCG-RLF; 3> initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

While random access problem detection is specified in 38.321:

2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider the Random Access procedure unsuccessfully completed. 3> else if the Random Access Preamble is transmitted on a SCell: 4> consider the Random Access procedure unsuccessfully completed.

In terms of configurations, the range of cases to consider goes from having only one cell configured to having DC with CA on both CGs. In terms of failures, we need to distinguish radio problems, RACH and RLC failures. Table 1 below summarises the possible MCG failure cases where it can be observed that in some scenarios, there is a choice for MCG recovery via SCells of MCG and via SCG.

Observation 1: when both CA and DC are configured, MCG failure recovery could either be done via SCells of MCG or via SCG.

Table 2 below summarises SCG failure cases, where it can be observed that SCG recovery could be done via SCells of SCG. However, considering that SCG Failure information is already in place, we do not think that additional enhancements would be required.

Table 1: MCG Failure Cases

| Case | MCG |  | SCG |  | Failure | Specified Behaviour | Fast MCG Recovery via |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell(s) | PCell | SCell(s) |  |  |  |
| 1 | 1 | - | - | - | Radio Problems on PCell of MCG | Connection Re-establishment | - |
| 2 | 1 | m | - | - | Radio Problems on PCell of MCG | Connection Re-establishment | SCells of MCG |
| 3 | 1 | - | 1 | n | Radio Problems on PCell of MCG | Connection Re-establishment | SCG |
| 4 | 1 | m | 1 | n | Radio Problems on PCell of MCG | Connection Re-establishment | SCells of MCG or SCG |
| 5 | 1 | - | - | - | RACH Failure on MCG | Connection Re-establishment | - |
| 6 | 1 | m | - | - | RACH Failure on MCG | Connection Re-establishment | SCells of MCG |
| 7 | 1 | - | 1 | n | RACH Failure on MCG | Connection Re-establishment | SCG |
| 8 | 1 | m | 1 | n | RACH Failure on MCG | Connection Re-establishment | SCells of MCG or SCG |
| 9 | 1 | - | - | - | Primary RLC failure in MCG | Connection Re-establishment | - |
| 10 | 1 | m | - | - | Primary RLC failure in MCG | Connection Re-establishment | SCells of MCG not mapped to failed RLC entity |
| 11 | 1 | - | 1 | n | Primary RLC failure in MCG | Connection Re-establishment | SCG |
| 12 | 1 | m | 1 | n | Primary RLC failure in MCG | Connection Re-establishment | SCells of MCG not mapped to failed RLC entity, or SCG |

Table 2: SCG Failure Cases

| Case | MCG |  | SCG |  | Failure | Specified Behaviour | Fast SCG Recovery via |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell(s) | PCell | SCell(s) |  |  |  |
| 13 | 1 | - | 1 | - | Radio Problems on PCell of SCG | SCG Failure Information | - |
| 14 | 1 | m | 1 | n | Radio Problems on PCell of SCG | SCG Failure Information | SCells of SCG |
| 15 | 1 | - | 1 | n | RACH Failure on SCG | SCG Failure Information | - |
| 16 | 1 | m | 1 | n | RACH Failure on SCG | SCG Failure Information | SCells of SCG |
| 17 | 1 | - | 1 | n | Primary RLC failure in SCG | SCG Failure Information |  |
| 18 | 1 | m | 1 | n | Primary RLC failure in SCG | SCG Failure Information | SCells of SCG not mapped to failed RLC entity |

# 3	Conclusion

This contribution has analysed the failure cases that can occur when both DC and CA are configured and the following observation was made:

Observation 1: when both CA and DC are configured, MCG failure recovery could either be done via SCells of MCG or via SCG.

It is therefore suggested to discuss how to prioritise the two.