3GPP TSG-RAN WG2 Meeting #105bis                                                             	R2-1903352

Xi'an, China, April 8th - 12th 2019                                              	Revision R2-1900278

Source:	CATT

Title:	Handling of Conditional Handover Failure

Agenda Item:	12.3.3.1

Document for:	Discussion and Decision

Introduction

In the RAN2#104 meeting, the agreement about conditional handover configuration was made as following:

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

From the agreement, network may configure more than one candidate cells for UE to choose one to access. Considering more than one cells prepared to allow the UE access, how to handle the candidate cells by the UE needs to be discussed.

Discussion

- Normal handover

In current handover procedure, the source cell sends the handover command to the UE which includes only one target handover cell. In the handover command, the target cell provides the configuration for the UE, and provides timer T304 to control the handover procedure. Upon the reception of the handover command, the UE is configured according to the handover command and starts timer T304, if the T304 is expired, the UE will consider handover failure and initiates the connection re-establishment procedure.

Observation 1: In normal handover procedure, only one target cell is configured for the UE, and the UE will initiate RRC connection reestablishment procedure upon T304 expiry.

- Conditional handover

Different from normal handover, the network may configure more than one candidate cells for the UE as the target handover cells in conditional handover. If the network configures more than one candidate cells for a UE, each candidate cell will store the configuration and reserves dedicated resources for the UE. Upon the reception of conditional handover command, the UE will not perform handover immediately. The UE initiates the handover to one candidate cell upon the channel quality of this cell meets the handover execution condition.

Observation 2: More than one candidate cells may be configured for the UE in conditional handover.

- Handling of handover failure

For the first time that the UE tries to access to one candidate cell, if the handover is failed, which procedure should be triggered needs to be discussed. Considering the source cell and other candidate cells besides the first target cell also store the UE configuration, the re-establishment procedure may not be the optimality policy for conditional handover failure.

- Initiation Reestablishment

If the conventional handover failure handling is used for conditional handover, that is when the UE fails to access the target cell, the UE will initiate RRC reestablishment procedure. An example of the procedure is as the following figure shown:

Figure 1 first handover attempt failure trigger RRC reestablishment

Random access with contention and context fetch procedure will be performed for RRC re-establishment, even for the reestablishment successful procedure, three RRC signaling messages should be used.

- Initiation access to other candidate cell

Excluding the serving cell and the first target candidate cell, there are other candidate cells that have the configuration of the UE. If the UE initiates access to these candidate cells according to handover procedure, the timer of context fetch from source cell will be avoided. An example is shown in the figure:

Figure 2 first handover attempt failure with handover to other candidate cell

As figure 2 shown, only one RRC signalling message is needed to access the target cell according to handover procedure, and the contention free random access procedure can be triggered if the dedicated RA configuration is configured by the target candidate cell. Comparing with reestablishment to other cells, re-initiation of handover to other candidate cells will reduce the latency of RRC connection and data transmission.

Observation 3: It will reduce the latency of RRC connection and data transmission if UE access to other candidate cells upon the failure of handover attempt to one candidate cell comparing with initiation of RRC reestablishment.

For conditional handover, the configuration of the multiple candidate cells should be taken full advantage. Upon the handover failure, the UE should take priority of the candidate cells than other cells to achieve fast recovery. The RRC re-establishment shouldn’t be initiated upon the handover failure for conditional handover with more than one candidate cells. The details of the mechanism should be discussed further.

Proposal 1: RRC reestablishment procedure shouldn’t be initiated when UE fails to handover to the selected cells if the UE is configured with more than one candidate cells.

Proposal 2: Fast recovery mechanism compared to the conventional connection re-establishment procedure in the scenario where the UE is configured with more than one candidate cells should be discussed.

Conclusion

In this contribution, we propose:

Observation 1: In normal handover procedure, only one target cell is configured for the UE, and the UE will initiate RRC connection reestablishment procedure upon T304 expiry.

Observation 2: More than one candidate cells may be configured for the UE in conditional handover.

Observation 3: It will reduce the latency of RRC connection and data transmission if UE access to other candidate cells upon the failure of handover attempt to one candidate cell comparing with initiation of RRC reestablishment.

Proposal 1: RRC reestablishment procedure shouldn’t be initiated when UE fails to handover to the selected cells if the UE is configured with more than one candidate cells.

Proposal 2: Fast recovery mechanism compared to the conventional connection re-establishment procedure in the scenario where the UE is configured with more than one candidate cells should be discussed.

Reference

- RAN2-104-Spokane-R16 feMOB_2018-11-14_final ;