3GPP TSG-RAN WG2#105-Bis	Tdoc R2-1903512

Xian, China, 8th – 12th April 2019

Agenda Item:	11.9.2.2

Source:	Ericsson

Title:	Reduced handover interruption time for services using RLC UM

Document for:	Discussion, Decision

# Introduction

At the RAN#80 meeting in June 2018, a new Rel-16 work item was approved [1] with the purpose to further enhance NR mobility by reducing handover interruption time and improve mobility robustness.

RAN2#105 was the first meeting where the NR mobility enhancement work item was discussed.
Following agreements were reached that affects reduced handover interruption time:

Agreements

1	The UE ability to simultaneously receive and transmit to/from the source and target cells is to be considered in the study on NR mobility enhancements.

2	We prioritize on intra-NR handovers in this WID.

This contribution addresses NR mobility enhancement and reduced handover interruption time in combination with low latency and high reliability type of services.

# Discussion

In NR, 0ms interruption time is one of the requirements to provide seamless handover UE experience. According to the WI description on NR mobility enhancements [1], mobility performance is one of the most important performance metrics for NR, thus it is important to identify handover solutions that renders high handover performance with 0ms interruption, low latency and high reliability.

In Rel-15, 0ms interruption time can be achieved with beam mobility for intra-cell handover or by means of CA operation by adding and releasing the SCell in response to mobility. In Rel-16, the requirements are extended to achieve 0ms handover interruption time in more scenarios, especially for URLLC type of services where further use cases with tighter requirements on reliability and low latency have been identified.

For instance, in the WI description for Physical layer enhancements for NR URLLC [2], higher requirements on latency and reliability are requested for use cases within factory automation, transport industry and electrical power distribution. As stated in the justification part of the WID, latencies as short as 0.5 to 1ms is needed for some of the new use cases.

Mobility performance such as latency and reliability need to be optimized in Rel-16, e.g. for URLLC type of services.

Examples of URLLC type of services and their QoS characteristics are listed in Table 5.7.4-1 in TS 23.501 [3] and can be seen in the table extract below.

It shall be noted that the latest version of the spec is 15.5.0, thus the table does not reflect the stricter latency requirements for the new Rel-16 URLLC use cases as indicated above.

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 82 | Delay Critical GBR | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 83 |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 84 |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms | Electricity Distribution- high voltage (see TS 22.261 [2]) |

Table 1: Extract from Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

As seen from the table above, most of the listed example services have very low packet delay budget (upper bound for the time that a packet may be delayed between the UE and the UPF) at the same time as the reliability requirements are high.

RLC UM is today typically used for delay-sensitive and error-tolerant applications like VoIP and other real-time services such as process automation and gaming. However, although the example services in the table above have very high reliability requirements, it is likely that these types of services will also be mapped to DRBs using RLC UM bearers due to their packet delay sensitivity.

RLC UM are today typically used for low latency and error-tolerant services but may also be used for low latency and high reliability type of services due to their packet delay sensitivity.

As of today, only RLC AM bearers may provide in-sequence delivery and duplication avoidance during an inter-node handover, i.e. PDCP SN is maintained from source gNB to target gNB only for RLC AM bearers. For RLC UM bearers the PDCP SN is reset in the target gNB and in the UE when transmission starts in the target cell, which means that packet loss and packet duplication is not guaranteed for services using this transmission mode.

Packet loss and packet duplication is not guaranteed for services mapped on DRBs using RLC UM.

Considering some of the low latency and high reliability type of services will be mapped on DRBs using RLC UM bearers, it seems reasonable that also this transmission mode provides shorter interruption time and lossless handover when Make-Before-Break (MBB) is configured.
It also seems likely that the delay-sensitive and error-tolerant services that today are mapped on DRBs using RLC UM also benefits from reduced handover interruption time when MBB is used.

It is therefore suggested that not only RLC AM, but also RLC UM, may provide in-sequence delivery and duplication avoidance (i.e. PDCP SN is maintained on a per DRB basis) during an inter-node handover in NR when MBB is configured.

RAN2 to consider in-sequence delivery and duplication avoidance also for RLC UM bearers during an inter-node handover when Make-Before-Break is configured.

# Conclusion

In section 2 we made the following observations:

Observation 1	Mobility performance such as latency and reliability need to be optimized in Rel-16, e.g. for URLLC type of services.

Observation 2	RLC UM are today typically used for low latency and error-tolerant services but may also be used for low latency and high reliability type of services due to their packet delay sensitivity.

Observation 3	Packet loss and packet duplication is not guaranteed for services mapped on DRBs using RLC UM.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to consider in-sequence delivery and duplication avoidance also for RLC UM bearers during an inter-node handover when Make-Before-Break is configured.

# References

RP-181433, New Work Item on NR Mobility enhancements, RAN#80

RP-190726, New Work Item on Physical layer enhancements for NR ultra-reliable and low latency communication (URLLC)

3GPP TS 23.501, System Architecture for the 5G System, v 15.5.0