3GPP TSG RAN WG2 #105bis	 R2-1903515

Xi’an, China, 8 – 12 April, 2019

Agenda Item:	11.9.3 Handover robustness improvements

Source: 	Ericsson

Title:  	User plane aspects of conditional handover in NR

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105 it has been agreed to support Conditional handover (CHO):

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

In this contribution some user plane aspects of conditional handover is discussed.

# Discussion

## Conditional Handover

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network. The advantage of the procedure is that the HO Command may be provided to the UE at an earlier stage before the radio conditions have become poor, which increases the chance of a successful transmission of the message. The basic signalling flow for conditional handover is shown in Figure 1.

Figure 1: Conditional handover

## User plane related actions

The difference between a legacy handover and a conditional handover is related to the procedure before the actual handover. When the handover is actually executed, the actions are the same as for a legacy handover. This applies also for the user plane, that the actions are the same for a legacy handover as for a conditional handover when it comes to the execution part of the handover. Thererfore, no new actions related to the user plane need to be specified for conditional handover.

Observation 1: The UE user plane related actions performed at execution of a legacy handover are the same as at execution of a conditional handover.

## Packet forwarding

Packet forwarding from source to target gNB can start at different points in time. The following options for when to start data forwarding at conditional handover have been discussed:

- After configuration of conditional handover

- After handover completion in target node

- After a new “bye” message/indication in source node

- After target indicates that the UE has accessed the cell

To start data forwarding after configuration of conditional handover means that a lot of data will be forwarded and also to potential target cells which the UE may never handover to. It would consume a lot of network resources, some of them of no use if the UE never connects to that target cell. For some URLLC cases the large amount of resources could be motivated if it is really important that no data is lost. Another drawback is that the source node will continue transferring packets to the UE, while forwarding packets to the target(s). It means that SN Status Transfer needs to be modified (i.e. no freeze) and/or sent later or several times, etc. However, if only one target cell is configured and the UE is expected to handover quickly, the network can choose to start data forwarding early.

To start data forwarding after handover completion (e.g. after reception of the end marker from UPF, after receiving UE Context Release) is on the other hand a bit late as the UE has already started receiving data in the new cell (i.e. after path switch). The two last options are means to enhance the starting time for data forwarding by providing information to source eNB that the UE is executing the handover and that it therefore is a good time to start data forwarding.

The first option is that the UE sends a “bye” message when leaving the source cell. The drawback with such a message is that it is unreliable as the radio conditions are usually quite bad when the UE executes the handover. If the UE starts transmitting the message and the message doesn’t get through, the UE may perform retransmissions instead of accessing the new cell. If the access to the new cell is delayed, the risk of a failed handover increases. Due to these reasons the option of sending a “bye” message is not a preferred solution.

An alternative could be that the target cell sends a message to source gNB when the UE has started accessing the cell. However, such a solution is more a RAN3 discussion. RAN2 could send an LS to RAN3 and ask them to evaluate a solution for data forwarding.

In summary, it is a network implementation decision when to start data forwarding and the exact point in time is not standardized. The starting point can depend on the specific use case and the network can make the decision based on the use case. Some improvement to provide source gNB with information about when the UE accesses the new cell can be considered in RAN3.

Observation 2: It is a network implementation decision when to start data forwarding and it depends on the specific use case.

Proposal 1: Send an LS to RAN3 and ask them to discuss possible enhancements of data forwarding for conditional handover (e.g. inform source eNB when the UE accesses target eNB).

# Summary

RAN2 is kindly asked to discuss the following observation and proposals:

Observation 1: The UE user plane related actions performed at execution of a legacy handover are the same as at execution of a conditional handover.

Observation 2: It is a network implementation decision when to start data forwarding and it depends on the specific use case.

Proposal 1: Send an LS to RAN3 and ask them to discuss possible enhancements of data forwarding for conditional handover (e.g. inform source eNB when the UE accesses target eNB).

# References

- RP-181544, Work Item on Even further Mobility Enhancements in E-UTRAN, China Telecom, RAN#81, September 2018