3GPP TSG RAN WG2 #105bis	 R2-1903520

Xi'an, China, 8 – 12 April, 2019

Agenda Item:	11.9.3 Handover robustness improvements

Source: 	Ericsson

Title:  	Conditional handover failures in NR

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105 it has been agreed to support Conditional handover (CHO):

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

2	We should consider how solutions work in FR2.

This contribution addresses the different failures cases that may occur in conditional handover.

# Discussion

## Conditional Handover

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network. The advantage of the procedure is that the HO Command may be provided to the UE at an earlier stage before the radio conditions have become poor, which increases the chance of a successful transmission of the message. The basic signalling flow for conditional handover is shown in Figure 1.

Figure 1: Conditional handover

CHO Configuration to the UE

In NR, the source receives the configuration prepared by target and provides to the UE in an RRCReconfiguration message containing a reconfigurationWithSync field with instructions for the UE to access the target cell e.g. RACH parameters, cell identity, frequency, SMTC configuration, etc. In CHO, similar information needs to be provided to the UE for a target candidate. In addition, the UE also needs the triggering condition associated to it e.g. an A3 event like configuration, so that the provided configuration is only applied if/when the condition is fulfilled.

In RAN2#105 in Athens, the content described above for the CHO has been agreed for LTE. In our view, there is no reason to deviate from that in NR:

Agreements

1: The baseline operation for E-UTRAN Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

. . .

CHO configuration consists of an RRC Reconfiguration (with reconfiguration With Sync) and associated trigger condition configuration (e.g. A3 event) for a target cell candidate.

UE is unable to comply with CHO configuration(s)

In NR, a reconfiguration failure is declared in case the UE is unable to comply with (part of) an RRC configuration. As CHO is configured with an RRC message (even though the exact message is still being discussed), the UE may be unable to comply with (part of) a CHO configuration, which may either be any triggering condition configuration and/or any dedicated RRC configuration (e.g. RRCReconfiguration with reconfigurationWithSync). This inability to comply with a CHO configuration is a first failure case related to CHO.

In our view, if the UE is unable to comply with CHO configuration, the UE declares a reconfiguration failure and continues using the configuration used prior to the reception of the CHO configuration.

If the UE is unable to comply with a received CHO configuration the UE declares CHO failure and continues using the configuration used prior to the reception of the CHO configuration.

The CHO configurations contain at least dedicated RRC configurations prepared by target, with content equivalent to an RRCReconfiguration including reconfigurationWithSync. In both NR and LTE, this is considered sensitive information which is only provided after security is activated. In that case, as in the case when the UE is unable to comply with an RRCReconfiguration, the UE should initiate connection re-establishment.

If the UE declares CHO failure (not able to comply with CHO configuration) the UE initiates re-establishment.

UE is unable to comply with a dedicated RRC configuration(s) upon CHO execution

In this second case, the UE can comply with the CHO configuration(s), or at least with the trigger condition configurations, that are immediately applied and lead to UE actions upon reception. And, upon receiving the HO triggering condition(s) and dedicated RRC configuration(s), the UE starts to perform the monitoring of the trigger conditions (which involves the UE performing measurements).

As multiple target cell candidates may be configured i.e. the UE may have stored one or multiple dedicated RRC configuration (e.g. multiple RRCReconfiguration(s) with reconfigurationWithSync(s)) and multiple trigger condition configuration(s). If one of these conditions are triggered, the UE performs the CHO execution which should be similar to a handover execution i.e. It consists of starting the timer T304 (or equivalent) and applying the stored RRCReconfiguration with reconfigurationWithSync message associated to the target cell candidate for which the trigger condition has been fulfilled.

When applying the message, which would be equivalent to having received at that moment an RRCReconfiguration with reconfigurationWithSync, the UE may be unable to comply with (part of) the configuration prepared by that target candidate.

Upon CHO execution, UE may be unable to comply with a dedicated configuration from target candidate.

Timer T304 expires upon CHO execution

In NR reconfiguration with sync, the RRCReconfiguration with reconfigurationWithSync contains a value for timer T304, that is started upon the reception of the message. Then, if the timer expires before the UE can access the target and send the RRCReconfigurationComplete according to target’s configuration, the UE declares a handover failure.

When the CHO trigger condition is fulfilled, and the UE executes CHO, the also UE starts an equivalent timer T304, configured in the stored reconfigurationWithSync associated to the selected target cell candidate. As in reconfiguration with sync procedure, before the UE can access the target candidate cell fulfilling the CHO trigger condition and being selected by the UE (in case multiple triggering cells), the timer T304 may expiry.

Upon CHO execution, timer T304 may expire before the UE can access the selected target cell candidate.

RLF declaration while CHO is being monitored

In NR, RLF is declared e.g. upon the timer T310 expires. That timer is started when the upper layers detect that the downlink quality is going below acceptable levels, which is controlled by indications provided by lower layers.

While the UE is monitoring CHO triggering conditions, as the network would likely configure the trigger conditions to be close to when the radio conditions of the PCell may not be so good any longer, RLF may be declared while the UE is monitoring CHO triggering conditions.

Timer T310 may expire while UE is monitoring CHO trigger conditions.

UE actions upon RLF declaration while CHO is being monitored, expiry of timer T304 in CHO or non-compliance with a dedicated RRC configuration(s) upon CHO execution

In NR, as in LTE, these different cases would lead respectively to a reconfiguration failure (in case the UE would not comply with a dedicated configuration issued by a target cell candidate), handover failure (in case the timer T304 expires after the UE tries to access a target cell candidate) and RLF declaration (in case timer T310 expires while the UE is monitoring CHO triggering conditions). And, upon declaration of any of these failures, the UE would first select a suitable cell to only then transmit an RRCReconfigurationRequest.

But then, in the case of CHO, if the UE selects a cell for which it has a stored RRCReconfiguration with reconfigurationWithSync, which is possible in the cases above, it would be much simpler, faster and efficient to apply the stored RRCReconfiguration with reconfigurationWithSync and complete the CHO compared to reverting the configuration and transmit an RRCReestablishment request.

Upon T304 expiry, T310 expiry or non-compliance of a dedicated RRC configuration of a target cell in CHO, the UE prioritizes cells it has CHO configured to. Details are FFS.

# Summary

RAN2 is kindly asked to discuss the following proposals:

- If the UE is unable to comply with a received CHO configuration the UE declares CHO failure and continues using the configuration used prior to the reception of the CHO configuration.

- If the UE declares CHO failure (not able to comply with CHO configuration) the UE initiates re-establishment if security is activated.

- Upon T304 expiry, T310 expiry or non-compliance of a dedicated RRC configuration of a target cell in CHO, the UE prioritizes cells it has CHO configured to. Details are FFS.

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1903517, Triggering of conditional handover, Ericsson

- R2-1903526, Configuration of conditional handover, Ericsson

- R2-1903518, Execution of conditonal handover, Ericsson