3GPP TSG RAN WG2 #105-bis	 R2-1903525

Xi'an, China, 8 – 12 April 2019

Agenda Item:	11.9.3 Handover robustness improvements

Source: 	Ericsson

Title:  	Conditional Handover Performance

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105 it has been agreed to support Conditional handover (CHO):

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

In this contribution we present some simulation results of CHO over baseline NR configuration to further motivate the selection of CHO as a solution for handover robustness improvements.

# Discussion

In RAN2#104 in Spokane it was agreed to support for LTE the configuration of one or more candidate cells for conditional handover:

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

In the simulation studies, we assume the above LTE agreements to be valid for NR, hence we consider multiple target cells as possible candidates for handover.

## Simulation Setup

The conditional handover as presented in [2] serves the basis for this simulation work. The call flow of Figure 1 was assumed during simulation.

Figure 1: Call flow for CHO

This procedure was compared to the baseline NR handover call flow as mentioned below

UE

Serving gNB

Target gNB

HO decision

UP data

1. Measurement report

2. HO Request

Accept HO, build RRC config

3. HO Ack (incl. RRC config)

4. HO command

UE accesses the new cell

5. Synchronization and random access

6. HO confirm

7. HO completed

UP  data

Figure 2: Call flow for Baseline handover

In conditional handover, the network configures the UE with triggering conditions for when a handover should be executed. UE is configured with possible target cell(s). Upon receiving the configuration, UE monitors for the conditions and performs handover when one of the configured conditions is successful, handover is initiated without sending notification to the source. Any event/combination of events can be used for conditional handover, however, in the simulation A3 event was used for both triggering conditional handover and handover execution. Simulations were performed for low, medium and high speed UEs. The threshold for triggering the conditional handover was set lower compared to triggering of legacy handover. The following setups were used:

| Simulation parameter | Parameter Settings |  |
| --- | --- | --- |
| Carrier Frequency | 2GHz, 20 MHz carrier BW, SCS:15KHz |  |
| Deployment | ISD=500 m, 7 sites, 3 sectors per site, Cell Radius=166.67 m |  |
| gNB antenna | Antenna 3gpp36873, 16 elements, 2 rows, 0.7ƛ separation, 80W |  |
| Propagation model | SCM – {5G-urban macro} |  |
| User properties | Number of users corresponding to ~80% DL resource utilization Speed 1 m/s, 80% indoor | Number of users corresponding to ~85% DL resource utilization Speed: 15 m/s, 30 m/s (0% indoor) |
| User session | FTP, mean file size 300000 bytes, ftp request size 400 bytes, mean reading time 1 s |  |
| NR-SS block settings | Periodicity 20 ms, 1 beam |  |
| Handover parameters | A3 event, RSRP threshold {1:5 dB}, Hysteresis {0 dB}, TTT{0.04} |  |
| CHO parameters | A3 early event, RSRP threshold {-1dB}, Hysteresis {0 dB}, TTT{0.04}. |  |
| T304 (handover) & T310 (out-of-sync) | 1 s |  |

- Conditional Handover issuing event threshold set to -1dB.

  - Controls when the conditional Handover command is issued to the UE.

  - Possible for network to add/modify target cell(s).

- Handover execution threshold varied from 1dB to 5dB.

  - Controls when the actual handover is initiated.

- Once a UE is configured with CHO configuration, the resources are reserved both at network and UE side until UE performs handover

## Simulation results

As mentioned in Section 2.1, simulations were performed for different UE speeds. Below handover parameters were compared to evaluate the performance of CHO over the baseline handover:

- Average number of handover attempts/UE

- Average number of successful handovers/UE

- Average number of failed handovers/UE

We also evaluate ping-pong handovers where ping-pong handovers are defined as handover to source cell within 1 second of performing first handover, i.e. A->B->A type of handover within 1 second.

Figure 3: Average number of Handover attempts

As can be seen from the Figure 3, the average number of handover attempts are similar between CHO and baseline procedure for respective UE speeds. This is an important observation since the target cells are selected earlier for CHO than baseline handover. Furthermore, it is understood that the number of handover attempts increases with the increase of the speed of the UE and decreases with increasing the execution threshold.

Figure 4: Average number of successful handovers

Comparison of ratio of successful handovers in Figure 4 shows CHO performs better than baseline handover procedure. Such observation leads to an expectation of better HOF performance as shown in Figure 5.

Figure 5: Average number of HOF

From Figure 5, it is evident that CHO reduces handover failures for all UE speeds. However, the best performance was observed for high speed UEs, where the baseline procedure results in the highest number of handover failures. If we consider 3 dB as threshold, 40% reduction in handover failure can be observed for a UE speed of 30m/s.

For completeness of the handover performance comparison, evaluation of ping pong handover performance showed no significant degradation of performance using CHO.

## Conclusion

Based on the above performance comparison, it can be concluded that conditional handover increases handover robustness in challenging scenarios while keeping similar performance as baseline in normal scenarios.

CHO provides handover robustness in NR.

Hence it is proposed that conditional handover procedures should be standardized in NR. The details of CHO procedures and proposals can be found in other companion contributions [2]-[9].

Consider CHO as a handover robustness solution for NR

# Summary

RAN2 is kindly asked to discuss the following proposals:

- Proposal 1	 Consider CHO as a handover robustness solution for NR

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1903515, Configuration of Conditional handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903518, Conditional Handover execution in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903520, Conditional Handover failure in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903517, Triggering of Conditional Handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903521, Suspend while monitoring Conditional handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903522, Security implication of Conditional handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903523, On FR2 impact on CHO in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019