3GPP TSG RAN WG2 #105-bis	 R2-1903526

Xi'an, China, 8 – 12April 2019

Agenda Item:	11.9.3 Handover robustness improvements

Source: 	Ericsson

Title:  	Validity Timer impact on Conditional Handover Performance

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105 it has been agreed to support Conditional handover (CHO):

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

In our companion contribution [2] we compared performance between baseline NR and Conditional handover. One key assumption in that contribution was CHO configurations are stored in UE until it performs handover to a target cell. In this contribution, we present simulation results evaluating the impact of validity timer on CHO performance.

# Discussion

In our companion contribution [3] different methods of de-configuring CHO configuration in a UE. Some key observations are listed here

It is difficult for the network to estimate good values of validity timers.

Validity timers cause a lot of network signalling

Validity timers are likely to cause more signalling towards the UE than if only explicit deconfiguration is used.

In these simulation work, we would evaluate these observations and discuss the results

## Simulation Setup

The conditional handover as presented in [4] serves the basis for this simulation work. The call flow of Figure 1 was assumed during simulation.

Figure 1: Call flow for CHO

This procedure was compared to the baseline NR handover call flow as mentioned below

UE

Serving gNB

Target gNB

HO decision

UP data

1. Measurement report

2. HO Request

Accept HO, build RRC config

3. HO Ack (incl. RRC config)

4. HO command

UE accesses the new cell

5. Synchronization and random access

6. HO confirm

7. HO completed

UP  data

Figure 2: Call flow for Baseline handover

In conditional handover, the network configures the UE with triggering conditions for when a handover should be executed. UE is configured with possible target cell(s). Upon receiving the configuration, UE monitors for the conditions and performs handover when one of the configured conditions is fulfilled. Handover is initiated without sending notification to the source. Any event/combination of events can be used for conditional handover. However, in the simulation A3 event was used for both triggering conditional handover and handover execution. Simulations were performed for low, medium and high speed UEs. The threshold for triggering the conditional handover was set lower compared to triggering of legacy handover. The following setups were used:

| Simulation parameter | Parameter Settings |
| --- | --- |
| Carrier Frequency | 2GHz, 20 MHz carrier BW, SCS:15KHz |
| Deployment | ISD=500 m, 7 sites, 3 sectors per site, Cell Radius=166.67 m |
| gNB antenna | Antenna 3gpp36873, 16 elements, 2 rows, 0.7ƛ separation, 80W |
| Propagation model | SCM – {5G-urban macro} |
| User properties | Number of users corresponding to ~85% DL resource utilization. Speed: 15 m/s (0% indoor) |
| User session | FTP, mean file size 300000 bytes, ftp request size 400 bytes, mean reading time 1 s |
| NR-SS block settings | Periodicity 20 ms, 1 beam |
| Handover parameters | A3 event, RSRP threshold {1:5 dB}, Hysteresis {0 dB}, TTT{0.04} |
| CHO parameters | A3 early event, RSRP threshold {-1dB}, Hysteresis {0 dB}, TTT{0.04}. |
| T304 (handover) & T310 (out-of-sync) | 1 s |

- Conditional Handover issuing event threshold set to -1dB.

  - Controls when the conditional Handover command is issued to the UE.

  - Possible for network to add/modify target cell(s).

- Handover execution threshold evaluated on 1 dB, 3 dB and 5dB

  - Controls when the actual handover is initiated.

- Once a UE is configured with CHO configuration, the resources are reserved both at network and UE side according to validity timers

  - Validity timers are iterated over 2sec to 10sec with a granularity of 2sec.

## Simulation results

As mentioned in Section 2.1, simulations were performed for UE speed of 15m/s. Below handover parameters were compared to evaluate the performance of CHO for different validity timers:

- Average number of handover attempts/UE

- Average number of successful handovers/UE

We also evaluate ping-pong handovers where ping-pong handovers are defined as handover to source cell within 1 second of performing first handover, i.e. A->B->A type of handover within 1 second.

Figure 3: Average number of Handover attempts

Figure 4: Average number of successful handovers

Figure 5: Average number of Ping-Pong handovers

Figure 3, Figure 4,and Figure 5 shows no discernible impact of CHO on average number of handovers attempts, successful handovers and ping-pong handovers, respectively per UE.

Simulation results shows no discernible impacts of validity timers on handover performance.

To evaluate the impact of validity timer we study the signalling load between network and UE. Furthermore, we make below assumptions:

- Message containing conditional configuration for a new cell is regarded as required and not counted as overhead.

- Message containing conditional configuration for a cell that was already configured but resent needed because of validity timer expiration is considered as overhead.

- Since one message can potentially carry configuration of multiple cells

  - If the message carries configuration for a new cell along with configurations of cells mentioned in 2), that is not regarded as overhead.

  - If the message only carries configuration for cells mentioned in 2), that is regarded as overhead.

The assumptions, in our view, regards only signalling as overhead that could be avoided by not configuring validity timer. We also note that re-configuring old cells with a new cell in same message increases message size and hence affects overhead (which needs to be considered also), but we ignore that since the signalling is anyway needed to be transmitted.

Figure 6 presents one plot of the evaluation mentioned for a handover execution threshold of 5dB.

Figure 6: Overhead introduced by validity timer

The configuration message, as mentioned before is the part that cannot be avoided by not using timer (however other optimizations can be utilized) and the Reconfiguration message can be avoided by removing validity timer. From the figure, it can be found that around 18% signalling reduction can be achieved if validity timer is set to 2sec. Also, a validity timer of 10sec fails to achieve 100% overhead while adding additional complexities in the network as mentioned in [3].

If the network is overloaded and needs to release the resources there will be extra signalling to deconfigure conditional handover, but that signalling is basically applicable both with and without a validity timer as it will only be in rare cases that the timer expires in the right moment when the network needs the resources.

The figure above also does not take network signalling into account, which is the largest source of increased signalling related to validity timer.

## Conclusion

Based on the above performance comparison, it can be concluded that validity timer in conditional handover increases signalling overhead in the air interface and adds complexity in network.

Hence it is proposed to consider this simulation results and discussions in [3] for setting validity timer in conditional handover

# Summary

RAN2 is kindly asked to discuss the results in this contribution and proposals in [3].

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1903525, Conditional Handover Performance, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903519, Deconfiguration of conditional handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019

- R2-1903516, Configuration of Conditional handover in NR, Ericsson, RAN2_105bis, Xi’An, China, 8-12 April 2019