3GPP TSG-RAN WG2 Meeting #105bis											R2-1903599

Xi'an, China, 8th April - 12th April 2019

Source: 	vivo

Title:  	Scheduling enhancements for periodic deterministic traffic

Agenda Item:	11.7.2.2

Document for:	Discussion and Decision

Introduction

According to the RAN2#105 meeting discussion, the “TSC traffic pattern related information include message periodicity, message size and message arrival time”. In this contribution, we discuss the potential scheduling impacts considering the traffic patterns given by 802.1 QCC and the agreement made by RAN2.

Discussion

Mapping TSN traffic to semi-persistent resource

Figure 1: Traffic characteristics of a TSN traffic flow

The above Figure 1 describing the traffic characteristics of a TSN traffic flow is extracted from the TSN specification [1].

Figure 2: Mapping between the TSN traffic and the semi-static resource

An example of how to map the traffic model to the semi-persistent resource allocation is illustrated in Figure 2. The corresponding parameters for the semi- persistent resource allocation is given the Table 1.

Table 1：Parameter mapping table between SPS/configured grant and the TSN traffic

| TSN traffic specification | UL configured grant Type 2 | DL SPS |
| --- | --- | --- |
| Interval | Periodicity configured by RRC | periodicity configured by RRC |
| Transmission duration = [EarliestTransmitOffset, LatestTransmitOffset] | resourceAllocationPeriod configured by RRC | resourceAllocationPeriod configured by RRC |
| EarliestTransmitOffset | SFNstart time, slotstart time, and symbolstart time indicated by DCI | SFNstart time and slotstart time indicated by DCI |
| MaxFrameSize | Uplink grant indicated by DCI | DL assignment indicated by DCI |
| MaxFramesPerInterval |  | numberOfSlotUsed configured by RRC |

According to the analysis give above, the resource allocation period is used to cover the transmission period within each interval, as the message arrival time could be within a period between EarliestTransmitOffset and LatestTransmitOffset.

Proposal 1: The resource allocation duration per period is configured for configured grant and SPS.

Regarding the “MaxFramesPerInterval”, for the UL configured grant, if the UE has no data for transmission, then the UE would skip the uplink grant according to the current specification. However, for the DL SPS, as there is no downlink skipping according to the current specification, the UE would keep receiving the PDSCH of the DL SPS during the whole resource allocation duration. To save the UE power, we consider that the number of slot used within the resource allocation duration should be configured to allow the UE to skip the subsequent SPS resources if all data frames are received by the UE. According to the discussion during the RAN2#105 meeting, some companies consider that there is only one message per each transmission interval. Here we consider that if companies consider that we only support the traffic pattern with one single message transmission per interval in Rel-16, the number of packet per interval to the UE does not need to be configured to the UE but rather fixed in the specification.

Proposal 2: The number of packets/slots used within the resource allocation duration should be known by the UE to skip the subsequent SPS resources if all packets/ slots (transmitting the UE-specific PDSCH) are received by the UE.

Conclusions

Based on the analysis given above, we have the following Proposals：

Proposal 1: The resource allocation duration per period is configured for configured grant and SPS.

Proposal 2: The number of packets/slots used within the resource allocation duration should be known by the UE to skip the subsequent SPS resources if all packets/ slots (transmitting the UE-specific PDSCH) are received by the UE.

Reference

[1] [104#36][NRIIOT] TSN Traffic Patterns (Nokia)

[2] IEEE P802.1 QCC, May 3, 2018