3GPP TSG-RAN WG2 Meeting #105bis	R2-1903653

# Xi’an, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | NNN | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Reconfiguration via SRB3 in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 04/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 38.331 does not clearly say, which fields can be present in RRCReconfiguration received via SRB3 in EN-DC. For SRB1 case (i.e. RRCReconfiguration received via LTE RRCConnectionReconfiguration), it is said for nr-SecondaryCellGroupConfig in 36.331 that: "In this version of the specification, the NR RRC message only includes fields secondaryCellGroup and/ or measConfig". In the 1st release of EN-DC in 38.331, it is clear that only the radioBearerConfig, secondaryCellGroup, and/ or measConfig could be sent via SRB3 because there is no other extension at that time. RRCReconfiguration-IEs ::= SEQUENCE { radioBearerConfig RadioBearerConfig OPTIONAL, -- Need M secondaryCellGroup OCTET STRING (CONTAINING CellGroupConfig) OPTIONAL, -- Need M measConfig MeasConfig OPTIONAL, -- Need M lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RRCReconfiguration-v1530-IEs OPTIONAL } However, with further extensions added in RRCReconfiguration due to NR SA, it is not clear whether the later IEs could be sent via SRB3 in EN-DC. We think that the new IEs such as masterCellGroup, fullConfig, dedicatedNAS-MessageList, otherConfig are not designed for EN-DC. Thus, the network should not send this IEs via SRB3 and the UE shall not handle this IEs in EN-DC. Therefore, we propose to clarify that UE only handles the measConfig, radioBearerConfig and/or secondaryCellGroup for RRCReconfiguration received via SRB3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.1.3, specify that UE only handles the fields measConfig, radioBearerConfig and/or secondaryCellGroup for RRCReconfiguration received via SRB3. Impact analysis Impacted 5G architecture options: EN-DC Impacted functionality: RRC Reconfiguration procedure received via SRB3 in EN-DC Inter-operability: The CR only impact UE behavior and the network does not have to implement it. If the UE implements the CR but the network does not, there is no Inter-operability issue. The IEs other than measConfig, radioBearerConfig and secondaryCellGroup are not designed for EN-DC. So, ignoring this IE does not cause problem to EN-DC operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is unclear if the configuration other than measConfig, radioBearerConfig and secondaryCellGroup are received via SRB3 in EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on draft 38.331-f50 |  |  |  |  |  |  |  |  |

### 5.1.3	Requirements for UE in EN-DC

In this specification, the UE considers itself to be in EN-DC if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10].

NOTE: 	This use of the term EN-DC deviates from the definition in TS 37.340 [43] and other specifications. In TS 37.340, the term EN-DC includes also the case where the UE is configured with E-UTRA MCG only (i.e. no NR SCG) but with one or more bearers terminated in a secondary node (i.e. using NR PDCP).

The UE in EN-DC only executes a sub-clause of clause 5 in this specification when the subclause:

-	is referred to from a subclause under execution, either in this specification or in TS 36.331 [10]; or

-	applies to a message received on SRB3 (if SRB3 is established); or

-	applies to field(s), IE(s), UE variable(s) or timer(s) in this specification that the UE is configured with.

When executing a subclause of clause 5 in this specification, the UE follows the requirements in clause 5.1.2 and in all subclauses of this specification applicable to the messages (including processing time requirements), fields, IEs, timers and UE variables indicated in the subclause under execution. In this version of the specification, the UE is required to handle only fields measConfig, radioBearerConfig and/or secondaryCellGroup for RRCReconfiguration received via SRB3.