3GPP TSG-RAN WG2 Meeting #105bis	R2-1903672

# Xi’an, China, 8th  – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0874 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE configuration for RRC Resume procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 04/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The restoring of UE configuration is twice in 5.3.13.3 (as following) 1> restore the RRC configuration and AS security context from the stored UE Inactive AS context except the masterCellGroup; ... 1> restore the RRC configuration and the KgNB and KRRCint keys from the UE Inactive AS context except the cellGroupConfig and pdcp-Config; The first restoring clause should be deleted as it does not handle the exception of pdcp-config. RAN2 agree to use default SRB1 configuration for RRCResume reception. Default SRB1 is used to prepare for case where target gNB does not comprehend UE Inactive AS context. The current text is unclear that whether the UE should use default PDCP configuration for SRB1 or use the configuration from UE inactive AS context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.3.13.3, delete the 1st clause about restore UE configuration. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: RRC Resume procedure Inter-operability: If the UE implements the CR but the network does not or vice-versa, there is inter-operability issue. The UE and network may have different understanding on PDCP configuration of SRB1 during RRC resume procedure. This may result in resume procedure failed. The consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that whether the UE should use default PDCP configuration or use the configuration from UE inactive AS context for SRB1 during resume procedure. This may result in resume procedure failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on draft 38.331-f50 |  |  |  |  |  |  |  |  |

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key in the UE Inactive AS Context and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	restore the RRC configuration and the KgNB and KRRCint keys from the UE Inactive AS context except the masterCellGroup and pdcp-Config;

1>	derive the KgNB key based on the current KgNB key or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

1>	configure lower layers to apply integrity protection for all radio bearers except SRB0 using the configured algorithm and the KRRCint key and KUPint key derived in this subclause immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to apply ciphering for all radio bearers except SRB0 and to apply the configured ciphering algorithm, the KRRCenc key and the KUPenc key derived in this subclause, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	re-establish PDCP entities for SRB1;

1>	resume SRB1;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running, perform actions specified in 5.3.13.5.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation.