3GPP TSG-RAN WG2 Meeting #105bis	R2-1903769

# Xian, China, 08 – 12 April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0765 | rev | - | Current version: | DRAFT 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to mobility state handling in case of LTE to NR reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Give the reasons for change: - In case of high mobility state is detected UE applies medium mobility state scaling factor. Instead high mobility state scaling factor should be applied |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the changes: - Change UE to apply sf-High instead of sf-Medium in case UE is in the high mobility state Impact analysis Impacted functionality: Mobility state and scaling. Inter-operability: - If the network is implemented according to the CR and the UE is not not UE would apply different scaling factor as assumed by network which could lead to unnecessary reselections - If the UE is implemented according to the CR and the network is not UE would apply different scaling factor as assumed by network which could lead to unnecessary reselections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE applies wrong scaling factor in the high mobility state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

##### 5.2.4.3.1	Scaling rules

UE shall apply the following scaling rules:

-	If neither Medium- nor Highmobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst" to Qhyst if sent on system information

-	For E-UTRAN cells multiply TreselectionEUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionEUTRA" if sent on system information

-	For UTRAN cells multiply TreselectionUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionUTRA" if sent on system information

-	For GERAN cells multiply TreselectionGERA by the sf-High of "Speed dependent ScalingFactor for TreselectionGERA state" if sent on system information

-	For CDMA2000 HRPD cells Multiply TreselectionCDMA_HRPD by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_HRPD" if sent on system information

-	For CDMA2000 1xRTT cells Multiply TreselectionCDMA_1xRTT by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_1xRTT" if sent on system information

-	For NR cells multiply TreselectionNR by the sf-High of "Speed dependent ScalingFactor for TreselectionNR" if sent on system information

-	If Medium-mobility state is detected:

-	Add the sf-Medium of "Speed dependent ScalingFactor for Qhyst" to Qhyst if sent on system information

-	For E-UTRAN cells multiply TreselectionEUTRA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionEUTRA" if sent on system information

-	For UTRAN cells multiply TreselectionUTRA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionUTRA" if sent on system information

-	For GERAN cells multiply TreselectionGERA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionGERA" if sent on system information

-	For CDMA2000 HRPD cells Multiply TreselectionCDMA_HRPD by the sf-Medium of "Speed dependent ScalingFactor for TreselectionCDMA_HRPD" if sent on system information

-	For CDMA2000 1xRTT cells Multiply TreselectionCDMA_1xRTT by the sf-Medium of "Speed dependent ScalingFactor for TreselectionCDMA_1xRTT" if sent on system information

-	For NR cells multiply TreselectionNR by the sf-Medium of "Speed dependent ScalingFactor for TreselectionNR" if sent on system information

In case scaling is applied to any TreselectionRAT parameter the UE shall round up the result after all scalings to the nearest second.

Next Modified Subclause