3GPP TSG-RAN WG2 Meeting #105bis	R2-1903822

Xi’an, China, 8th – 12th April 2019	Revision of R2-1900456

Title:	DRAFT LS on the ETWS/CMAS in connected mode

Response to:

Release:	Release 16

Work Item:	LTE_eMTC5

Source:	Huawei, HiSilicon [TSG RAN WG2]

To:	TSG-RAN WG1

Cc:

Contact Person:

Name:	Yulong Shi

E-mail Address:	shiyulong5@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

To support ETWS/CMAS in connected mode for non-BL UE in CE mode, RAN2 considers monitoring P-RNTI or C-RNTI for the notification of ETWS/CMAS and delivering the content of ETWS/CMAS by legacy broadcasted SI.

First, RAN2 would like to kindly ask RAN1 to confirm whether it is feasible to support monitoring P-RNTI in connected mode for Rel-16 non-BL UEs in CE mode, so that UE receives the ETWS/CMAS notification via paging.

Second, RAN2 would like to kindly ask RAN1 to confirm whether a new DCI format needs to be introduced if the DCI addressed by C-RNTI includes ETWS/CMAS notification and possible resource scheduling.

At last, RAN2 would like to kindly ask RAN1 to confirm whether a non-BL UE in CE mode can receive SI on the SI narrow band and receive data on its dedicated band simultaneously.

2. Actions:

To RAN1:

RAN2 respectfully asks RAN1 to confirm:

1) Whether it is feasible to support monitoring CSS in connected mode for Rel-16 non-BL UEs in CE mode.

2) Whether a new DCI format needs to be introduced if the DCI addressed by C-RNTI includes ETWS/CMAS notification and possible resource scheduling.

3) Whether a non-BL UE in CE mode can receive SI on the SI narrow band and receive data on its dedicated band simultaneously.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#106	13 - 17 May 2019   	Reno       	US

3GPPRAN2#107	26 - 30 August 2019   	Prague   	Czechoslovakia