3GPP TSG-RAN WG2 #105bis	Tdoc R2-1903832

Xi’An, China, 8 – 12 April 2019	Revision of R2-1900736

Agenda Item:	12.2.4

Source:	Ericsson

Title:	PUR Configuration

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 Work Item Descriptions for LTE-M [1] and NB-IoT [1] contain a common objective on improving the uplink transmission efficiency and/or UE power consumption by means of transmission in preconfigured resources:

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

  - Note: This is limited to orthogonal (multi) access schemes

In RAN1 the following agreements have been reached for LTE-M and NB-IoT which are relevant for this contribution (‘LTE-M’ and ‘NB-IoT’ in brackets when not applicable to both):

Agreement Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE - PUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS. Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported. Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement In idle mode, the TA validation configuration can include “PUR Time Alignment Timer” - Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer - Details on how to specify the “PUR Time Alignment Timer” is up to RAN2 Agreement For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration. - PUR search space is the search space where UE monitors for (M/N)PDCCH - FFS: Whether PUR search space is common or UE specific Agreement When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures - FFS whether only TA is acquired and then data sent on PUR is supported - FFS other approaches to obtain a valid TA Agreement For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern Agreement In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission: - Timing advance adjustment - UE TX power adjustment - FFS: Repetition adjustment for (N)PUSCH FFS: Whether the above update is done in L1 and/or higher layer Agreement [LTE-M] For dedicated PUR in idle mode, the PUR resource configuration includes at least the following - A PUSCH frequency hopping indication to enable or disable legacy frequency hopping Agreement [LTE-M] In idle mode, a UE can be configured such that TA is always valid within a given cell. - FFS: up to RAN2 how to implement e.g. PUR Time Alignment Timer = infinity

In RAN2 the following agreements have been reached which are relevant for PUR configuration:

Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Methods for eNB to obtain information used to help configuring the dedicated preconfigured uplink resource to the UE is FFS. Multi-shot D-PUR is supported with the possibility to configure as a single shot. UE may perform a D-PUR request/information, if D-PUR is indicated as enabled in the cell. Network makes the decision on the D-PUR configuration. Request/information can include: - Requested TBS - Requested periodicity - Other information FFS. The eNB can (re)configure and release D-PUR by dedicated RRC signalling D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions The UE must release the D-PUR when it does a RA procedure on a new cell. D-PUR configuration can be set up without a pre-defined end (infinite).

In this contribution we continue to discuss some issues in addition to the PUR email discussion 104#43.

# 2	Discussion

In the following, dedicated PUR in Idle mode is discussed unless mentioned otherwise.

# 2.1	Data transmission

RAN1 has now defined the PUR data transmission as a PUSCH resource for both dedicated PUR, CFS PUR, and CBS PUR. To avoid a lot of work in RAN2, the PUR data transmission could reuse a lot from Rel-15 EDT. That is, the PUR data transmission could be similar to the EDT data transmission in Msg3. RAN2 should also consider using the EDT Msg3 and Msg4 RRC messages as a baseline for new PUR RRC messages.

Rel-15 EDT Msg3/Msg4 transmission is used as baseline for the PUR messages.

In RAN2#103bis it was agreed at least initially to focus on dedicated PUR in Idle mode. The UE would in this case be provided with a PUR configuration and TA during a connection via dedicated RRC signaling (see Figure 1). The PUR resource can be configured tailor-made to the UE, e.g. with respect to its traffic profile, capabilities, CE-level, etc.

Figure 1: Dedicated PUR with periodic resources.

## 2.1.1	PUR retransmission

Since the PUR periodicity can be in the order of hours, it is not feasible to use the subsequent PUR occasion for retransmission in the case a PUR transmission fails. That would further mean that the subsequent PUR occasion could not be used as intended and with every retransmission a delay as long as the PUR periodicity would be added and the overall delay would keep becoming longer and longer and accumulate with every retransmission. That is not a good solution and it is therefore proposed that dynamic HARQ retransmissions are used for PUR in the same way as for EDT. This is also in line with RAN1’s agreements on the topic:

-In IDLE mode, HARQ is supported for transmission in dedicated PUR.

-For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in (MPDCCH) search space

Dynamically scheduled adaptive HARQ re-transmission is used for PUR (i.e. the subsequent PUR occasion is not used for re-transmission).

## 2.1.3	Transport block size

For the TBS, the assumption is that only one PUR resource is configured per PUR period. That is, similar to EDT, it is focusing on small data transmission and the max TBS is limited to e.g. 1000 bits. A difference is however that for dedicated PUR configuration the UE capability is known, therefore a TBS up to the max TBS supported by the UE category could be used.

The PUR TBS is limited to the max TBS supported by the UE category.

## 2.1.4	Additional data

Occasionally the UE might get unexpected data to transmit and it could be of interest to specify a generic solution which could handle also this case. I.e., in the (infrequent) case the UE UL buffer exceeds the configured PUR TBS it could be considered to allow the UE to include a BSR in the PUR transmission. Without any need for RA, eNB could then move the UE to RRC_CONNECTED, in a similar way as for EDT, for subsequent transmission of the remaining data. Such EDT-like mechanism of being able to move the UE to Connected would also be of interest for when the network has pending DL data (i.e. more than can fit in PUR “Msg2”).

Inclusion of BSR in the PUR transmission is supported.

Moving the UE to RRC_CONNECTED after PUR transmission is supported.

# 2.2	Configuration information for PUR

So far it has been agreed that the NW will configure the UE with PUR and communicate the configuration over dedicated RRC signaling. It has been agreed that some information can be provided from the UE to guide the NW decision to configure PUR.

UE may perform a D-PUR request/information, if D-PUR is indicated as enabled in the cell.

Network makes the decision on the D-PUR configuration.

Request/information can include:

    - Requested TBS

    - Requested periodicity

    - Other information FFS.

Traffic prediction is the weak spot of PUR, and eNB must be able to trust the information to be accurate if it is to allocate radio resources for a long time ahead to a UE in RRC_IDLE mode. In addition to UE information there are also two more possible sources of information:

- UE information: I.e. the periodicity and TBS suitable for PUR according to the UE.

- Subscription information: E.g. the ‘Subscription UE Differentiation Information’ introduced in Rel-15.

- eNB information: Any traffic statistics stored by eNB itself.

Since the UE is the origin of the uplink transmission one could argue that UE information would be the best information source. Unfortunately, the radio protocol layers may know little or nothing about the application layer, and an application layer would typically be agnostic of the radio access technology. To our knowledge there is no application assistance information available which could be used for the purpose, and NB-IoT further does not support QoS or QCI.

Figure 2: Illustration of PUR information for eNB configuration of PUR.

Related to option 2 above, support for ‘Subscription Based UE Differentiation Information’ was introduced in Rel-15. The ‘Subscription Based UE differentiation Information’ is generated by the MME and contains a number of fields for the traffic profile according to RP-182115 (quoted in Appendix 2 for convenience). This information, if it exists, can therefore be made available to the eNB for the configuration of PUR already now.

Rel-15 ‘Subscription Based UE Differentiation Information’ can already be used by eNB for configuration of PUR if available (i.e. implemented and used in both CN and eNB).

From RP-182115 it is also seen that there is currently no TBS field specified, and it could be beneficial to add TBS information for use with PUR.

TBS info would be of interest for PUR configuration but is not part of Rel-15 ‘Subscription Based UE Differentiation Information’.

Related to option 3 above, the eNB could store traffic history of UEs and determine if PUR would be suitable for certain UEs. That is, operators careful of potential misuse of PUR may want to base any PUR configuration on the traffic patterns of UEs. For UEs operating with the same periodic traffic for several years, i.e. the intended use case for PUR, a few initial data transmissions using legacy procedure will have a negligible impact on e.g. the device battery life.

After having decided to configure PUR for a UE, the PUR configuration must be kept in the eNB since the scheduler must be aware of upcoming PUR transmissions. For the UP-solution, this information could be stored with the UE AS context.  For the CP-solution since there is no AS UE context, but we don’t see any other solution that one would have to be introduced if PUR is to be supported for the CP-solution. (It could be called something else, but it does not change the fact that it is radio related parameters for the UE which needs to be kept in eNB).

The PUR configuration must be stored in UE AS context in eNB.

UE AS context must be introduced for the CP-solution.

# 2.3	Joint configuration of periodic and one-shot PUR

The configuration of both one-shot and periodic PUR can be done jointly. This is illustrated below with the same nomenclature as in the email discussion where a parameter n indicates for how many instances the PUR configuration is valid. n=1 refers to one-shot PUR and n=∞ refers to infinite periodic PUR.

Figure 3: Illustration of joint configuration for one-shot and periodic PUR. A) periodic PUR, B) one-shot PUR, C) both one-shot and periodic PUR.

In this example, the UE is configured with infinite PUR if n=∞ is configured and this configuration is maintained for the subsequent PUR occasions (Figure 3A). If the UE is configured with n=1 it means one-shot PUR, a configuration is then required each time but the PUR periodicity and TBS can change from time to time (Figure 3B). If needed, it is also straight forward to change the configuration between periodic PUR and one-shot PUR by reconfiguring the UE with a new n-value (Figure 3C).

The drawback for one-shot PUR is the slight increase in signaling overhead from having to provide a configuration for each instance. However, if the interval and TBS remain unchanged, a flag could indicate this or only the parameters to be changed could explicitly be included.

In PUR re-configuration, to reduce the signaling overhead, only changed PUR parameters are explicitly included and unchanged parameters are assumed to be the same.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Rel-15 ‘Subscription Based UE Differentiation Information’ can already be used by eNB for configuration of PUR if available (i.e. implemented and used in both CN and eNB).

Observation 2	TBS info would be of interest for PUR configuration but is not part of Rel-15 ‘Subscription Based UE Differentiation Information’.

Observation 3	The PUR configuration must be stored in UE AS context in eNB.

Observation 4	UE AS context must be introduced for the CP-solution.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Rel-15 EDT Msg3/Msg4 transmission is used as baseline for the PUR messages.

Proposal 2	Dynamically scheduled adaptive HARQ re-transmission is used for PUR (i.e. the subsequent PUR occasion is not used for re-transmission).

Proposal 3	The PUR TBS is limited to the max TBS supported by the UE category.

Proposal 4	Inclusion of BSR in the PUR transmission is supported.

Proposal 5	Moving the UE to RRC_CONNECTED after PUR transmission is supported.

Proposal 6	In PUR re-configuration, to reduce the signaling overhead, only changed PUR parameters are explicitly included and unchanged parameters are assumed to be the same.

# References

RP-190532, “Revised WID for Additional MTC enhancements for LTE”, Ericsson, RAN#83, Shenzhen, China, March 2019.

RP-190337, “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#83, Shenzhen, China, March 2019.

RAN1 Chairman’s Notes, RAN1#94bis, Chengdu, China, October 8th – 12th, 2018.

R2-1818633, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, Blackberry, RAN2#104, Spokane, USA, November 2018.

# Appendix

# Appendix 2

RAN#81 endorsed the following way-forward (RP-182104):

- Local RRM Policy Information storage and retrieval is a generic functionality not restricted to NB IoT only - WF - Explicitly capture the Local RRM Policy Information storage and retrieval as objective of the SID on RAN-centric data collection and utilization for LTE and NR - Note: this includes studying the Local RRM Policy Information storage and retrieval per UE for LTE and NR (RP-182105) - SON TU should be extended - Approved the CRs related to Subscription based UE: - RP-182115 Introduction of Subscription based UE differentiation - RP-182116 Introduction of Subscription based UE differentiation - Completed the NB IoT WI at RAN#81

The finally approved RAN3 CRs can be found in RP-182115 and RP-182127. The IEs stored for ‘Subscription Based UE differentiation Information’ is given in the former document and contain the following:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodic Communication Indicator | O |  | ENUMERATED(periodically, on demand, …) | This IE indicates whether the UE communicates periodically or not, e.g. only on demand. |
| Periodic Time | O |  | INTEGER (1..3600, …) | This IE indicates the interval time of periodic communication, the unit is: second |
| Scheduled Communication Time |  | 0..1 |  | This IE indicates the time zone and day of the week when the UE is available for communication. |
| >>Day of Week | O |  | BIT STRING (SIZE(7)) | If Day-Of-Week is not provided this shall be interpreted as every day of the week. Each position in the bitmap represents a day of the week: first bit = Mon, second bit =Tue, third bit =Wed, and so on. Value ‘1’ indicates ‘scheduled. Value ‘0’ indicates ‘not scheduled’. |
| >>Time of Day Start | O |  | INTEGER (0..86399, …) | This IE indicates the time to start of the day, each value represent the corresponding second since 00:00 of the day. If Time-Of-Day-Start is not provided, starting time shall be set to start of the day(s) indicated by Day-Of-Week-Mask. |
| >>Time of Day End | O |  | INTEGER (0..86399, …) | This IE indicates the time to start of the day, each value represent the corresponding second since 00:00 of the day. The value of this IE should be bigger than the value of Time of Day Start IE. If Time-Of-Day-End is not provided, ending time is end of the day(s) indicated by Day-Of-Week-Mask. |
| Stationary Indication | O |  | ENUMERATED(stationary, mobile, …) |  |
| Traffic Profile | O |  | ENUMERATED(single packet, dual packets, multiple packets, …) | “single packet” indicates single packet transmission (UL or DL), “dual packets” indicates dual packet transmission (UL with subsequent DL, or DL with subsequent UL), “multiple packets” indicates multiple packets transmission. |
| Battery Indication | O |  | ENUMERATED(battery powered, battery powered not rechargeable or replaceable, not battery powered, …) | “battery powered” indicates that the UE is battery powered and the battery is rechargeable/replaceable, “battery powered not rechargeable or replaceable” indicates that the UE is battery powered but the battery is not rechargeable/replaceable,, “not battery powered” indicates that the UE is not battery powered. |