3GPP TSG-RAN WG2 Meeting #105bis			Tdoc R2-1903896

Xian, China, 8th– 12th April 2019

Title:	[DRAFT] Draft LS on Data forwarding for conditional handover

Response to:

Release:	Rel-16

Work Item:	NR_Mob_enh

Source:	Ericsson [To be RAN2]

To:	RAN3

Cc:

Contact Person:

#### Name:	Cecilia Eklöf

E-mail Address:	cecilia.eklof@ericsson.com

Send any reply LS to:3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network.

RAN2 has concluded that packet forwarding from source to target gNB can start at different points in time. The following options for when to start data forwarding at conditional handover have been discussed:

- After configuration of conditional handover in the UE

- After handover completion in target node (i.e. after path switch)

- After a new “bye” message/indication sent by the UE to the source node before HO execution

- After target indicates that the UE has accessed the cell

To start data forwarding after configuration of conditional handover in the UE means that a lot of data will be forwarded and also to potential target cells which the UE may never handover to. It may also lead to packet duplication as the source node will continue sending data to the UE.

To start data forwarding after handover completion is on the other hand a bit late as the UE has already started receiving data in the new cell.

One option is that the UE sends a “bye” message when leaving the source cell, triggering data forwarding. The drawback with such a message is that it is unreliable as the radio conditions are usually quite bad when the UE executes the handover.

An alternative could be that the target gNB sends a message to source gNB when the UE has started accessing the new cell.

RAN2 would like to ask RAN3 for feedback regarding possible enhancements to data forwarding for CHO, such as sending an early message from target gNB to source gNB when the UE has started accessing the new cell.

2. Actions:

To RAN3 group.

ACTION:  RAN2 kindly asks RAN3 to give feedback to RAN2 on the question above.

3. Date of Next TSG WG RAN2 Meetings:

RAN2#106	13 - 17 May 2019		Reno, US

RAN2#107	26 - 30 Aug 2019		Prague, Czech Republic