3GPP TSG-RAN WG2 Meeting #105bis	R2-1903902

Xian, China, 08 – 12 April 2019	(was R2-1900438)

Agenda item:	11.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on RSSI and CO measurements

WID/SID:	NR-U - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the NR-U WID [1] the RRM and RLM extensions to be designed for NR-U are described as follows

Further, for mobility the WID states:

Here we will expand on some details on solutions that have been proposed during the SI phase, and expand on the use of RSSI and CO for mobility.

# 2	Discussion

## Defining RSSI and CO

RSSI was defined in LAA as a wideband measurement across the carrier. The measurement is configured according to [36.331] in the RMTC (RSSI MTC) of a measurement object and will apply to the measurement object’s carrier frequency.

In LAA the RMTC can be configured as

rmtc-Period-r13 ENUMERATED { ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r13 INTEGER (0..639) OPTIONAL, -- Need ON

measDuration-r13 ENUMERATED { sym1, sym14, sym28, sym42, sym70},

That is, RSSI may be gathered for instance every 40 msec over 5 msec (sym70), either at a random location within the period, or a defined offset. The long periodicity is designed to capture long-term variations in channel usage, while the offset and measurement duration allow to avoid recurring transmissions from the own or neighbour cells, or measure particular periods of neighbour cells. By defining more than one measurement object one may also establish more than one RMTC for a given carrier.

In NR-U, as a starting point, above configuration options should be introduced as well. The RSSI measurement could be defined per 20-MHz sub-band, or for the wider bandwidths of 40, 80, 160 MHz. While now many unlicensed deployments even in the 5 GHz range use a 20 MHz granularity, in the future this may change and having to maintain that level may not be efficient in reporting. It should be up to the network to decide whether it sees a benefit in gathering only 20-MHz measurements from which it can compose a wideband measurement.

Channel Occupancy (CO) was defined in LAA as a percentage of time of the RSSI being above a threshold. More advanced definitions such as measuring the rate of busy periods [2] or measuring gaps as the amount of times the RSSI drops below a threshold for longer than 25 usec [3] have been already proposed. Also measuring access delay from past transmission as is e.g. known from Wi-Fi is conceivable. However, our view is that NR-U deployments will work also without those enhancements, therefore they should be considered an optimization to be considered only in the next releases. Moreover, any new measurements should only be introduced if they bring clear quantifiable gains.

Proposal 1: Do not introduce new measurements beyond CO and RSSI in this release, as they are not essential for the operation of NR-U.

## RSSI and CO for mobility

During the SI phase it was proposed to define measurement events based on the RSSI and CO, with the purpose of triggering mobility. We recall that the RSRQ is using the RSSI as well, and hence for mobility corresponding measures are already indirectly available. Further, the RSSI and CO measurements would not be designed to react to a UE moving from one cell to the next, but to react to for instance changing load conditions in a cell. Thus, a fast reaction as needed for HOs triggered by moving out of the coverage of a cell does not seem necessary, as the UE is not in danger of losing connectivity (even when the load increases, the gNB or UE should be able to eventually gain the channel). Moreover, the measurements of RSSI and CO requires a longer measurement period and thus they are not appropriate measurements quantities for fast reaction.

Observation 1: RSSI and CO measurements are designed to capture the load in a cell, and not whether a UE is moving out of the coverage of a cell (other measurements are available for that purpose).

Observation 2: RSSI and CO measurements are carried out in relatively longer period of time, and thus they are not appropriate for fast reaction.

Proposal 2: Limit the RSSI and CO measurement reporting to periodic reports.

# 3	Conclusion

We discussed RSSI and CO measurements and made following observations and proposals:

Proposal 1: Do not introduce new measurements beyond CO and RSSI in this release, as they are not essential for the operation of NR-U.

Observation 1: RSSI and CO measurements are designed to capture the load in a cell, and not whether a UE is moving out of the coverage of a cell (other measurements are available for that purpose).

Observation 2: RSSI and CO measurements are carried out in relatively longer period of time, and thus they are not appropriate for fast reaction.

Proposal 2: Limit the RSSI and CO measurement reporting to periodic reports.

# 4	References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc.

- R2-1902025, Measurement for medium contention, LG Electronics Inc.

- R2-1900438, Considerations on RSSI and CO measurements, Nokia, Nokia Shanghai Bell