3GPP TSG-RAN WG2 Meeting #105bis	R2-1903904

Xian, China, 08 – 12 April 2019	(was R2-1900440)

Agenda item:	11.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on mobility and measurement gap enhancements in NR-U

WID/SID:	NR-U - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss one of the proposed enhancements for increasing robustness in connected mode mobility. We also address the need for measurement gap modifications in unsynchronized scenarios.

# 2	Discussion

## 2.1	Robustness for Mobility Procedures

The NR-U WID [1] the Mobility procedures to be designed for NR-U are described as follows:

The technical report to the study item [TR 38.889], detailed the solutions to be studied for mobility as:

While not mentioned by name, the above highlighted text points to conditional handover (CHO).

Our view is that the applicability and possible NR-U specific enhancements of CHO can only be investigated after the concepts and the procedures for CHO have been agreed in NR mobility enhancements work item to avoid parallel work.

Proposal 1: Address condition handover in NR-U towards the end of the WI, to allow to re-use the decisions and agreements concerning CHO made in NR mobility enhancements work item.

## 2.2	Measurement Gaps

In order for a UE to perform inter-frequency neighbour cell RRM measurements, measurement gaps are needed. During measurement gaps a UE can typically neither receive from nor transmit to its serving cell(s).

The possible gap patterns in NR (as defined in 38.133 table 9.2.2-1) have a maximum length of 6 msec, and minimum repetition period of 20 msec and are designed to allow capturing the SSB(s) of neighbour cells.

NR-U should support asynchronous deployments. Baseline Rel-15 NR can also support asynchronous deployments by sending the SSBs at a periodicity of 5ms. In this way a measurement gap of 6 ms will always be able to cover the SSBs transmitted by asynchronous neighbour cells. However, transmitting the SSBs with a periodicity of 5 ms might be challenging in NR-U due to the restrictions imposed by LBT to the frequency of transmission of discovery reference signals, including SSBs. Therefore, we propose that RAN2 discusses whether enhancements to the Rel-15 NR measurement gap configuration are needed to handle asynchronous deployments in NR-U.

Proposal 2: Study what changes of the measurement gap configuration are needed to support reliable measurements of unsynchronized neighbour NR-U cells.

If the length of measurement gaps were to be increased as well as their number, this would negatively impact DRX and power consumption. Indeed, the DRX timers would have to be long enough to avoid expiring when a measurement gap occurs. The longer the gap and the more often they occur the more serious the problem become. To avoid increasing power consumption, the subframes occurring during a measurement gap should be excluded from the counters.

Proposal 3: Exclude the PDCCH subframes colliding with a measurement gap from the DRX timers.

# 2	Conclusion

In this paper we have the following proposals:

Proposal 1: Address condition handover in NR-U towards the end of the WI, to allow to re-use the decisions and agreements concerning CHO made in NR mobility enhancements work item.

Proposal 2: Study what changes of the measurement gap configuration are needed to support reliable measurements of unsynchronized neighbour NR-U cells.

Proposal 3: Exclude the PDCCH subframes colliding with a measurement gap from the DRX timers.

# 3	References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc.