3GPP TSG-RAN WG2 #105bis	R2-1903962

# Xi’an, P. R. China, 8th – 12th Apr 2019

Agenda Item:	11.1.4

Source:	Ericsson

Title:	SR Enhancement in IAB Network

Document for: Discussion, Decision

# 1	Introduction

For an IAB path, there are multiple radio interfaces, comprising one access interface and at least one backhaul interface. A packet to be transmitted in uplink or downlink through the IAB path experiences uplink/downlink radio transmissions in each radio interface. For each uplink data transmission, there is uplink transmission delay according to the SR procedure in Rel-15 in each hop. In order to reduce the uplink transmission delay along an IAB path, certain enhancement to reduce the delay due to SR procedure may be desired. This paper discusses the options to optimize the SR procedure in IAB network by referring to the SR procedure in Rel-15.

# 2	Discussion

## 2.1	Uplink Transmission Delay

The uplink scheduling delay was discussed in Section 8.6 of [1]. Below is the brief introduction of uplink scheduling delay for a packet to be transmitted through an IAB path.

The SR/BSR procedure of Rel-15 is defined in Section 5.4.4 and Section 5.4.5 of 3GPP TS 38.321 V15.3.0. In summary, the uplink data transmission procedure according to Rel-15 procedure looks like:

- Step.1 When new data comes, a regular BSR may be triggered and a SR transmission is initiated to request for an uplink grant(s) for BSR;

- Step.2 Upon the reception of SR from a UE, the gNB may send an initial grant to allow the UE to transmit regular BSR;

- Step.3 Upon the reception of the UL grant, the UE constructs the MAC PDU with the BSR MAC CE included and transmit the MAC PDU using the UL grant.

- Step.4 Upon the reception of a BSR from the UE, the gNB sends subsequent UL grants to the UE for the rest data transmission according to the received BSR;

- Step.5 Upon the reception of the subsequent UL grants, the UE performs uplink data transmission accordingly.

For an IAB path, there are an access hop and at least one backhaul hop, and the data transmission procedure for a packet at the next hop can only be started when the data transmission procedure is finished in previous hop. This means that a packet sequentially experiences the same uplink transmission procedure at each hop according to Rel-15 SR/BSR procedure. In this way, the total uplink transmission delay in the IAB path is the sum of transmission delay of all these hops. Figure 1 shows the sequential uplink transmissions of all hops along an IAB node for a data packet. No that the HARQ retransmission procedures are not illustrated in the figure.

Figure 1 Uplink data transmission in IAB path reusing Rel-15 SR/BSR procedure

According to Rel-15 SR/BSR procedure, the uplink transmission procedure of a packet (i.e. SR-BSR-data TX) is sequential across hops, i.e. the uplink transmission procedure of a packet at a hop can only be started when the uplink transmission procedure of the packet ends at the previous hop.

The total uplink transmission delay of a data packet along an IAB path increases proportionally with the number of hops.

SR/BSR procedure enhancement is needed in order for uplink latency reduction in IAB network.

## 2.2	Solutions

In order to reduce the latency of sequential uplink data transmission procedures according to Rel-15 procedure, in [2] an enhanced SR/BSR procedure to allow cascading uplink data transmission procedure across hops through an IAB path is proposed. According to the enhanced SR/BSR procedure, an SR can be triggered based on the expected available bits by an IAB node, where the expected bits can be determined based on the received BSR from a child IAB node and/or the uplink transmissions to be scheduled. According to such procedure, the SR/BSR procedure at a hop can be triggered when an IAB receives BSR at the previous hop.

In addition to SR triggering based on the expected bits to be received, the SR transmission can be improved to carry buffer status information. There are the following options:

- Option 1: Buffer status indication using multiple single-bit PUCCH SR resources. There are further the following two alternatives with respect to buffer status indication.

  - Option 1.1, multiple single-bit SR resources are configured per SR configuration and each SR resource corresponds to a respective buffer status, wherein the buffer status indicates the number of available bits in the buffer when the SR is transmitted.

  - Option 1.2, multiple single-bit SR resources are configured per SR configuration and each SR resource corresponds to a respective number of expected available bits in the buffer according to the uplink transmissions to be scheduled and/or the received BSR in the previous hop.

- Option 2: buffer status indication using multiple-bit PUCCH SR resource. Similar to Option 1, there are the following two alternatives with respect to buffer status indication.

  - Option 2.1 a multiple-bit PUCCH SR resource is configured per SR configuration and each value of the PUCCH SR indicates a respective buffer status, wherein the buffer status indicates the number of available bits in the buffer when the SR is transmitted.

  - Option 2.2 a multiple-bit PUCCH SR resource is configured per SR configuration and each value of PUCCH SR indicates a respective buffer status, wherein the buffer status indicates the expected number of available bits in the buffer according to the uplink transmissions to be scheduled and/or the received BSR in the previous hop.

The comparison between the two options are listed in the following table.

|  | Latency reduction | Standardization change in RRC/MAC | Standardization change in PHY | Buffer status error |
| --- | --- | --- | --- | --- |
| Option 1.1 | Medium: SR TX occurs when there is non-zero buffer size. | Yes | No: reuse PUCCH SR format in Rel-15 | Reported buffer status does not consider the data to be received. Grant waste is avoided but large delay is expected. |
| Option 1.2 | Large: SR TX occurs when there is non-zero expected buffer size. | Yes | No: reuse PUCCH SR format in Rel-15 | Reported buffer status reflects the predicated buffer status. Latency could be reduced with potential grant waste.. |
| Option 2.1 | Medium: SR TX occurs when there is non-zero buffer size | Yes | Yes: new PUCCH SR to carry multiple bits need to be defined. | Reported buffer status does not consider the data to be received. Grant waste is avoided but large delay is expected. |
| Option 2.2 | Large: SR TX occurs when there is non-zero expected buffer size. | Yes | Yes: new PUCCH SR to carry multiple bits need to be defined. | Reported buffer status reflects the predicated buffer status. Latency could be reduced with potential grant waste.. |

Based on the comparison the above table, Option 1 has similar latency reduction performance as Option 2. However, Option 2 requires new PUCCH SR format to carry multiple bits while Option 1 just reuses the single bit PUCCH SR defined in Rel-15, which means Option 1 requires lower standardization /implementation complexity.

Option 1 and Option 2 have similar latency reduction performance.

Option 1 has lower standardization and implementation complexities than Option 2.

Based on the above analysis, we would like to propose the following:

In addition to SR/BSR enhancement considering the anticipated available data in [2], RAN2 may consider an implicit buffer status indication through the usage of different SR resources.

If implicit buffer status indication via SR transmission is adopted, RAN2 support Option 1.1 and/or Option 1.2.
Option 1.1 – multiple SR resources are configured so that each resource points to a buffer range. The SR transmitted by the UE will indicate the buffer range within the current buffer status is.
Option 1.2 - multiple SR resources are configured so that each resource points to a buffer range. The SR transmitted by the UE will indicate the buffer range within the expected buffer status will be.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	According to Rel-15 SR/BSR procedure, the uplink transmission procedure of a packet (i.e. SR-BSR-data TX) is sequential across hops, i.e. the uplink transmission procedure of a packet at a hop can only be started when the uplink transmission procedure of the packet ends at the previous hop.

Observation 2	The total uplink transmission delay of a data packet along an IAB path increases proportionally with the number of hops.

Observation 3	SR/BSR procedure enhancement is needed in order for uplink latency reduction in IAB network.

Observation 4	Option 1 and Option 2 have similar latency reduction performance.

Observation 5	Option 1 has lower standardization and implementation complexities than Option 2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In addition to SR/BSR enhancement considering the anticipated available data in [2], RAN2 may consider an implicit buffer status indication through the usage of different SR resources.

Proposal 2	If implicit buffer status indication via SR transmission is adopted, RAN2 support Option 1.1 and/or Option 1.2. Option 1.1 – multiple SR resources are configured so that each resource points to a buffer range. The SR transmitted by the UE will indicate the buffer range within the current buffer status is. Option 1.2 - multiple SR resources are configured so that each resource points to a buffer range. The SR transmitted by the UE will indicate the buffer range within the expected buffer status will be.

# 5	References

3GPP TS 38.874 V16.0.0.

R2-1903965, Uplink scheduling in IAB networks: BSR