3GPP TSG-RAN2 Meeting #105-Bis 	R2-1904003

Xi’an, China, April 8th – April 12th, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR 38.331 - Introduction of RRC Segmentation - Uplink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS_RAN |  |  |  |  |  |  | Date: |  | 2019-03-27 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To introduce RRC Segmentation – general solution + support for use of RRC Segmentation during transmission or UECapabilityInformation. Signaling of RRC segmentation capability / support by the network in enquiry message, to indicate to UE whether it can use RRC segmentation |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding support of RRC Segmentation - Adding support of RRC Segmentation for transmission of UECapabilityInformation - Adding network support indication of RRC segmentation to UECapabilityEnquiry message - Adding explanation of how to use TransactionIdentifier together with RRC Segments |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC Segmentation for UECapabilityInformation will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.3.0, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | A corresponding CR is introduced for DL messages (R2-1904004) Note: This draft CR is to be re-written to RAN2 #106, based on approved version of the specification (15.5.0). Intention is to merge this CR into running CR |  |  |  |  |  |  |  |  |

First CHANGE

### 6.2.2	Message definitions

#### === Unmodified sections omitted ===

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request UE radio access capabilities for NR as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UECapabilityEnquiry information element

-- ASN1START

-- TAG-UECAPABILITYENQUIRY-START

UECapabilityEnquiry ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityEnquiry                 UECapabilityEnquiry-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityEnquiry-IEs ::=     SEQUENCE {

ue-CapabilityRAT-RequestList        UE-CapabilityRAT-RequestList,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                UECapabilityEnquiry-IEs-v16xy                                             OPTIONAL

}

UECapabilityEnquiry-IEs-v16xy ::=   SEQUENCE {

rrc-Segmentation                    ENUMERATED {supported}                                                  OPTIONAL,   -- Need N

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYENQUIRY-STOP

-- ASN1STOP

UECapabilityEnquiry field descriptions

rrc-Segmentation If present, the UE may segment its UECapabilityInformation message, i.e., it may use the field ueCapabilityInformationSegment in the UECapabiltyInformation message if the actual UECapabilityInformation-IEs exceed the message size limit.

#### –	UECapabilityInformation

The IE UECapabilityInformation message is used to transfer UE radio access capabilities requested by the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UECapabilityInformation information element

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityInformation             UECapabilityInformation-IEs,

criticalExtensionsFuture            SEQUENCE {

ueCapabilityInformationSegment          RRCSegment { UECapabilityInformation-IEs, TBD },

criticalExtensionsFuture2               SEQUENCE {}

}

}

}

UECapabilityInformation-IEs ::=     SEQUENCE {

ue-CapabilityRAT-ContainerList      UE-CapabilityRAT-ContainerList                                          OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

-- ASN1STOP

NEXT CHANGE– Same as in R2-1904004

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	RRCSegment

The IE RRCSegment is used to convey segments of large information elements that exceed the maximum RRC message size (e.g. due to the PDCP SDU size limit of 9000 byte). The ElementTypeParam determines the IE type to be segmented by the transmitting RRC entity and reassembled by the receiving RRC entity. The maxNrofElements determines the maximum number of segments for an instance of ElementTypeParam.

The RRCSegment type should be used on the top level of an RRC message. The RRC messages containing the segments of the same original RRC information element shall contain the same rrc-TransactionIdentifier.

The transmitting entity (UE or NW) uses the RRCSegment only if the unsegmented message exceeds the size limit of PDCP (9000 byte).

RRCSegment information element

-- ASN1START

-- TAG-RRCSEGMENT-START

RRCSegment { ElementTypeParam, INTEGER:maxNrofSegments } ::=          SEQUENCE {

segmentNumber           INTEGER(1..maxNrofSegments),

isLastSegment           BOOLEAN,

segment                 OCTET STRING

}

-- TAG-RRCSEGMENT-STOP

-- ASN1STOP

RRCSegment field descriptions

isLastSegment Indicates that segment contains the last segment of the segmented information element.

segment One segment of a contained information element.

segmentNumber The ID of the segment which the receiving RRC entity uses to reconstruct the original information element.

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUP-RELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUP-RELEASE-STOP

-- ASN1STOP

NEXT CHANGE – Same as in R2-1904004

### 6.3.4	Other information elements

#### === Unmodified sections omitted ===

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type and together with the segmentNumber (if the message contains an RRCSegment), for the identification of an RRC procedure (transaction). If the RRC message contains an RRCSegment, the RRC-TransactionIdentifier shall be set to the same value in all segments of the same information element.

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=       INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

END OF CHANGES