3GPP TSG-RAN WG2 Meeting #105bis	DRAFT R2-1904029

Xi’an, China, 08 – 12 April 2019

Title:	[DRAFT] LS on restrictions for RAR payload size for NR-U

Response to:	-

Release:	Release 16

Work Item:	NR_unlic

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Samuli Turtinen

E-mail Address:	samuli.turtinen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

As concluded during the study item, the additional Tx opportunities should be also applicable to Msg3 transmissions, RAN2 will wait for RAN1 to design the details how to achieve this in the provided RAR UL grant.

It is noted, however, that the RAR MAC PDU format is not easily extendable unless a new format is introduced for the use of NR-U capable UEs and used only in cells operating in unlicensed bands. However, as LAA (License Assisted Access) can also be employed, the RAR format needs to be applicable to the legacy UEs as well in this case as the RAR is transmitted on SpCell of the UE – it could be a bit clumsy to have RAR PDU format dependent on the band it is received from. Hence, the possible enhancements to support more Tx opportunities for Msg3 transmission should consider the limits exposed by the current RAR MAC PDU format and the RAR payload size of 7 bytes.

The MAC RAR format is provided hereinafter for reference (TS 38.321):

Figure 6.2.3-1: MAC RAR

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to consider enhancing the Tx opportunities for Random Access Msg3 by considering the RAR payload size of 7 bytes should not be increased.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#106		13 – 17 May 2019		Reno

3GPP RAN2#107		26 – 30 August 2019		Prague