3GPP TSG-RAN WG2 Meeting #105bis	R2-1904100

Xi’an, China, 8-12 Apr 2019 	Resubmission of: R2-1901281

Agenda item:	11.8.2.1

Source: 	Huawei, HiSilicon

Title: 	Considerations on OTDOA in NR

Document for:	Discussion and Decision

Introduction

In RAN1 #95 meeting, DL Timing of arrival path based positioning technique was agreed as candidate DL positioning method in NR. In the last RAN1 meeting, there were the following agreements about OTDOA [1].

Agreement: - NR should support timing (DL-TDOA) based DL-only positioning techniques in FR1 and FR2 - NR should support angle (DL-AoD) based DL-only positioning techniques with at least beam sweeping at least at the gNB in FR1 and FR2 Note: This does not necessarily need different reference signals from the timing based DL-only positioning techniques Agreement: NR DL PRS design for FR1 and FR2 supports: - Configurable NR DL PRS signal bandwidth - FFS granularity of configuration, relationship with BWPs, whether the configuration is cell and/or UE specific - Configurable NR DL PRS signal numerology (SCS) - FFS configurability of CP for NR DL PRS - Configurable NR DL PRS frequency and time allocation - Use of DL beam sweeping / alignment - i.e. beam alignment of gNB DL PRS transmission and UE reception of DL PRS - Localized in time NR DL PRS transmissions with periodic and/or on-demand resource allocation - FFS signaling details - Dedicated NR DL PRS resources - time-frequency grid at resource block level - PRS transmitted in one cell may or may not collide with PRS transmissions in other cell - e.g. frequency vShift/comb-offset is the same or different for two different PRSs in the same RB - There is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources - FDM multiplexing with other signals at RE level inside of PRS time-frequency grid is precluded - FFS interference randomization techniques across PRS signals - FFS shared in time/frequency NR DL PRS resources with other transmissions including data/control - FFS which physical channel/signals can share resources with NR DL PRS - FFS interference randomization techniques for PRS transmission with other signals

In this contribution, we provide our consideration on OTDOA in NR from RAN2 perspective.

Discussion

OTDOA in Rel-15 includes the following procedures

- Assistance data transfer procedure between LMF and UE. The purpose of this procedure is to enable the LMF to provide assistance data to the UE (e.g., as part of a positioning procedure) and the UE to request assistance data from the LMF (e.g., as part of a positioning procedure).

- Capability information transfer. The purpose of this procedure is to let the LMF obtain the capability information from the UE to the LMF.

- Location information transfer procedure. The purpose of this procedure is to enable the LMF to request position measurements from the UE, or to enable the UE to provide location measurements to the LMF for position calculation.

The three procedures are basic procedures of OTDOA and also are essential for OTDOA in Rel-16. Hence, the three procedures should be included OTDOA procedures in Rel-16.

Proposal 1: LPP procedure for OTDOA in Rel-16 should include assistance data transfer procedure between LMF and UE, capability information transfer, and location information transfer procedure.

PRS is omnidirectional transmitted in LTE but the transmission of reference signal in NR could be spatially filtered. In the last RAN1 meeting, it was agreed to use DL beam sweeping / alignment to transmit PRS signal. For DL beam sweeping based PRS transmission, from UE's perspective, it is essentially similar to the omnidirectional PRS transmission in LTE. The only problem may be the reduction of PRS density received by UE due to some PRS occasions have to be used to other beams/directions, which may have an impact on positioning performance. But this can be mitigated by some other means, e.g., by increasing PRS bandwidth, or using PRS resource set where each PRS resource has its own spatial filtering. So, no additional procedure is needed to support beam sweeping based PRS transmission.

Proposal 2: No additional procedure in LPP is needed to support beam sweeping based PRS transmission.

For DL beam alignment based PRS transmission, a pair of gNB transmitting beam and UE receiving beam is maintained and PRS could only be transmitted on given Tx beams and UE receives PRS on corresponding Rx beams, which is helpful to improve PRS occasion utilization efficiency comparing with the beam sweeping scheme. DL beam alignment include Tx beam alignment and Rx beam alignment.

To achieve Tx beam alignment, gNBs performing OTDOA should collect UE’s beam information to know which beams of them UE can receive first. For example, a few neighbor gNBs perform OTDOA positioning of one UE. UE could know indexes of the SSB/CSI-RS it receiving from each gNB, respectively, by neighbor cell measurement. If neighbor gNBs obtain this information by some means, they could transmit PRS on corresponding SSB beam. In this way, the orientated PRS transmission could be achieved. However, this mechanism is not included in current OTDOA procedure. Therefore, a beam information collection procedure should be added into OTDOA procedure to support beam alignment based PRS transmission.

Proposal 3: Beam information collection procedure should be added into OTDOA procedure to support beam alignment based PRS transmission.

To achieve Rx beam alignment, the Rel-15 QCL (quasi co-location) indication could be considered and reused. For example, UE may be configured for PRS with a QCL source RS, whose Rx beam information is already known by the UE, so UE does not have to perform Rx beam training to receive PRS, and the oriented PRS reception could be achieved.

Observation 1: Rel-15 QCL indication should be considered to support beam alignment based PRS reception.

Moreover, according to the agreement in the last RAN1 meeting, OTDOA in Rel-16 needs to support localized-in-time NR DL PRS transmissions with periodic and/or on-demand resource allocation. PRS transmission with on-demand resource allocation is not supported in LTE, where PRS is preconfigured to the network nodes as part of their intrinsic properties and it is transferred to the E-SMLC/LMF via the LPPa/NRPPa protocol for positioning usage. To achieve the on-demand resource allocation for DL PRS, there may be two options:

- Option1 is to enable LMF to configure PRS resources of related gNBs based on QoS requirement of UE’s LCS.

- Option 2 is to enable each gNB to decide it PRS resources configuration according some inputs from LMF.

But anyway, no matter which option is adopted, a new procedure is needed to achieve PRS transmission with on-demand resource allocation. Meanwhile, since PRS could be on-demand, LMF can also indicate gNB to switch off some PRS transmission as long as the respective demand does not exist anymore.

Proposal 4: RAN2 should study a new procedure in LPP to support PRS transmission with on-demand resource allocation.

Conclusion

In this contribution, we considered our consideration on OTDOA in NR from RAN2 perspective, and we get the following observation and proposals:

Observation 1: Rel-15 QCL indication should be considered to support beam alignment based PRS reception.

Proposal 1: LPP procedure for OTDOA in Rel-16 should include assistance data transfer procedure between LMF and UE, capability information transfer, and location information transfer procedure.

Proposal 2: No additional procedure in LPP is needed to support beam sweeping based PRS transmission.

Proposal 3: Beam information collection procedure should be added into OTDOA procedure to support beam alignment based PRS transmission.

Proposal 4: RAN2 should study a new procedure in LPP to support PRS transmission with on-demand resource allocation.

References

[1] Chairman’s Notes of RAN1 AdHoc 1901.

Text proposal to TS 38.305

- X.Y.Z	OTDOA Positioning Procedures

The procedures described in this sub-clause support OTDOA positioning measurements obtained by the UE and provided to the LMF using LPP.

In this version of the specification only the UE-assisted OTDOA positioning is supported.

- X.Y.Z.1	Capability Transfer Procedure

The Capability Transfer procedure for OTDOA positioning is described in TS 36.305 sub-clause 7.1.2.1.

- X.Y.Z.2	Assistance Data Transfer Procedure

X.Y.Z.2.1 	Assistance Data Transfer between LMF and UE

The purpose of this procedure is to enable the LMF to provide assistance data to the UE (e.g., as part of a positioning procedure) and the UE to request assistance data from the LMF (e.g., as part of a positioning procedure).

X.Y.Z.2.1.1	LMF initiated Assistance Data Delivery

Figure X.Y.Z.2.1.1-1 shows the Assistance Data Delivery operations for the OTDOA positioning method when the procedure is initiated by the LMF.

Figure X.Y.Z.2.1.1-1: LMF-initiated Assistance Data Delivery Procedure

- The LMF determines that assistance data needs to be provided to the UE (e.g., as part of a positioning procedure) and sends an LPP Provide Assistance Data message to the UE.

X.Y.Z.2.1.2	UE initiated Assistance Data Transfer

Figure X.Y.Z.2.1.2-1 shows the Assistance Data Transfer operations for the OTDOA positioning method when the procedure is initiated by the UE.

Figure X.Y.Z.2.1.2-1: UE-initiated Assistance Data Transfer Procedure

(1)	The UE determines that certain OTDOA positioning assistance data are desired (e.g., as part of a positioning procedure when the LMF provided assistance data are not sufficient for the UE to fulfil the request) and sends an LPP Request Assistance Data message to the LMF. This request includes an indication of which specific OTDOA assistance data are requested.  Additional information concerning the UE's approximate location and serving and neighbour cells may also be provided in the Request Assistance Data message and/or in an accompanying Provide Location Information message to help the LMF provide appropriate assistance data. This additional data may include the UE's last known location if available, the cell IDs of the UE serving NG-RAN node and possibly neighbour NG-RAN nodes, as well as NR E-CID measurements.

(2)	The LMF provides the requested assistance in an LPP Provide Assistance Data message, if available at the LMF. If any of the UE requested assistance data in step (1) are not provided in step 2, the UE shall assume that the requested assistance data are not supported, or currently not available at the LMF. If none of the UE requested assistance data in step (1) can be provided by the LMF, return any information that can be provided in an LPP message of type Provide Assistance Data which includes a cause indication for the not provided assistance data.

X.Y.Z.2.2	Assistance Data Delivery between LMF and gNB

The purpose of this procedure is to enable the gNB to provide assistance data to the LMF, for subsequent delivery to the UE using the procedures of sub-clause X.Y.Z.2.1 or for use in the calculation of positioning estimates at the LMF.

X.Y.Z.2.2.1	LMF-initiated assistance data delivery to the LMF

Figure X.Y.Z.2.2.1-1 shows the Assistance Data Delivery operation from the gNB to the LMF for the OTDOA positioning method, in the case that the procedure is initiated by the LMF.

Figure X.Y.Z.2.2.1-1: LMF-initiated Assistance Data Delivery Procedure

(1)	The LMF determines that certain OTDOA positioning assistance data are desired (e.g., as part of a periodic update or as triggered by OAM) and sends an NRPPa OTDOA INFORMATION REQUEST message to the gNB. This request includes an indication of which specific OTDOA assistance data are requested.

(2)	The gNB provides the requested assistance in an NRPPa OTDOA INFORMATION RESPONSE message, if available at the gNB. If the gNB is not able to provide any information, it returns an OTDOA INFORMATION FAILURE message indicating the cause of the failure.

- X.Y.Z.3	Location Information Transfer Procedure

The purpose of this procedure is to enable the LMF to request position measurements from the UE, or to enable the UE to provide location measurements to the LMF for position calculation.

X.Y.Z.3.1	LMF-initiated Location Information Transfer Procedure

Figure X.Y.Z.3.1-1 shows the Location Information Transfer operations for the OTDOA positioning method when the procedure is initiated by the LMF.

Figure X.Y.Z.3.1-1: LMF-initiated Location Information Transfer Procedure

(1)	The LMF sends an LPP Request Location Information message to the UE. This request includes indication of OTDOA measurements requested, including any needed measurement configuration information, and required response time.

(2)	The UE obtains OTDOA measurements as requested in step 1. The UE then sends an LPP Provide Location Information message to the LMF, before the Response Time provided in step (1) elapsed, and includes the obtained OTDOA measurements.  If the UE is unable to perform the requested measurements, or the Response Time elapsed before any of the requested measurements were obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

X.Y.Z.3.2	UE-initiated Location Information Delivery procedure

Figure X.Y.Z.3.2-1 shows the Location Information Delivery procedure operations for the OTDOA positioning method when the procedure is initiated by the UE.

Figure X.Y.Z.3.2-1: UE-initiated Location Information Delivery Procedure.

(1)	The UE sends an LPP Provide Location Information message to the LMF. The Provide Location Information message may include any UE OTDOA measurements already available at the UE.

- X.Y.Z.4	Beam information collection Procedure

The purpose of this procedure is to enable the gNBs to collect beam information from the UE via LMF, to support beam alignment based PRS transmission.

Figure X.Y.Z.3.1-1 shows the beam information collection operations for the OTDOA positioning method.

Figure X.Y.Z.3.1-1 beam information collection procedure

1. The LMF sends an LPP beam information request message to UE. This request includes serving and neighbour gNB identities and downlink reference signal type indicator.

2. The UE measures the reference signal of serving and neighbour gNBs indicated in beam information request message and obtain the beam information of each gNB.

3. The UE send an LPP beam information report message to LMF. This report includes the beam information of each gNB obtained in step 2.

4. The LMF sends beam information to corresponding serving and neighbour gNB by on-demand PRS request message.