3GPP TSG-RAN WG2 Meeting #105	R2-1904104

Xi'an, China, 08 Apr - 12 Apr 2019

Agenda item:	11.8.2

Source: 	Huawei, HiSilicon

Title: 	Considerations on E-CID positioning in NR

Document for:	Discussion and Decision

Introduction

In RAN1#95 [1], the following candidate techniques were agreed for study of downlink and uplink positioning.

Agreement: The following candidate techniques are considered for study of DL and UL positioning: - Timing based techniques - Round trip time measurement including support for multiple TRPs - Combination of DL and UL techniques for NR positioning - e.g. E-CID like techniques (including one or multiple cells)

In RAN1 #AH1901 meeting, the following agreement has been made on the additional aspects to consider.

Agreement: - Additional aspects that have been proposed during the study: - Receive signal waveform reporting - UE-based positioning - Multipath measurements - SFN transmissions for positioning - Techniques to lower UE complexity for OTDOA - Broadcast of assistance data - LMUs

In this contribution, we provide our consideration on E-CID in NR from RAN2 perspective.

E-CID

In the legacy E-CID in LTE and R15, two types of T_ADV have been defined:

- Type1 introduced in Rel-9: T_ADV = (eNB Rx – Tx time difference) + (UE Rx – Tx time difference)

- Type2 introduced in Rel-8: T_ADV = eNB Rx – Tx time difference

Correspondingly, the UE Rx-Tx time difference has been defined in RRC spec and LPP spec in Rel-8 and Rel-9, respectively.

TS 36.331:

TS 36.355:

While in the LPPa spec, the TADV result obtained by the eNB is transmitted to the E-SMLC in the LPPa message, as specified in TS 36.455:

Furthermore, in the WID for R16 positioning, the following objective has been incldued.

- Specify NR DL and UL reference signals to facilitate support of NR positioning techniques (DL-TDOA, DL-AoD, UL-TDOA, UL-AoA, multi-cell RTT and E-CID) [RAN1] - Support E-CID downlink measurements based on at least RRM measurements defined in NR Rel. 15 - Identify whether and which Rel-15 NR reference signals can be used for different NR positioning techniques - Define new DL positioning reference signals applicable at least for DL-TDOA, DL-AoD, RTT - Define UL SRS with possible enhancements for positioning which is applicable at least for RTT, UL-TDOA, UL-AoA

Based on the above discussion we think the stage-2 aspects of R16 positioning can follow that of the LTE as baseline.

Proposal 1: The procedure in TS 36.305 for E-CID can be adopted for the E-CID positioning in R16 as baseline.

Conclusion

In this contribution, we considered our consideration on E-CID in NR from RAN2 perspective, and we get the following observation and proposals:

Proposal 1: The procedure in TS 36.305 for E-CID can be adopted for the E-CID positioning in R16 as baseline.

References

[1] Chairman’s Notes of RAN1 AdHoc 1901.

TP for Downlink E-CID in TS 38.305

- X.Y.Z	Downlink E-CID Positioning Procedures

The procedures described in this subclause support E-CID related measurements obtained by the UE and provided to the E-SMLC using LPP. The term "downlink" is intended to indicate that from the E-SMLC perspective the involved measurements are provided by the UE; this set of procedures might also be considered as "UE-assisted, E-SMLC-based E-CID".

- X.Y.Z.1	Capability Transfer Procedure

The Capability Transfer procedure for E-CID positioning is described in subclause 7.1.2.1.

- X.Y.Z.2	Assistance Data Delivery Procedure

Assistance data delivery is not required for UE- or eNB-assisted forms of E-CID positioning.

- X.Y.Z.3	Location Information Transfer Procedure

The purpose of this procedure is to enable the E-SMLC to request position measurements from the UE, or to enable the UE to provide location measurements to the E-SMLC for position calculation (e.g., in case of basic self location where the UE requests its own location).

X.Y.Z.3.1	E-SMLC-initiated Location Information Transfer

Figure 8.3.3.3-1 shows the Location Information Transfer operations for the E-CID method when the procedure is initiated by the E-SMLC.

Figure 8.3.3.3-1: E-SMLC-initiated Location Information Transfer Procedure.

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of E-CID positioning. This request includes the E-CID measurements requested by the E-SMLC and supported by the UE as listed in Table 8.3.2.2-1 together with a required response time.

(2)	The UE performs the requested measurements and sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

X.Y.Z.3.2	UE-initiated Location Information Delivery procedure

Figure 8.3.3.3.2-1 shows the Location Information Delivery procedure operations for the E-CID method when the procedure is initiated by the UE.

Figure 8.3.3.3.2-1: UE-initiated Location Information Delivery Procedure.

(1)	The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include any UE measurements already available at the UE.

TP for Uplink E-CID in TS 38.305

- X.Y.Z	Uplink E-CID Positioning Procedures

The procedures described in this sub clause support E-CID related measurements obtained by the eNodeB and provided to the E-SMLC using LPPa. The term "uplink" is intended to indicate that from the E-SMLC point of view, the involved measurements are provided by the eNodeB; this set of procedures might also be considered as "eNB-assisted E-CID". An example of this uplink E-CID positioning method is AoA+TADV.

NOTE:	Uplink E-CID positioning not using TADV type 1 does not require LCS support in the UE and therefore, works also with legacy UEs (assuming that any involved measurements delivered in RRC are supported by the legacy UEs).

- X.Y.Z.1	Position Capability Transfer Procedure

The position capability transfer procedure is not applicable to uplink E-CID positioning not using TADV type 1. For uplink E-CID positioning using TADV type 1, the Capability Transfer procedure for E-CID positioning is described in subclause 7.1.2.1.

- X.Y.Z.2	Assistance Data Delivery Procedure

The assistance data delivery procedure is not applicable to uplink E-CID positioning.

- X.Y.Z.3	Position Measurement Procedure

The purpose of this procedure is to enable the E-SMLC to request position measurements from the eNodeB.

X.Y.Z.3.1	E-SMLC-initiated Position Measurement

Figure 8.3.4.3.1-1 shows the Position Measurement operations for the uplink E-CID method when the procedure is initiated by the E-SMLC.

Figure 8.3.4.3.1-1: E-SMLC-initiated Position Measurement Procedure

(1)	The E-SMLC sends an LPPa E-CID MEASUREMENT INITIATION REQUEST message to the eNodeB. This request includes indication of E-CID measurements requested and whether the result is expected only once or periodically.

(2)	If the E-SMLC in step (1) requested UE measurements (i.e., RSRP, RSRQ measurements), or if the eNodeB requires UE measurements associated with the measurements requested by the E-SMLC (e.g., TADV type 1, which requires a UE Tx-Rx time difference measurement to be delivered from the UE to the eNodeB), the eNodeB may configure the UE to report the measurement information requested as specified in TS 36.331 [14].

(3)	If the result is expected only once and the eNodeB initiates at least one of the E-CID measurements as requested, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION RESPONSE to the E-SMLC, which includes the obtained E-CID measurements. If the result is expected periodically and the eNodeB is able to initiate at least one of the E-CID measurements as requested, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION RESPONSE to the E-SMLC, which does not include any result. The eNodeB reports then the obtained measurements by initiating the E-CID Measurement Report procedure, with the requested periodicity. If the eNodeB is unable to initiate any of the requested measurements as requested from the E-SMLC, or is unable to instigate any of the required RRC procedures to obtain the requested measurements from the UE, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION FAILURE message providing the error reason. If the failure occurs during a periodic reporting, the eNodeB sends an LPPa E-CID MEASUREMENT FAILURE INDICATION message.