3GPP TSG-RAN2 Meeting #105bis	R2-1904142

Xi'an, China, 8 - 12 April 2019

Agenda Item:	11.11.2 Power saving enhancements of paging procedure

Source: 	Ericsson

Title:  	CMAS reception and power saving

Document for:	Discussion and Decision

# Introduction

In this contribution CMAS reception and power consumption is discussed. CMAS reception was not discussed during RAN2#105 [1].

# Background

A CMAS capable UE is required to acquire SIB8 immediately when it receives a Paging message including etwsAndCmasIndication. In case the CMAS warning message does not fit into a single SIB8 transmission, the CMAS warning message is segmented. The UE is required to support reception of concurrent CMAS transmissions, i.e. repetitions of different CMAS warning messages which are interleaved (but segments of different CMAS messages cannot be interleaved).

When the gNB receives a WRITE-REPLACE WARNING REQUEST from the AMF this message includes, among others, the Repetition Period (0 = indefinitely, or 1..217-1 ~ 36 hours) and Number of Broadcasts Requested (0= no repetitions, or 1.. 65535) of the warning message. The gNB broadcasts the warning messages in all the cells of the gNB, unless otherwise indicated by Warning Area List IE (e.g. list of NR Cell IDs).

In case the cell coverage is large, and the emergency is only applicable in a smaller geographical area of the cell, the gNB may also receive Warning Area Coordinates in the WRITE-REPLACE WARNING REQUEST. The gNB appends the geographical information to the warning message. The CMAS capable UE uses this information in the upper layers to display the warning message only when the UE is inside the Warning Area Coordinates. This function improves the CMAS service and may avoid users to opt-out of CMAS notifications when received frequently but not applicable at the location of the UE.

System and higher layer aspects for CMAS are standardized in ATIS, where CMAS is denoted as Wireless Emergency Alert (WEA).

# Discussion

There are different CMAS warning message types, ranging from Presidential Level, Extreme Alerts with Severity of Extreme, Urgency of Immediate, and Certainty of Observed, to Child Abduction Emergency alerts (aka Amber alerts). From statistics derived from logs in live networks broadcasting CMAS messages it can be observed that CMAS messages can be repeated and broadcasted for a long time [2,3]. From example Amber alerts are typically broadcasted for 24 hours, with a repetition interval in the order of 10-15 minutes.

Observation 1: A CMAS capable UE may receive multiple repetitions of the same alert during a long time.

When a CMAS alert (first or repetition) is broadcasted by the gNB, the gNB has to page the UEs in the cells for some time to ensure that all UEs have reliably received the CMAS alert, e.g. page the UEs during at least one or more DRX cycle(s). During that time the CMAS alert is repeated in SIB8 with the scheduling periodicity indicated in SIB1.

ATIS recently introduced an enhancement coupled with the Warning Area Coordinates. At the time of the reception of the alert, the user may not be inside the geographical area where the alert is applicable, but the UE may roam into that area later. To trigger the UE to re-check whether it is inside the area indicated by the Warning Area Coordinates, ATIS introduced a new WEA message called WEA Handset Action Message (WHAM) message. When the UE receives a WHAM message it is required to take the indicated action, e.g. re-check if it is inside the Warning Area Coordinates of a previously received CMAS alert. A WHAM message may be repeated frequently e.g. every 1-5 minutes:

Observation 2: A CMAS capable UE cannot ignore repetitions to save power.

From the above description it can be noted that CMAS reception may not be that efficient from a power consumption perspective.

Proposal 1: RAN2 to study potential power saving enhancements for CMAS reception without changing the ATIS requirements.

RAN2 cannot change the ATIS requirements, and the external warning center works in a RAT-independent way. But potentially there can be more intelligent handling of CMAS message in RAN that make it more power efficient for the UE.

# Summary

RAN2 is kindly asked to discuss CMAS reception and power saving:

Observation 1: A CMAS capable UE may receive multiple repetitions of the same alert during a long time.

Observation 2: A CMAS capable UE cannot ignore repetitions to save power.

Proposal 1: RAN2 to study potential power saving enhancements for CMAS reception without changing the ATIS requirements

# References

- R2-1901837, Paging enhancement for CMAS reception, Apple, DISC, RAN2#105

- 76 WEA 3.0, Analysis of WEA alert requested durations, Sprint, ATIS STSC WEA

- 76 WEA 3.0, Analysis of WEA amber alert requested durations, Sprint, ATIS WTSC WEA