3GPP TSG-RAN2 Meeting #105bis	R2-1904146

Xi'an, China, 8 - 12 April 2019	Revision of R2-1901270

Agenda Item:	11.11.4.4 Other

Source: 	Ericsson

Title:  	Enhanced cDRX

Document for:	Discussion and Decision

# Introduction

Power saving in connected mode was discussed in RAN2#105 [1-30], but only the cDRX coupled with WUS in connected mode was discussed. In this contribution cDRX enhancements are discussed that are not coupled with WUS in connected mode.

cDRX enables the UE to save power in connected mode when there is no data to send or receive. Furthermore cDRX enables the UE to save power when the UE is just waiting to be released to Idle/Inactive mode, i.e. while the NW InactivityTimer is running.

# Background

cDRX and power saving

When cDRX is configured, and the UE does not have data to send or receive, the UE is only required to monitor PDCCH during the ON duration (drx-onDurationTimer) with the DRX cycle periodicity (drx-ShortCycle or drx-LongCycle). In first instance the power saving with cDRX depends on the ON duration vs DRX cycle length ratio, i.e. a short ON duration with a long DRX cycle length is the most power efficient. However with a long DRX cycle (i.e. maximum value for drx-LongCycle is 10.24 sec) the latency increases, i.e. it takes longer time to reach the UE, and some applications may not tolerate such long delay (e.g. voice/video). Furthermore, a minimum ON duration is required to provide sufficient scheduling flexibility to the gNB, i.e. the gNB can only reach all the connected mode UEs during their individually configured ON durations. It is noted that with a short ON duration, i.e. shorter than the SSB periodicity, the UE may be required to perform RRM measurements of the serving cell (and intra-frequency measurements) outside the ON period, i.e. not able to go to sleep yet. If needed, the UE performs inter-frequency measurements outside the ON period.

The NW InactivityTimer runs in parallel with the cDRX timers in the UE, i.e. when during multiple consecutive ON periods no data was sent or received, the NW InactivityTimer expires and the UE is released to Inactive (or Idle). The NW InactivityTimer is typically much longer than the drx-InactivityTimer, i.e. the former is in the order of seconds or 10’s of seconds, while the latter is in the order of 10’s or 100’s of msec. Inactive (or Idle) mode is the preferred power saving state when the UE does not have data to send or receive for a long time.

cDRX configuration and operation

The cDRX configuration (DRX-Config IE) can be configured separately for masterCellGroup (CA) and secondaryCellGroup (DC) in RRCReconfiguration-IEs, i.e. in RRCReconfiguration message. The masterCellGroup can also already be configured in RRCResume and RRCSetup message. Thus CA can be configured early in MSG4 during RRCResumeRequest, while DC can only be configured later through RRCReconfiguration.

Observation 1: The SCells with CA/DC form a bitpipe with varying bandwidth dependent on the number of activated SCells where the cDRX configuration and operation is common for the SCells in CA or the SCells in the same secondary/master group in DC.

The drx-onDurationTimer ranges between 1/32..31/32 msec or between {ms1, ms2, …, ms1200, ms1600}. The ShortCycle ranges between {ms2, ms3, …, ms512, ms640}, while the LongCycle ranges between {ms10, ms20, …, ms5120, ms10240}. The drx-InactivityTimer can be configured between {ms0, ms1, …, ms1920, ms2560}. The position of the ON durations for the UEs in connected mode can be configured individually with a drx-SlotOffset.

When the gNB knows that there is no more data to send or receive, the gNB can force the UE into DRX with Long DRX or DRX Command MAC CE e.g. while drx-onDurationTimer or drx-InactivityTimer is running.

QoS/QFI

The gNB can use the QoS/QFI of the DRB to adapt the cDRX configuration. Standardized or pre-configured 5G QoS characteristics may include (see 23.501):

- Resource Type (GBR, Delay critical GBR or Non-GBR);

- Priority Level;

- Packet Delay Budget;

- Packet Error Rate;

- Averaging window (for GBR and Delay-critical GBR resource type only);

- Maximum Data Burst Volume (for Delay-critical GBR resource type only).

# Discussion

ON duration

To enable scheduling flexibility to the gNB and power saving to the UE it can be evaluated to apply light or micro sleep during the ON period.

| State | Relative power | Additional transition energy | Total transition time (msec) |
| --- | --- | --- | --- |
| Deep Sleep | 1 | 450 | 20 |
| Light Sleep | 20 | 100 | 6 |
| Micro sleep | 45 | 10 | 0 |
| PDCCH-only | 100 | - | - |

Proposal 1: RAN2 to study the benefit of enabling DRX (micro/light sleep) during the ON duration.

Further optimization can be considered when a power efficient bandwidth (e.g. a different coreset/search space) is used for the active BWP when the UE is monitoring PDCCH during the ON durations. This bandwidth adaptation is triggered and coupled to the cDRX OnDuration:

Proposal 2: RAN2 to study the benefit of bandwidth adaptation (e.g. “dormant” bandiwdth) during ON duration.

The above behaviour is more difficult to achieve bwp-inactivityTimer settings, i.e. with a too short timer the UE might pre-maturely switch to a “dormant” BWP while the UE is still in the cDRX active time. When there switching is coupled with the cDRX timers, i.e. start of the OnDuration, the wanted behavior is achieved.

cDRX cycle length

An optimal cDRX configuration depends on the traffic type and the following two cDRX parameters are considered critical:

- cDRX cycle length: dependent on tolerated latency

- cDRX inactivity timer: dependent on inter traffic-burst distribution

An optimal cDRX configuration depends on the traffic pattern, which in turn is mainly determined by the application(s) running on top. A longer cDRX cycle length is more power efficient, but many applications may not tolerate a LongDRXCycle up to 10 seconds. Example of applications with increasing tighter delay requirements:

- APP sync

- WEB browsing

- Instant messaging

- FTP

- Video streaming

- VoIP

- Gaming

Certain applications generate bursty traffic (e.g. WEB browsing), while others generate a more steady traffic stream (e.g. video streaming). Some applications have a strong latency requirement (e.g. VoIP), while others tolerate long latencies (e.g. APP sync). The UE may have knowledge about the application(s) and thus type of traffic triggering a connection setup:

Proposal 3: RAN2 to study UE assistance information signalling that can assist the gNB to configure cDRX (e.g. DRX cycle length and inactivity timer) after connection setup.

In REL-15 the UE can indicate preferred adjustments to the cDRX cycle length via DelayBudgetReporting. This functionality is connected to voice services, and dynamic adjustments to varying delays. This proposal is not limited to voice, and not necessarily connected with dynamic updates, but assumed to be more semi-static signalling.

This UE assistance information does not necessarily imply an explicit preferred DRX cycle length, but also delay tolerance signalling can be considered. For example it is assumed that currently the NW reluctantly would configure a 10 sec LongDRX. But perhaps certain UE types (e.g. iPad or other data-oriented devices) could tolerate longer delays than a typical MBB UE.

The OnDuration timer length is important from a gNB scheduling perspective, i.e. it provides needed scheduling flexibility to the gNB to reach one or more of the UE’s listening during the OnDuration. It is clear from the NW side, that the UE prefers a short OnDuration time from a power saving perspective:

Observation 2: There is no need for the UE to signal the preference for a (short) OnDuration.

cDRX is typically already configured in MSG4, i.e. UE assistance information requires RRCReconfiguration. In case the gNB can store and retrieve a UE (traffic) profile from the AMF after MSG3, as discussed for UE differentiation [32,33], then optimal cDRX configuration can already be enabled in RRCResumeRequest, and RRCReconfiguration may not be required:

Proposal 4: RAN2 to study UE differentiation to enable optimal cDRX configuration already in RRCResumeRequest.

As indicated earlier the SCells can be considered to enable a varying bitpipe, and for that reason we do not see the need for cDRX configuration/operations per individual SCell. From a UE perspective it is efficient to use all activated SCell when there is data to send or receive, i.e. to “get it over and done with”, and then go to sleep for all SCells. Furthermore individual SCell configurations and operations is more complex for UE and gNB to handle:

Proposal 5: RAN2 not to evaluate cDRX configuration/operations per SCell or group of SCells.

# Discussion

RAN2 is kindly asked to discuss enhanced cDRX:

Observation 1: The SCells with CA/DC form a bitpipe with varying bandwidth dependent on the number of activated SCells where the cDRX configuration and operation is common for the SCells in CA or the SCells in the same secondary/master group in DC.

Observation 2: There is no need for the UE to signal the preference for a (short) OnDuration.

Proposal 1: RAN2 to study the benefit of enabling DRX (micro/light sleep) during the ON duration.

Proposal 2: RAN2 to study the benefit of bandwidth adaptation (e.g. “dormant” bandiwdth) during ON duration.

Proposal 3: RAN2 to study UE assistance information signalling that can assist the gNB to configure cDRX (e.g. DRX cycle length and inactivity timer) after connection setup.

Proposal 4: RAN2 to study UE differentiation to enable optimal cDRX configuration already in RRCResumeRequest.

Proposal 5: RAN2 not to evaluate cDRX configuration/operations per SCell or group of SCells.

# References

- R2-1900202, Power Saving Signal/Channel and Related Procedures, CATT, DISC, RAN2#105

- R2-1900723, Power saving in RRC_CONNECTED, Intel, DISC, RAN2#105

- R2-1900201, Timer Based Uplink BWP Switch, CATT, DISC, RAN2#105

- R2-1900203, Uplink Transmission Enhancements for Power Saving, CATT, DISC, RAN2#105

- R2-1900256, Enhancements to DRX for power saving, OPPO, DISC, RAN2#105

- R2-1900396, C-DRX enhancement for power saving, vivo, DISC, RAN2#105

- R2-1900623, Consideration on power saving scheme with DRX adaptation, ZTE, DISC, RAN2#105

- R2-1901044, Analysis on UE power consumption in NR, Samsung, DISC, RAN2#105

- R2-1901045, RAN2 work for UE power saving in NR, Samsung, DISC, RAN2#105

- R2-1901060, Mobility history information in NR, Nokia, DISC, RAN2#105

- R2-1901074, Power efficient adaptation of multi-carrier operation, MediaTek, DISC, RAN2#105

- R2-1901077, DRX enhancements for power saving, MediaTek, DISC, RAN2#105

- R2-1901188, SR triggering and DRX operation, OPPO, DISC, RAN2#105

- R2-1901190, BWP operation enhancements for power saving, OPPO, DISC, RAN2#105

- R2-1901224, Discussion on assistance information for UE power saving, Huawei, , DISC, RAN2#105

- R2-1901225, Discussion on procedure of assistance information reporting, Huawei, DISC, RAN2#105

- R2-1901270, Enhanced cDRX, Ericsson, DISC, RAN2#105

- R2-1901287, Discussion on C-DRX enhancement for UE power saving, Huawei, DISC, RAN2#105

- R2-1901288, On the potential RNA2 impacts of power saving signals, Huawei, DISC, RAN2#105

- R2-1901289, Discussion on UE power saving in CA scenario, Huawei, DISC, RAN2#105

- R2-1901347, UE energy efficiency measurement, MediaTek, DISC, RAN2#105

- R2-1901437, Power Consumption Adaptation for NR RRC Connected, Sony, DISC, RAN2#105

- R2-1901764, DRX enhancements for better adaptation to traffic, Qualcomm, DISC, RAN2#105

- R2-1901765, Cross cell group power management in MR-DC, Qualcomm, DISC, RAN2#105

- R2-1901766, DRX group for carrier aggregation, Qualcomm, DISC, RAN2#105

- R2-1901767, UE assistance information for power saving, Qualcomm, DISC, RAN2#105

- R2-1901814, DRX enhancement for further power saving, LG, DISC, RAN2#105

- R2-1901838, UE assistance information, Apple, DISC, RAN2#105

- R2-1901914, Considerations on UE power saving for C-DRX, Xiaomi, DISC, RAN2#105

- R2-1902214, UE traffic adaptation for power saving, MediaTek, DISC, RAN2#105

- R2-1902563, LS to RAN1 on RAN2 agreements, LS out, To: RAN1, RAN2#105

- RP-182105, Study on RAN-centric Data Collection and Utilization for LTE and NR, SID, CMCC, RAN#81

- R3-190835, Status Quo and way forward for UE RRM policy handling, Ericsson, DISC, RAN3 #103