3GPP TSG-RAN WG2 Meeting #105bis	R2-1904184

# Xi’an, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of general rule on handling of dummified fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Until now 26 fields and 9 IEs have been dummified and in some cases a respective field description has been added. In view of this it is recommended to add a general rule on handling of dummified fields. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 6.1.3 a general rule on handling of dummified fields is added. Impact analysis Impacted 5G architecture options: Standalone, MR-DC Impacted functionality: Handling of dummified fields in RRC Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may send dummified fields which may lead to RRC failures in network. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. CR is based on V15.4.0 and need to be updated to V15.5.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC is not fully clear in how dummified fields should be handled which may result in IOT issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.1.3	General rules

Some messages may include one or more fields with name dummy. The UE shall avoid sending fields that are named dummy to the network. Likewise, the network should avoid sending fields with name dummy to the UE. If the UE anyhow receives a field named dummy, it shall ignore the field and process the rest of the message as if the field was not included.

NOTE:	A field with name dummy concerns a field that was erroneously included in a previous version of the specification and has been removed by replacing it with a dummy.