3GPP TSG-RAN WG2 Meeting #105bis	R2-1904194

Xian, China, 08 – 12 April 2019	Resubmission of R2-1901448

Agenda item:	11.6.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Mobility aspects for LEO based networks

WID/SID:	FS_NR_NTN_solutions - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN agreed the SI – “Study on solutions evaluation for NR to support Non Terrestrial Network” in [1]. This contribution captures some of the issues relevant to NTN mobility scenarios for LEO.

In [4] we propose RAN2 to discuss on the need to prioritize the LEO satellite scenarios over others, as they are most challenging from a mobility point of view. This contribution lists mobility related issues related to LEO satellite scenarios.

# 2	Mobility issues

## 2.1 	Moving radio cells – moving satellite beam footprint

According to the scenarios in [3] and updated SID scope in [5] a LEO satellite (cell/beam) footprint can be assumed as moving on Earth. The corresponding LEO satellite 5G NR radio cells and beams can be assumed to have the same properties as the LEO satellite footprints: moving on Earth.

In case the 5G NR radio cell(s) provided by a LEO satellite is moving on Earth (Scenarios C2 and D2), the UEs would detect both geographical radio coverage boundary changes and received signal levels changes due to changing distance to the satellite.

The above observations are true for both stationary and moving UEs. In case of a moving UE, the speed of the changes in received signal levels and/or 5G NR cell coverage area location is slightly higher, but still dominated by the speed of the LEO satellite on its orbit and, to a lesser degree also by the rotation of Earth.

In the Scenarios C2 and D2 the UE perceived radio coverage would vary in time according to the movement of satellite on its orbit and the elevation angle of the satellite. Other system parameters, such as antenna radiation patterns (UE and satellite), use of directional antenna at the UE, would also impact the UE perceived radio coverage.  In this scenario a) a UE would need to be able to perform 2 types of handovers (HO) in RRC-NR CONNECTED mode:

- Between two 5G NR cells within the same satellite (intra-footprint) and,

- Between two 5G NR cells from two different satellites (inter-footprints)

Most of the HOs in case i), when the 5G NR cells are closer to the centre of the satellite footprint, are likely to happen at higher signal levels and lower signalling delays, compared to the HO in case ii). In case ii) the HO must be performed at lower signal levels, thus with potentially higher measurement uncertainties and signalling delays. This means that the use of the same HO settings (parameters) for both case i) and case ii) is inefficient. Similar considerations must be given for radio measurements (Radio Link Failure) configurations (see Section 2.5).

Proposal 1: In the LEO scenarios C2 and D2 with moving beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR network-controlled mobility mechanisms for optimal mobility configurations in intra- and inter- satellite beam footprint mobility cases.

Proposal 2: In the LEO scenarios with moving beam footprint on Earth, RAN2 to study a) the suitability of the existing 5G NR network-controlled mobility measurements options for optimal configuration in inter-satellite beam footprint mobility cases, and b) the suitability of the existing 5G NR cell (re)selection mechanisms for best UE parameter configuration regardless of its location on Earth with respect to the LEO satellite orbits.

Furthermore, the NTN network (5G RAN) is expected to have detailed knowledge about its radio cells, their coverage areas and their movement. Hence it is natural to assume that this information is also used to optimize the radio performance of the system and reduce signalling load. The NTN UEs might or might not have sufficient knowledge about the availability and movement of the 5G NR radio cells, in order to optimize ‘autonomously’ their HO and radio measurement procedures.

Observation: The NTN network (5G RAN) is expected to have detailed knowledge about its radio cells, their coverage areas and their movement, and this information is likely to be used to optimize the radio performance of the system, reduce signalling load and reduce UE energy consumption.

# 3	Conclusion

Proposal 1: In the LEO scenarios C2 and D2 with moving beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR network-controlled mobility mechanisms for optimal mobility configurations in intra- and inter- satellite beam footprint mobility cases.

Proposal 2: In the LEO scenarios with moving beam footprint on Earth, RAN2 to study a) the suitability of the existing 5G NR network-controlled mobility measurements options for optimal configuration in inter-satellite beam footprint mobility cases, and b) the suitability of the existing 5G NR cell (re)selection mechanisms for best UE parameter configuration regardless of its location on Earth with respect to the LEO satellite orbits.

Observation: The NTN network (5G RAN) is expected to have detailed knowledge about its radio cells, their coverage areas and their movement, and this information is likely to be used to optimize the radio performance of the system, reduce signalling load and reduce UE energy consumption.

# References

- RP-181370 Study on solutions evaluation for NR to support Non Terrestrial Network, Thales.

- R3-186257 RAN3 101bis Email: # 1_NTN_Mob_mgmt_principles, Thales

- R2-1816538 Report of email discussion [103bis#33] [NR - NTN], Thales

- R2-1905160 NTN architecture and scenarios and the need for prioritization, Nokia, Nokia Shanghai Bell.

- RP-182880 Study on solutions for NR to support non-terrestrial networks (NTN)