3GPP TSG RAN WG2 Meeting #105bis		                        		R2-1904259

Xi’an, China, 8 -12 April 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on the configuration of CHO execution conditions

Agenda item:		12.3.3.1

Document for:	    Discussion and Decision

- Introduction

The following agreements are made for conditional handover at RAN2#104 and 105:

RAN2#104:

Agreements

1	RAN2 will consider a conditional handover: This is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

2	Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

=>	FFS on the exact details of the procedures

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

RAN2#105:

1: The baseline operation for E-UTRAN Conditional HO procedure assumes HO command type of message contains HO execution condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met. 3: The baseline operation for E-UTRAN Conditional HO assumes the source eNB remains responsible for RRC until UE successfully sends RRC Connection Reconfiguration Complete message to target eNB. 4: RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be more suitable for E-UTRAN CHO when there are multiple candidate target cells. In case of single prepared candidate target cell, early packet forwarding could be considered as an option. Detailed decisions require RAN3 study. 5: RAN2 will inform the Conditional HO assumptions (including the baseline operation) to RAN3 via LS at RAN#105bis, requesting RAN3 to kindly work on the CHO scheme aspects matching their expertise (e.g. data forwarding).

In this contribution, we share some views on the configuration of CHO trigging conditions.

- Discussion

The baseline operation for E-UTRAN Conditional HO procedure assumes HO command type of message contains HO execution condition(s) and dedicated RRC configuration(s). It’s straightforward that the dedicated RRC configuration(s) should be completely generated by the target, just like what we do in the legacy handover procedure. The question is whether the CHO execution condition(s) is generated by the source or the target?

Generally speaking, it’s possible to configure the CHO execution condition(s) either by the source or the target. However, it should be noted that in the current specification, the MeasConfig is configured by the source. So from this perspective, the simplest is to let the source cell configure the CHO execution condition(s). Otherwise, new procedures should be specified to let the target configure the CHO execution condition(s), e.g. based on the SourceMeasConfig transferred from the source, which is a substantially different model.

Proposal 1: The CHO execution condition(s) included in the HO command type of message is generated by the source cell.

One or more candidate cells can be supported. Then it should be discussed whether single CHO execution condition(s) is configured for all the candidate cells or separate CHO execution condition(s) are configured for each individual candidate cells? Different candidate cells may have different radio conditions. So we should have the flexibility to allow the network to configure separate CHO execution condition(s) for each individual candidate cells.

Proposal 2: Separate CHO execution condition(s) can be configured for each individual candidate cells.

Another question for the configuration of CHO execution condition(s) is whether only one execution condition or multiple execution conditions can be configured for a single candidate cell?

In our opinion, it would be beneficial to configure more than one CHO execution conditions for a single candidate cell in some situations. During the email discussion for comparison of LTE and NR Conditional handover [1], the majority of companies agreed that Ax events can be reused for CHO execution condition and A3/5 is taken as baseline. So let’s take event A5 as an example, the network should be allowed to configure more than one execution conditions for a single candidate cell, e.g.:

Condtion1 (A5 event with lower threshold for serving and neighbour): Thresh1 = -110; Thresh2 = -100;

Condition2 (A5 event with higher threshold for serving and neighbour): Thresh1 = -100; Thresh2 = -90;

With the above two conditions, typically, Conditional handover will be initiated when Condition2 is fulfilled. However, if the serving quality deteriorates dramatically, e.g. below -110 due to shadow or blockage etc, while the candidate quality is somehow good for camping, e.g. above -100, Condition1 is fulfilled and the UE can initiate handover to the candidate. RLF can be avoided in this particular case.

In addition, during the email discussion for comparison of LTE and NR Conditional handover [1], a question was raised whether multiple trigger quantity is allowed for CHO execution condition. In the legacy handover procedure, the measurement report is only evaluated based on a single trigger quantity. The network indicates just one single trigger quantity (RSRP, RSRQ or RS-SINR) to use in the report configuration. In our opinion, there’s no strong justification to change the baseline of the current mechanism due to the introduction of Conditional handover. In other words, configuring only one single trigger quantity in the report configuration should be the baseline for CHO execution condition. However, in a real network deployment, the network may wish to handover the UE to a neighbour cell when multiple quantities (e.g. RSRP and RSRQ) of the neighbour cell are good enough. To facilitate such kind of operation, the network should be allowed to configure more than one execution conditions for a single candidate cell, e.g. one condition configured with trigger quantity RSRP and the other configured with trigger quantity RSRQ. The UE accesses the candidate cell when both the relevant conditions are met.

Proposal 3: Configuring only one single trigger quantity in the report configuration is the baseline for CHO execution condition.

Proposal 4: Support configuring one or more CHO execution conditions for a single candidate cell.

- Conclusion and proposals

In this contribution, we share some views on the configuration of CHO execution condition(s) with the following observations and proposals:

Proposal 1: The CHO execution condition(s) included in the HO command type of message is generated by the source cell.

Proposal 2: Separate CHO execution condition(s) can be configured for each individual candidate cells.

Proposal 3: Configuring only one single trigger quantity in the report configuration is the baseline for CHO execution condition.

Proposal 4: Support configuring one or more CHO execution conditions for a single candidate cell.

- Reference

[1]. [105#58][NR/MOB] Comparison of LTE and NR Conditional handover (Intel)