3GPP TSG-RAN WG2 Meeting #105bis	R2-1904267

Xian, China, 08 – 12 April 2019

Agenda item:	10.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification on the UE RRC processing time for UE capability retrieval

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During the RAN2#105 meeting in Athens, the topic of processing time at RRC was discussed in the context of capability retrieval.

R2-1900762	RRC processing delay in NR	Intel Corporation	discussion	Rel-15	NR_newRAT-Core

-	Intel clarify the 10ms would be if the UE is requested to report only E-UTRA capabilities. Qualcomm think this is not such an important case.

-	Qualcomm think the UE processing time can be quite different depending on the filtering with amount of processing depending on what is requested. And we also need to consider a bit the testability issues. Prefer to have a big value such as 80ms

-	Intel is also fine with a larger value.

-	Samsung think nothing would be wrong with having a very long value for UE capability.

-	Intel explain it is infrequency and also not in the critical path as the NAS procedures take a long time.

Agreements

1	For TS 38.331, RRC processing delay for UE capability transfer is [15-80] ms.

2	For TS 36.331, RRC processing delay for UE capability transfer to report NR band combinations and/or MR-DC band combinations is corrected to [15-80] ms.

3 	Not define RRC processing delay for the combined procedure of SecurityModeCommand and RRCConnectionReconfiguration.

4	RRC processing delay for RRCReestablishment is 10 ms

5 	Not define RRC processing delay for the combined procedure of RRCReestablishment and RRCConnectionReconfiguration.

=>	Offline discussion to progress the UE capability transfer number (Offline discussion 87, Qualcomm)

In this contribution, we propose to clarify a few of the agreements above.

# 2	Discussion

On Agreement 2, we would like to clarify if the RRC processing delay also applies for the LTE capabilities retrieval when requesting the E-UTRA feature sets.

2	For TS 36.331, RRC processing delay for UE capability transfer to report NR band combinations and/or MR-DC band combinations is corrected to [15-80] ms.

Proposal 1: Clarify for TS 36.331 if the RRC processing delay for UE capability transfer to report LTE feature sets is [15-80] ms or a different value must be applied.

Then, it is not so much clear if the LTE legacy capability retrieval time is impacted if the UE is capable of EN-DC and/or NR. We think that logically there is no connection, hence this should be untouched.

Proposal 2: Agree that for TS 36.331 the RRC processing delay for UE capability transfer to report legacy LTE standalone capabilities is not affected for a UE that is capable of EN-DC and/or NR.

# 3	Conclusion

Based on the very short discussion above, we propose the following:

Proposal 1: Clarify for TS 36.331 if the RRC processing delay for UE capability transfer to report LTE feature sets is [15-80] ms or a different value must be applied.

Proposal 2: Agree that for TS 36.331 the RRC processing delay for UE capability transfer to report legacy LTE standalone capabilities is not affected for a UE that is capable of EN-DC and/or NR.